3GPP TSG RAN WG1 Meeting #86	R1-167219

Gothenburg, Sweden 22nd – 26th August 2016

Agenda Item:	8.1.7

Source:	Huawei, HiSilicon, Volkswagen

Title:	Modeling the Evolution of Line-of-Sight Blockage for V2V Channels

Document for:	Discussion and decision

# Introduction

The whitepaper on channel modeling by key 3GPP stakeholders [1] noted that the missing components in 3GPP standardized channel models, both above and below 6 GHz, include:

- spatial consistency for dual-mobility communication (i.e., V2V or D2D);

- spatially consistent LOS probability/existence;

- blockage modeling;

- moving environment (e.g., cars).

This contribution provides a LOS blockage model that addresses these shortcomings for V2V communication.

In the 12-step channel coefficient generation process defined by [2] (Figure 1), the second step assigns propagation condition, depending on whether the link is in LOS or NLOS.

Figure 1 Channel coefficient generation procedure [2].

Realistic LOS blockage realization is required in order to assign the appropriate path loss, shadowing, small-scale, and large-scale parameters over time and space. LOS blockage modeling is particularly important for V2X communication, because of the following:

- High mobility, possibly on both sides of the link (e.g., in the case of V2V communication), resulting in more dynamic LOS blockage

- have low antenna heights, resulting in more frequent LOS blockage

- V2X communication will be used for applications related to safety, either directly (e.g., emergency braking, intersection collision avoidance application [4], etc.) or indirectly (e.g., platooning [5], lane-change maneuvers [3], etc.).

Observation 1: realistic LOS blockage modeling is particularly important for V2X communication.

While V2X channel measurements and modeling have attracted considerable attention in recent years, a comprehensive model for time and space evolution of LOS blockage is currently not available for V2V channels. Furthermore, TR 38.900 [2], the channel model for phase 1 NR evaluation, has not dealt with V2X and D2D communication.

Observation 2: model for time and space evolution of LOS blockage is currently not available for V2V channels.

Markov chains are often used to efficiently model the time-dependent evolution of different systems. Because of their low computational complexity and capability to model serial dependence between adjacent events, they have also been used to characterize different aspects of wireless channels, including time-dependent bit error rate modeling, channel noise modeling, etc. To model LOS blockage and its evolution for V2V channels, we propose to employ Markov chain comprised of three states: i) LOS; ii) NLOSb – non-LOS due to static objects (e.g., buildings, trees, etc.); and iii) NLOSv – non-LOS due to mobile objects (vehicles), since these three states were shown to have distinct path loss, shadowing, large-scale, and small-scale parameters [6]. Figure 2 shows the resulting Markov chain.

Figure 2 Markov chain for modeling the time evolution of V2V links. States: Line-of-Sight (LOS); ii) non-LOS due to static objects (NLOSb); and iii) non-LOS due to mobile objects (NLOSv).

To calculate LOS and transition probability statistics, we perform geometry-based deterministic simulations of LOS blockage and extract the parameters from real cities and highways for LOS blockage and transition probabilities. Based on the empirical results from five large cities (downtown Rome, New York, Munich, Tokyo, and London) and a highway section (10 km, including two- and three-lane per direction as well as on-ramp traffic), we perform curve fitting and obtain a set of distance-dependent polynomial equations for both LOS probabilities and transition probabilities. Detailed explanation of the modeling approach and validation of the model can be found in [9].

Proposal 1: model the propagation condition of V2V links using three states: i) LOS; ii) NLOSb – non-LOS due to static objects (e.g., buildings, trees, etc.); and iii) NLOSv – non-LOS due to mobile objects (vehicles).

Proposal 2: employ a Markov chain comprised of three states (LOS, NLOSb, NLOSv) to ensure time consistent modeling of V2V channels.

# Model for Time Evolution of V2V Channels

## Mobility modeling

In order to obtain realistic V2V LOS blockage behavior from simulations, vehicles need to move in a realistic way over real roads. For this reason, we used SUMO (Simulation of Urban MObility) [11] to generate vehicular mobility in cities and on highway. By using real roadways and traffic rules and employing vehicular traffic dynamics models such as car-following, SUMO is capable of generating accurate vehicle positions, speeds, inter-vehicle distance, acceleration, overtaking and lane-changing maneuvers, etc. Previous work (e.g., [6]) has shown that V2V channel characteristics are affected by the density of vehicular traffic. To that end, for each environment, we generated three traffic densities, qualitatively characterized as low, medium and high and based on values proposed by [12] for urban and [13] for highway.

## Datasets used for training the model

## Urban

To determine LOS and transition probabilities in urban environments, we extracted the roadway (used for mobility simulation) and object outlines (buildings, trees, walls, etc.) from OpenStreetMap.org for downtown areas of five cities: downtown Rome, New York, Munich, Tokyo, and London (see Table 1). The reason for using multiple cities is to obtain a set of “generic” urban parameters, i.e., those that can readily describe a typical urban environment in terms of LOS blockage, mobility patterns, road configurations, etc. Note that, while the areas used for modeling are of limited size, for high vehicle density there were up to a hundred thousand V2V communication pairs per time step in the urban area since, for a given vehicle, all other vehicles within 500 m radius were considered as possible V2V communication pairs.

Figure 3 Area of downtown London, one of the five cities where LOS blockage analysis was performed. White outlines represent buildings, green represent trees, and red are vehicle outlines.

## Highway

We performed highway simulations with SUMO on a 10 km stretch of A6 highway between Braunsbach and Wolpertshausen in Bavaria, Germany. The road alternates between two and three lanes per direction and has several entry/exit ramps. We simulated the traffic so that 80% of the vehicles enter and exit at the each end of the highway, whereas remaining 20% enter and exit over two ramps near the beginning and the end of the stretch. Since vehicle speeds and dimensions affect the LOS blockage results, 10% of the simulated vehicles were trucks and remaining were personal cars. Some buildings and forest exist on each side of the highway; they can block LOS for larger Tx-Rx distances, particularly for the vehicles moving on the entry/exit ramps.

Table 1 Locations and vehicle densities used for analysis

| Environments |  |  |  |
| --- | --- | --- | --- |
| Location | Boundary (Latitude,Longitude) |  | Size |
|  | Lower left | Upper right |  |
| Rome | 41.8896,12.4751 | 41.9011,12.5016 | 2.8 km |
| New York | 40.7274,-74.0068 | 40.7397,-73.9764 | 3.5 km |
| Munich | 48.1301,11.5553 | 48.1431,11.5928 | 4 km |
| Tokyo | 35.6540,139.7256 | 35.6673,139.7561 | 4 km |
| London | 51.5105,-0.1166 | 51.5230,-0.0794 | 3.6 km |
| A6 Highway | 49.1715,9.7441 | 49.1846,9.8630 | 10 km |
| Test data set for urban environment |  |  |  |
| Paris | 48.8489,2.3161 | 48.8575,2.3439 | 2 km |
|  | Vehicle Densities |  |  |
| Environment | Low | Medium | High |
| Urban [12] | 24 veh/km | 120 veh/km | 333 veh/km |
| Highway [13] | 500 veh/hr/dir. | 1500 veh/hr/dir. | 3000 veh/hr/dir. |

## LOS blockage analysis

We use the LOS blockage classification provided by GEMV, a freely available, geometry-based V2X propagation modeling tool [6]. GEMV uses the outlines of vehicles, buildings, and foliage to distinguish between LOS, NLOSv, and NLOSb links. When determining LOS, GEMV takes into account the electromagnetic interpretation of LOS, wherein it calculates whether 60% of first Fresnel ellipsoid is free of any obstructions, in order to determine whether LOS is blocked. That said, we evaluated the LOS blockage results for frequencies between 2 GHz and 6 GHz and the results do not differ significantly, since the largest difference between 60% of first Fresnel zone between 2 GHz and 6 GHz at 500 m Tx-Rx distance is 1 meter. The results showed hereafter refer to 2 GHz carrier frequency.

## Transition probability analysis

We model the system as a three-state discrete-time Markov chain, as shown in Figure 2. We separate the data based on Tx-Rx distance into 10-meter distance bins and generate the transition probabilities for each of them. For any single step transition between states a and b, we have the following transition probability from measurement time step t to t+1 and for a given distance bin d:

(1)

In effect, this creates a set of distance-dependent transition probabilities with one transition probability matrix for each Tx-Rx distance bin. In addition to LOS blockage, GEMV provides information about transitions between the LOS states. We use the results from GEMV to calculate the transition probability using a frequency-based approach: for each LOS state a (where a∈{LOS,NLOSv,NLOSb}) , we count the transition from a to state b within a distance bin d (note that b=a is possible, since state a can transition in itself). Then, we divide this number by the total number of transitions from a to all possible states (i.e., total number of occurrences of a) to obtain the empirical transition probability:

(2)

where  is the indicator of event A. We use the value of time interval equal to one second as a good trade-off between the ability to train the model (shorter time interval would require proportionally more data to obtain representative results) and precision (within one second, in vast majority of cases, there will be at most one transition between LOS states, whereas in a longer period this might not be the case).

# Results

## LOS probabilities

Figure 4 shows the LOS probabilities for each of the three states in urban, whereas Figure 5 shows the results for highway. Results for different vehicle densities show an intuitively expected result: the higher the vehicle density, the more probable the NLOSv state, since in low density scenarios there simply are not that many vehicles around to block the LOS. For high density urban scenario, NLOSv probability reaches 50% when Tx and Rx are between 30 and 70 meters apart.

Figure 4 LOS probabilities in urban environment: combined Rome, New York, Munich, Tokyo, and London results.

Figure 5 LOS probabilities on A6 highway.

## LOS transition probabilities

Figure 6 and Figure 7 show the state transition probabilities for urban and highway environments, respectively, along with the curve fits according to equations presented in Table 2. Across densities in urban environment, LOS transition probability to itself reduces with increasing distance and corresponding transition from LOS to NLOSv and NLOSb increases. The remaining transition probabilities are comparatively independent of distance, with the exception of the expected increase from NLOSb to itself with increasing distance.

Figure 6 Transition probabilities in urban environment: combined Rome, New York, Munich, Tokyo, and London results.

Figure 7 Transition probabilities on A6 highway.

Proposal 3: use specific set of parameters for LOS blockage and transitions in distinct V2V scenarios: a) highway/urban; b) low/medium/high vehicular density.

## Curve fitting for LOS blockage and transition probabilities

To provide a tractable model for generation of time-evolved V2V links, we perform curve fitting for LOS and transition probabilities. Table 2 shows the equations of resulting LOS probability curves, whereas Table 3 shows the equations for transition probabilities. Figure 4 to Figure 7 show how well the curve fits (full lines) match the empirical data (markers).

For LOS probabilities in highway, the equations are in the form of second degree polynomials: , where d is the Tx-Rx distance. In urban environment, exponential and log-normal distributions are a better fit for LOS probabilities. Similarly, curve fits for transition probabilities predominantly take the form of second degree polynomials, with the exception of transitions involving NLOSb state in highway environment, which are better approximated by a log-normal distribution. For more complex transition probability curves in highway, we perform piece-wise curve fitting, since there are notable discontinuities.

Table 2 Curve fitting results: LOS probabilities

Table 3 Curve fitting results: Transition probabilities.

Proposal 4: to model the LOS probabilities as a function of Tx-Rx distance, use the equations listed in Table 2.

Proposal 5: to model the LOS transition probabilities as a function of Tx-Rx distance, use the equations listed in Table 3.

# Usage of the model

Algorithm 1 describes how the developed model can be used to generate LOS states for a V2V link. For n time steps, the algorithm generates a set of n-tuple states by using LOS probability (Table 2) and transition probability equations (Table 3). The states represent the time evolution of the link in the given environment and for a given density. Since the model takes as input distance between Tx and Rx for each time step, there needs to exist consistency between subsequent distances between Tx and Rx in order to obtain credible results for LOS blockage and transition probabilities.

Proposal 6: use Algorithm 1 for generating time- and space-evolved LOS states.

# Path loss comparison with 3GPP/ITU Urban Micro (UMi) LOS probability model

Since there are no comprehensive V2V link LOS blockage and transition probability models available in the literature, we compare the proposed model with the well-established 3GPP/ITU Urban Micro (UMi) LOS probability model [8], which is currently used for LOS probability of V2X and D2D links:

(5)

where d is distance between Tx and Rx,  is a parameter set to 18 meters, and  to 36 meters. For illustration purposes, we use the states generated by the two models to calculate the path loss for a Tx-Rx pair that moves apart at 1 m/s starting from 1 to 500 meters. We use the parameters for urban medium density. For LOS and NLOSb path loss parameters, we use the values based on measurements reported in [6]. For NLOSv, we use the multiple knife-edge attenuation model described in [6]. For clarity reasons, we show path loss only (i.e., without shadow fading). Figure 8 shows the path loss results for the proposed model and 3GPP UMi LOS probability model [8]. Since UMi model does not model dependency on the previous LOS state, the number of transitions between the states is considerably higher than in the proposed model, particularly when the probability of LOS is close to 50% (i.e., between 50 and 100 meters). This is clearly an unrealistic behavior, since two vehicles will not move between LOS and non-LOS states so rapidly. While Figure 8 shows a single realization of state changes and resulting path loss, we ran simulations for a large number (10) of V2V pairs with distances between 0 and 500 m. UMi model resulted in an average state change every 5 seconds, while the proposed model averaged one state change every 17 seconds.

Figure 8 Comparison of path loss generated by proposed model and 3GPP UMi LOS probability model for a single V2V link with Tx and Rx moving apart from 0 to 500 m with relative speed of 1 m/s. For reference, path loss for LOS, NLOSv, and NLOSb states is plotted.

Figure 8 also indicates why models based on correlation distance for spatial consistency (e.g., model in Section 7.6.3 in [2] are not sufficient for modeling time- and space- evolved V2V links. Shadowing decorrelation distance in urban and highway environment for V2V links is on the order of tens of meters (e.g., [14] reports decorrelation distances of 5 m in urban and 30 m in highway environment) and is assumed to be independent of the Tx-Rx distance. However, Figure 8 shows that a single decorrelation distance value cannot capture the changing behavior as Tx-Rx distance changes: at low distances, the LOS decorrelation distance is high and decreases with increasing Tx-Rx distance, whereas NLOSb decorrelation distance increases with increasing Tx-Rx distance. By using distance-dependent transition probabilities, the employed model is capable of capturing this behavior. The above results bring us to the following observation.

Observation 3: Not employing a time-evolved LOS blockage model results in:

- inaccurate estimate of interference, since the calculated interference contributions from specific vehicles will be more sustained than what a simple model estimates;

- overestimating the benefits of retransmissions; a link in NLOS state is likely to stay in that state for a longer period of time than what is estimated by a simple model, thus making the retransmissions less effective;

- erroneous estimate of performance for applications requiring continuous transmission between two vehicles, since the link duration will be impacted by the unrealistically high number of transitions between the states.

# Conclusions

We performed a comprehensive analysis of LOS blockage evolution for V2V links in real cities and highways. To efficiently model time evolution of LOS blockage for V2V links, we employed a three-state Markov chain, which we trained using a large set of realistically simulated V2V links (more than 10 in high density scenarios) in urban and highway environments. The proposed model can be used to:

- generate time- and space-consistent LOS probability for V2V channels by assigning appropriate parameters for each of the three states in representative urban and highway environments.

- incorporates the moving environment (vehicles) into the calculations of LOS and transition probability

- enables efficient simulations of time-evolved V2V channels, without the need for complex geometry-based deterministic modeling.

The following observations were drawn:

Observation 1: realistic LOS blockage modeling is particularly important for V2X communication.

Observation 2: model for time and space evolution of LOS blockage is currently not available for V2V channels.

Observation 3: Not employing a time-evolved LOS blockage model results in:

- inaccurate estimate of interference, since the calculated interference contributions from specific vehicles will be more sustained than what a simple model estimates;

- overestimating the benefits of retransmissions; a link in NLOS state is likely to stay in that state for a longer period of time than what is estimated by a simple model, thus making the retransmissions less effective;

- erroneous estimate of performance for applications requiring continuous transmission between two vehicles, since the link duration will be impacted by the unrealistically high number of transitions between the states.

We propose the following:

Proposal 1: model the propagation condition of V2V links using three states: i) LOS; ii) NLOSb – non-LOS due to static objects (e.g., buildings, trees, etc.); and iii) NLOSv – non-LOS due to mobile objects (vehicles).

Proposal 2: employ a Markov chain comprised of three states (LOS, NLOSb, NLOSv) to ensure time consistent modeling of V2V channels.

Proposal 3: use specific set of parameters for LOS blockage and transitions in distinct V2V scenarios: a) highway/urban; b) low/medium/high vehicular density.

Proposal 4: to model the LOS probabilities as a function of Tx-Rx distance, use the equations listed in Table 2.

Proposal 5: to model the LOS transition probabilities as a function of Tx-Rx distance, use the equations listed in Table 3.

Proposal 6: use Algorithm 1 for generating time- and space-evolved LOS states.

# References

“5G channel model for bands up to 100 GHz, v2.0,” in IEEE Globecom 2015 Workshops, Mar. 2016. [Online]. Available: http://www.5gworkshops.com

3GPP TR 38.900, “Channel model for frequency spectrum above 6 GHz”, v2.0.0, June 2016.

S1-152251, “Use Case Cooperative Lane Merging (CLM)”, 3GPP TSG-SA WG1 Meeting #71, Aug. 2015.

S1-152252, “Use Case Cooperative Longitudinal & Lateral Control for Collision Avoidance (CoLCA)”, 3GPP TSG-SA WG1 Meeting #71, Aug. 2015.

S1-152253, “Use Case Cooperative Short Distance Platooning (CoSdP)”, 3GPP TSG-SA WG1 Meeting #71, Aug. 2015.

M. Boban, J. Barros, and O. Tonguz, “Geometry-based vehicle-to-vehicle channel modeling for large-scale simulation,” IEEE Transactions on Vehicular Technology, Vol. 63, No. 9, November 2014.

S1-160111, “Preparations for eV2X Rel-15”, LG Electronics Inc.

“Report ITU-R M.2135-1: Guidelines for evaluation of radio interface technologies for IMT-Advanced,” 2009.

M. Boban, X. Gong, W. Xu: 'Modeling the Evolution of Line-of-Sight Blockage for V2V Channels', IEEE Vehicular Technology Conference: VTC2016-Fall, Montreal, Canada, Sept. 2016. Available online: http://arxiv.org/pdf/1607.04420v1.pdf

3GPP TR 36.885, “Study on LTE-based V2X Services”

M. Behrisch, L. Bieker, J. Erdmann, and D. Krajzewicz, “SUMO–simulation of urban mobility,” in The Third International Conference on Advances in System Simulation (SIMUL), Barcelona, Spain, 2011.

M. Ferreira, H. Conceição, R. Fernandes, and O. K. Tonguz, “Stereoscopic Aerial Photography: An Alternative to Model-Based Urban Mobility Approaches,” in Proceedings of the Sixth ACM International Workshop on VehiculAr Inter-NETworking (VANET). ACM New York, NY, USA, 2009.

N. Wisitpongphan, F. Bai, P. Mudalige, V. Sadekar, and O. K. Tonguz, “Routing in sparse vehicular ad hoc wireless networks,” IEEE Journal on Selected Areas in Communications, vol. 25, no. 8, pp. 1538–1556, Oct. 2007.

T. Abbas, K. Sjöberg, J. Karedal, and F. Tufvesson, “A measurement based shadow fading model for vehicle-to-vehicle network simulations,” International Journal of Antennas and Propagation, 2015.

# Appendix A: Validation of the proposed model

To test how well the proposed model generalizes to other cities (i.e., those on which it was not trained), we compared the model trained on the data from five cities shown in Table 1 and a data set obtained in downtown Paris (also described in Table 1). Table 4 shows the results of comparison in terms of Pearson Correlation Coefficient for both the LOS probabilities and transition probabilities. The correlation is above 0.95 for all three LOS probabilities. Similarly, the correlation coefficients for transition probabilities are high (average correlation equals 0.84), with only the NLOSvNLOSv below 0.8. Overall, however, the correlation results show that the model fits well to urban environments outside those that it was trained on and can thus be used with confidence as a representative model for “typical” urban environments.

Table 4 Correlation coefficients: Paris vs model (trained on Rome, New York, Munich, Tokyo, and London data sets). Medium density.