3GPP TSG RAN WG1 Meeting #86                                                                 R1- 167659

Göteborg, Sweden, 22-26 August, 2016

Agenda item:	7.1.5.1

Source: 	National Taiwan University

Title: 	Double QC-LDPC codes with degree-3 for NR

Document for:	Discussion

- Introduction

The quasi-cyclic low density parity check (QC-LDPC) code has been identified as one of the promising channel coding candidates for NR in RAN1#84bis meeting. In this contribution, we provide a Double QC-LDPC (DQC-LDPC) code design with parity-check matrices for evaluation.

- Double QC-LDPC code design

The QC-LDPC code can be represented by the parity-check matrix H that consists of small square blocks (or submatrices) of size Q x Q which are the null matrix or circulant permutation (right-shifted identity) matrices. The QC-LDPC codes are designed with the following parameters.

- Circulant permutation matrix size Q. For easier implementation, Q = 2q is usually selected. In this contribution Q = 64 is selected for illustration purpose, other sizes of Q are  viable.

- Codeword length N = Q∙nq, information block K = Q∙kq,

- Code rate r = K/N = kq/nq. However, these codes are not classified by rate but by parity check size. It can be more flexible with varying information bits.

- Shift value sq (sub-block size or lift size).

- Column degree dv (the same as variable degree), which is strictly regular-3 for each matrix without any exception. To put it again, it is not average 3 with dv=2 or 4.

- Row degree rv  (the same as check degree), designed as nearly regular. Due to code rate, it is ranged from 5 to 32.

The circulant permutation matrix Pk=pij (0≤i,j<Q) is obtained from the Q x Q identity matrix by cyclically shifting the columns to the right by k columns.

For example, for Q = 5, the following show example matrices,

,,.

Using the above notation, parity-check matrices for large block sizes can be represented in  a compact form. For example, an expanded matrix HM (of size mq x nq) can be used to denote the mqQ x nqQ binary parity-check matrix obtained by using a compact base matrix Hbm (of size mq x nq) and a shift size value sq.

Encoding

In this contribution, a DQC-LDPC code is presented. The DQC-LDPC is a special  form of the classical QC-LDPC codes in the parity check matrix. The DQC code features double layers of circulant matrices (or circulant of circulants) as shown in Fig. 1. For convenience, “*” is used to denote the null matrix of size Q x Q. As in QC-LDPC code, each small square blocks (or submatrices) of size Q x Q are the null matrix (denoted by “*”) or circulant permutation (right-shifted identity) matrices. A square collection of the small square blocks forms the outer layer of circulant matrix in H. The unique feature helps to encode the parity with double shift register array.

Fig. 1. Example of DQC-LDPC

The DQC codes with different rates have advantage in encoder implementation. The error correction performance is not affected by the DQC structure at all.

Decoding

The scalar min-sum decoder is used to evaluate block error rate (BLER). The scalar min-sum decoder has general column shuffle architecture with early termination. The convergence is achieved by checking whether the syndrome vector is all-zero. Within decoding process, the scalar is 3/4 due to bit-shift-adder hardware implementation. In principle, the parity check matrix generally determines the error correction performance. In general, the classical QC-LDPC decoding strategies are all suitable to the DQC-LDPC codes.

Block size support

Similar to the LTE turbo codes, the block size selection needs to consider underlying code structure. We select several parity-check matrix sizes such as 256, 512, 768, 1024, 1536, 2048 to meet various user cases. Each parity-check matrix size could construct 3~7 base matrices with the best correction performance. In the following step, a flexible matrix size can be obtained by column-truncation from the base matrix if necessary. The number of column truncation is the multiples of circulant matrix size (e.g. 64). In principle, DQC-LDPC codes can be designed to support any arbitrary information/code block size.

Fig. 2. Column truncation of DQC-LDPC for flexible code rate support

In the illustration example, all the base matrices are designed by column-degree 3, QC block size 64, and DQC encoding feature.

- Performance evaluation

RAN1 agreed for initial evaluations of different channel coding schemes in RAN1#84bis. In the attached text file (R1_dqc_ldpc.xls), we provide DQC-LDPC parity-check matrices capable of supporting coding parameters applicable to eMBB scenarios. The evaluation are presented in the companion contribution.

The example DQC-LDPC code designs is listed in Table 1.  The code rates covers from 0.111 to 0.889 with variable-degree-3. We utilize scalar min-sum decoder to evaluate the block error rate (BLER) versus SNR (Es/N0). The simulation results for parity size 256, 512, 768, 1024, 1536, and 2048 are shown in Fig. 3 to Fig. 8 respectively. All the figures are under maximum iterations=31 with 10 soft-bits to behave as nearly-floating input. The input quantization could be further discussed in the future.

Table 1. Parameters of the proposed parity-check matrices for evaluations.

| Parity | Base Codeword Length N | Flexible Codeword Length N | Code Rate |
| --- | --- | --- | --- |
| 256 | {1280, 768, 512} | 256+64*i, i in{1,2…,16} | 0.5~0.8 |
| 512 | {4608, 3584, 2560, 1536, 1024, 768, 640} | 512+64*i, i in{1,2…,64} | 0.2~0.889 |
| 768 | {4864, 4096, 3072, 1792, 1280, 1024} | 768+64*i, i in{1,2…,64} | 0.25~0.842 |
| 1024 | {5120, 4096, 3072, 2048, 1536, 1280} | 1024+64*i, i in{1,2…,64} | 0.2~0.8 |
| 1536 | {5632, 4096, 3584, 2560, 2048} | 1536+64*i, i in{1,2…,64} | 0.25~0.727 |
| 2048 | {6144, 5120, 4096, 3072, 2560, 2304} | 2048+64*i, i in{1,2…,64} | 0.111~0.667 |

Fig. 3.The performance results of codes with 256-bit parity are shown in the companion contribution.

Fig. 4. The performance results of codes with 512-bit parity are shown in the companion contribution.

Fig. 5. The performance results of codes with 768-bit parity are shown in the companion contribution.

Fig. 6. The performance results of codes with 1024-bit parity are shown in the companion contribution.

Fig. 7. The performance results of codes with 1536-bit parity are shown in the companion contribution.

Fig. 8. The performance results of codes with 2048-bit parity are shown in the companion contribution.

- Conclusion

In this contribution, we provide DQC-LDPC code design with various parity size to satisfy flexible use scenarios. The contribution also includes the aspects related to encoding/decoding/flexibility in terms of block-sizes/code-rate support. We also provide the parity-check matrices for performance evaluations. We propose to further study LDPC code for NR, including the following statements:

Proposal 1: The DQC-LDPC code is flexible in supporting low to high code rates.

Proposal 2: The encoding structure of DQC-LDPC is hardware friendly.

References

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- R1-164697, “LDPC Design Overview”, R1-164697, Qualcomm Inc., 3GPP TSG RAN WG1 #85, Nanjing, China, May 23-25, 2016..

- R1-165164, "Discussion on LDPC Code Design and Performance Evaluation,"   MediaTek Corp., 3GPP TSG RAN WG1 #85, Nanjing, China, May 23-25, 2016.