3GPP TSG RAN WG1 #86	R1-167889

# Gothenburg, Sweden, 22th – 26th Aug. 2016

Agenda item:	8.1.4.1

Source: 	Samsung

Title: 	Design of Flexible LDPC Codes

Document for:	Discussion and Decision

Introduction

In the RAN1 #85 meeting, it was agreed that companies providing evaluations or proposals for LDPC codes are encouraged to show how; 1) Multiple code rates and multiple code sizes would be supported, 2) Suitable granularity of  information block size and code rate would be supported, 3) To support HARQ with/without IR [1].

In [2], [3], Samsung proposed a quasi-cyclic (QC) LDPC code obtained by concatenating a small QC LDPC and many single parity-check codes. Furthermore, Samsung conduct the simulation for evaluating the performance of the proposed QC LDPC code in [4].

In this contribution, we present the parity-check matrix of the proposed code and introduce a detailed lifting, shortening and puncturing method.

Flexible LDPC Codes Based on Lifting and Shortening/Puncturing

Quasi-cyclic LDPC codes

Let H be the mbZ×nbZ matrix given by

H=Pa11Pa21Pa12Pa22⋯⋯Pa1nbPa2nb⋮Pamb1⋮Pamb2⋱⋯⋮Pambnb

where aij∈{-1, 0, 1, 2, …, Z-1} are exponent indices of permutation matrices, nb and mb are the numbers of column and row blocks, respectively. Paij is just the circulant permutation matrix which shifts the identity matrix I(=P0) to the right by aij times for any integer aij, 0≤aij<Z. For simple notation, we denote the Z×Z zero matrix O by P-1. When H has full rank, we can assign (nb-mb)Z information bits to some nb-mb column blocks. (For our convenience, we call these kb=nb-mb column blocks information column blocks). Then the code with H is referred to as a QC LDPC code. Furthermore, let E(H) be the expoment matrix of H given by

EH=a11a21a12a22⋯⋯a1nba2nb⋮amb1⋮amb2⋱⋯⋮ambnb.

An example of a parity-check matrix for a (8Z, 4Z) QC LDPC code with nb=8 and mb=4 is given by

H=P0OOP1OP0P4OOP2P3OOP6P5OP1P0OP0OOP0OP0OP1OP0P0OP0,   EH=0-1-11-104-1-123-1-165-110-10-1-10-10-11-100-10.

Proposed structure

The structure of the proposed QC LDPC code is depicted in Figure 1. The proposed code can be regarded as a QC LDPC code obtained by concatenating a small QC LDPC and many single parity-check codes. The more single parity-check codes are concatenated, the lower rate QC LDPC codes can be obtained. Furthermore, the structure of single parity-check (SPC) extension from the higher rate code could be a good candidate to support IR-HARQ since the SPC extension makes it possible to create additional parity bits as much as needed. Note that when puncturing of single parity-check bits (so called degree-1 parity bits) to obtain a high code rate, the punctured parity bits are completely ignored in the LDPC decoder.

Figure 1. Structure of proposed QC LDPC code

Lifting method

When adjusting the size of circulant permutation matrices according to the target code block size, each exponent indices can be easily calculated by the specified formula. For example, we can obtain the exponent matrix EHZ for the parity-check matrix HZ from the exponent matrix EH=aij for the parity-check matrix H as follows:

[Lifting]

EHZ=aij(Z)    s.t.   aij(Z)=aij,aij<0f(aij, Z),aij≥0.

Here, HZ is the parity-check matrix consisting of Z×Z circulant permutation matrices and/or zero matrices for given integer Z and f(aij, Z) is an integer function of aij and Z.

We propose the lifting function faij, Z as follows:

faij, Z=aijaij<0mod(aij,2k)aij≥0  and  2k≤Z<2k+1

where mod(aij,A) means a modulo operation aij mod A. Note that for 2k≤Z<2k+1, the 2k exponent matrices have exactly the same integer entries. Therefore, if Zmax=2kmax, a given exponent matrix for Zmax can create kmax+1 exponent matrices corresponding to Zmax parity-check matrices.

Figure 2. Lifting technique for length compatibility

Shortening

If the information block size is K after the segmentation of transport block, we first apply the proposed lifting to a given exponent matrix with Z×Z submatrices in Section 2.2. Here, Z is the least positive integer satisfying kbZ≥K, i.e., Z is the positive integer such that K/kb≤Z<K/kb+1. Then, we can derive the parity-check matrix for the QC LDPC code from the exponent matrix by Z-lifting. As described in Figure 3, zeros are inserted to (kbZ-K) bits for shortening and they are not transmitted, which is also known to the receiver. Note that since 0≤kbZ-K<kb due to the minimality for the choice of Z, the maximum shortening size is kb-1.

In this contribution, for our convenience, we assume that the shortening is applied to the last (kbZ-K) information bits, as depicted in Figure 3.

Figure 3. Concept of shortening LDPC codes

Puncturing and Repetition

Rate-matching can be simply implemented in the same manner as LTE standard. First, the information block is shortened to fit the number of information size and then the puncturing is applied to some information or parity bits to support variable code rates, as described in Figure 4.

Figure 4. Concept of rate-matching process

Note that the parity puncturing is sequentially applied from the last parity bits in reverse order.

To support code rate lower than a given LDPC code, we apply repetition. The repetition is applied as described in Figure 4.

Observation 1: QC LDPC codes constructed by the lifting, shortening and puncturing support variable code rates efficiently.

Proposal 1: To support the rate-compatibility of QC LDPC codes comparable to that of turbo code, the lifting, shortening, and puncturing techniques should be adopted.

- References

- R1-164813, "Chairman’s note," 3GPP TSG RAN WG1 #85, Nanjing, China, May 23-25, 2016.

- R1-166769, Samsung, "Discussion on Length-Compatible Quasi-Cyclic LDPC Codes," 3GPP TSG RAN WG1 #86, Gothenburg, Sweden, 22-26 Aug. 2016.

- R1-166770, Samsung, "Discussion on Rate-Compatible Quasi-Cyclic LDPC Codes," 3GPP TSG RAN WG1 #86, Gothenburg, Sweden, 22-26 Aug. 2016.

- R1-164812, Samsung, "Preliminary evaluation results on Quasi-Cyclic LDPC codes," 3GPP TSG RAN WG1 #86, Gothenburg, Sweden, 22-26 Aug. 2016.

- Appendix: Example of Parity-Check Matrix

- nb, mb, kb: 98, 66, 32

- Maximum variable and check degrees: 15, 19

- No. of layers: 16

- The information bits corresponding to the first two column blocks are always punctured.

- Lifting function:

faij, Z=aijaij<0mod(aij,2k)aij≥0  and  2k≤Z<2k+1 .

- The code rate R can be defined by

R=K(nb-2)Z-(kbZ-K)-Npunc=KK+nb-kb-2Z-Npunc.

where Npunc is the number of parity bits to be punctured.

※ Please refer to the excel sheet attached separately.