3GPP TSG RAN WG1 Meeting #86	R1-168399

# Gothenburg, Sweden, 22nd -26th August 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 729 | rev | 1 | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RV version for PDSCH carrying paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1-Core |  |  |  |  |  |  | Date: |  | 2016-08-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Currently the specs. do not state how RV is to be calculated for PDSCH carrying paging. Existing text only covers, RV determination for: - BL/CE UE configured with CEModeA with PDSCH configured by DCI 6-1A - Use RV field within DCI - BL/CE UE configured with CEModeB - Fix rvdci to 0 Note, that a BL/CE UE receiving paging PDSCH, does not have accurate knowledge of its CE mode and is configured by a DCI (DCI 6-2) that does not support a RV field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the following text to “Modulation order and redundancy version determination” section: For a BL/CE UE configured with CEModeB, or a BL/CE UE receiving PDSCH associated with P-RNTI, for FDD and for TDD, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE fails to decode paging messages due to incorrect assumption regarding rvdci. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 7.1.7.1	Modulation order and redundancy version determination

For BL/CE UEs configured with CEModeA, is used in place of in the rest of this subclause.

The UE shall use = 2 if the DCI CRC is scrambled by P-RNTI, RA-RNTI, or SI-RNTI, or if PDSCH is assigned by MPDCCH DCI Format 6-1B, or if PDSCH carriers SystemInformationBlockType1-BR, or if PDSCH carries BL/CE SI messages, otherwise,

-	if the higher layer parameter altCQI-Table-r12 is configured, and if the PDSCH is assigned by a PDCCH/EPDCCH with DCI format 1/1B/1D/2/2A/2B/2C/2D with CRC scrambled by C-RNTI,

-	if the assigned PDSCH is transmitted only in the second slot of a subframe, the UE shall useand Table 7.1.7.1-1A to determine the modulation order (). The modulation order () used in the physical downlink shared channel is set to ;

-	otherwise, the UE shall useand Table 7.1.7.1-1A to determine the modulation order () used in the physical downlink shared channel.

-	else

-	if the assigned PDSCH is transmitted only in the second slot of a subframe, the UE shall useand Table 7.1.7.1-1 to determine the modulation order (). The modulation order () used in the physical downlink shared channel is set to ;

-	otherwise, the UE shall useand Table 7.1.7.1-1 to determine the modulation order () used in the physical downlink shared channel.

Table 7.1.7.1-1: Modulation and TBS index table for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 1 |
| 2 | 2 | 2 | 2 |
| 3 | 2 | 2 | 3 |
| 4 | 2 | 2 | 4 |
| 5 | 2 | 4 | 5 |
| 6 | 2 | 4 | 6 |
| 7 | 2 | 4 | 7 |
| 8 | 2 | 4 | 8 |
| 9 | 2 | 4 | 9 |
| 10 | 4 | 6 | 9 |
| 11 | 4 | 6 | 10 |
| 12 | 4 | 6 | 11 |
| 13 | 4 | 6 | 12 |
| 14 | 4 | 6 | 13 |
| 15 | 4 | 6 | 14 |
| 16 | 4 | 6 | 15 |
| 17 | 6 | 6 | 15 |
| 18 | 6 | 6 | 16 |
| 19 | 6 | 6 | 17 |
| 20 | 6 | 6 | 18 |
| 21 | 6 | 6 | 19 |
| 22 | 6 | 6 | 20 |
| 23 | 6 | 6 | 21 |
| 24 | 6 | 6 | 22 |
| 25 | 6 | 6 | 23 |
| 26 | 6 | 6 | 24 |
| 27 | 6 | 6 | 25 |
| 28 | 6 | 6 | 26/26A |
| 29 | 2 | 2 | reserved |
| 30 | 4 | 4 |  |
| 31 | 6 | 6 |  |

Table 7.1.7.1-1A. Modulation and TBS index table 2 for PDSCH

| MCS Index | Modulation Order | Modulation Order | TBS Index |
| --- | --- | --- | --- |
| 0 | 2 | 2 | 0 |
| 1 | 2 | 2 | 2 |
| 2 | 2 | 2 | 4 |
| 3 | 2 | 4 | 6 |
| 4 | 2 | 4 | 8 |
| 5 | 4 | 6 | 10 |
| 6 | 4 | 6 | 11 |
| 7 | 4 | 6 | 12 |
| 8 | 4 | 6 | 13 |
| 9 | 4 | 6 | 14 |
| 10 | 4 | 8 | 15 |
| 11 | 6 | 8 | 16 |
| 12 | 6 | 8 | 17 |
| 13 | 6 | 8 | 18 |
| 14 | 6 | 8 | 19 |
| 15 | 6 | 8 | 20 |
| 16 | 6 | 8 | 21 |
| 17 | 6 | 8 | 22 |
| 18 | 6 | 8 | 23 |
| 19 | 6 | 8 | 24 |
| 20 | 8 | 8 | 25 |
| 21 | 8 | 8 | 27 |
| 22 | 8 | 8 | 28 |
| 23 | 8 | 8 | 29 |
| 24 | 8 | 8 | 30 |
| 25 | 8 | 8 | 31 |
| 26 | 8 | 8 | 32 |
| 27 | 8 | 8 | 33/33A |
| 28 | 2 | 2 | reserved |
| 29 | 4 | 4 |  |
| 30 | 6 | 6 |  |
| 31 | 8 | 8 |  |

For BL/CE UEs, the same redundancy version is applied to PDSCH transmitted in a given block of  consecutive subframes, if the PDSCH is not carrying SystemInformationBlockType1-BR or SI message. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies , where  for FDD and  for TDD. Denote  as the subframe number of the first downlink subframe intended for PDSCH. The PDSCH transmission spans  consecutive subframes including non-BL/CE subframes where the PDSCH transmission is postponed. Note that BL/CE subframe(s) refers to either BL/CE DL subframe(s) or BL/CE UL subframe(s). For the  block of consecutive subframes, the redundancy version (rvidx) for PDSCH is determined according to Table 7.1.7.1-2 using , where  and . The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured in CEModeA,  and  is determined by the ‘Redundancy version’ field in DCI format 6-1A. For a BL/CE UE configured with CEModeB, or a BL/CE UE receiving PDSCH associated with P-RNTI,  for FDD and  for TDD, and .

Table 7.1.7.1-2: Redundancy version

| Redundancy version Index | rvidx |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 3 |
| 3 | 1 |