3GPP TSG RAN WG1 Meeting #86	R1-1609613

Lisbon, Portugal 10th - 14th October 2016

Agenda Item:	8.1.1.2

Source: 	MediaTek Inc.

Title:  	Benchmark with NB-IoT with non-orthogonal multiple access

Document for:	Discussion/Decision

# Introduction

New multiple access schemes are discussed in New Radio study item. Several new multiple access schemes are proposed, targeting to improve capacity and better support grant-free/contention based uplink. A LS [1] is agreed in RAN # 72, that,

Within the scope of the NR Study, NB-IoT and eMTC should be used as the reference benchmark for assessing the performance of new proposals according to mMTC use case.

In this contribution, we provide benchmark with NB-IoT with non-orthogonal multiple access technique.

# Evaluation assumption

Two settings are considering to compare NOMA and OMA over NB-IoT structure.

- Case 1: Total Resource: 1 PRB x 4 TTI.

  - NOMA: 2UE and each over 12 subcarrier x 2 RU x 2 repetition

  - OMA (TDM): 2 UE and each over 12 subcarrier x 2 RU x 1 repetition

- Case 2: Total Resource: 6 subcarrier x 8 TTI

  - NOMA: 2 UE and each over 6 subcarrier x 2 RU x 2 repetition

  - OMA (FDM): 2 UE and each over 3 subcarrier x 2RU x 1 repetition.

For both case 1 and case 2, the total radio resource to deliver two UE are the same, therefore, with same TBS, the spectrum efficiency is the same. For case 1, the data rate of OMA is double of NOMA because the transmission for OMA is half of NOMA. And Case 2 provide same per UE data rate for both NOMA and OMA. Other simulation assumption can be found in Table 1.

# Evaluation Result

Table 1 and Table 2 summarize SNR gain of NOMA over OMA under NB-IoT structure given the same sum throughput. With realistic channel estimation, the SNR gain is less than with ideal channel estimation. The SNR gain with high code rate (per UE SE) is larger than low code rate case. This is because OMA’s code rate is much high than NOMA, and the channel coding gain obtained by NOMA is large.

Observation # 1: NOMA provide 2-3dB gain over OMA under NB-IoT frame work.

Table 1 Sum throughput (Ideal Channel Estimation) SNR gain in dB @ BLER=0.1

| SE per UE (bps/Hz) | Number of UEs | Baseline | MA signature selection |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | 1T2R, TDL-C, DS=300ns |
| 0.5 | 2 | Case 1: TDM, QPSK, TBS=360, RU=2,Repitition=1 ,SC=12 | fixed | 3.7dB |
|  |  |  | random | ** |
|  | 2 | Case2: FDM, QPSK, TBS=360, RU=2,Repitition=1 ,SC=3 | fixed | 3.9dB |
|  |  |  | random | ** |
| 0.25 | 2 | Case 1: TDM, QPSK, TBS=180, RU=2,Repitition=1 ,SC=12 | fixed | 2.7dB |
|  |  |  | random | ** |
|  | 2 | Case2: FDM, QPSK, TBS=180, RU=2,Repitition=1 ,SC=3 | fixed | 3.1dB |
|  |  |  | random | ** |

Table 2 Sum throughput (Realistic Channel Estimation) SNR gain in dB @ BLER=0.1

| SE per UE (bps/Hz) | Number of UEs | Baseline | MA signature selection |  |
| --- | --- | --- | --- | --- |
|  |  |  |  | 1T2R, TDL-C, DS=300ns |
| 0.5 | 2 | Case 1: TDM, QPSK, TBS=360, RU=2,Repitition=1 ,SC=12 | fixed | 3.3dB |
|  |  |  | random | ** |
|  | 2 | Case2: FDM, QPSK, TBS=360, RU=2,Repitition=1 ,SC=3 | fixed | 2.8dB |
|  |  |  | random | ** |
| 0.25 | 2 | Case 1: TDM, QPSK, TBS=180, RU=2,Repitition=1 ,SC=12 | fixed | 2.3dB |
|  |  |  | random | ** |
|  | 2 | Case2: FDM, QPSK, TBS=180, RU=2,Repitition=1 ,SC=3 | fixed | 1.7dB |
|  |  |  | random | ** |

Figure 2 and 3 shows sum SE vs MCL and per UE data rate vs MCL of NOMA and OMA with realistic channel estimation. The scheme with smaller occupied bandwidth provides a better MCL given sum spectrum efficiency. FDM OMA provides the best MCL but the lowest code rate (i.e., Case 2 OMA SC=3).  That is because the UE transmission power boosts to a narrow bandwidth. TDM OMA (Case 1 OMA SC=12) provide the worst MCL but highest per UE data rate. Targeting to the same sum SE, NOMA provides better MCL compared with TDM OMA (Case 1), however the performance of NOMA (i.e., Case 1 NOMA SC=6) has poor or similar MCL than FDM OMA (i.e., Case 2 OMA SC=3) with the same sum SE.  In cell edge or extended coverage (i.e., larger MCL), FDM OMA provide better sum SE by sacrificing per UE data rate. In good coverage, NOMA can provide larger MCL than OMA TDM with given sum SE.

Observation #2: NOMA has better performance in good coverage.

Proposal: For mMTC, support NOMA at least in cell center coverage.  NB-IoT frame work can be a starting point for mMTC.

Figure 2 Sum SE vs MCL with realistic channel estimation

Figure 2 per UE data rate vs MCL with realistic channel estimation

# Conclusion

In this paper, we provided evaluations on NB-IoT system with and without overloading, and in normal or extreme coverage case. We observed that:

Observation # 1: NOMA provide 2~3dB gain over OMA under NB-IoT frame work.

Observation #2: NOMA has better performance in good coverage.

Proposal: For mMTC, support NOMA at least in cell center coverage.  NB-IoT frame work can be a starting point for mMTC.

# Appendix

Table 1 Simulation assumption

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | SC-FDMA |
| Channel coding | Turbo |
| Numerology | 15kHz subcarrier spacing |
| System Bandwidth | Case 1: 1 PRB Case 2: 6 subcarriers |
| Total allocated resource for transmission | Case 1: 1 PRB x 4 subframes Case 2: 6 subcarriers x 8 subframes |
| Overhead | 2 DMRS symbols, no SRS, i.e., 144 available RE per RB for data transmission |
| Target spectral efficiency | 0.5bps/Hz and 1bps/Hz |
| BS antenna configuration | 2 Rx |
| UE antenna configuration | 1Tx |
| Transmission mode | TM1 (refer to TS36.213) |
| Suggested SNR distribution of multiple UEs | Equal average SNR |
| Propagation channel & UE velocity | TDL-C (400ns) 3km/h for multi-tone |
| Max number of HARQ transmission | 1 |
| Given BLER level (to calculate sum throughput) | 0.1 for 1 transmission |
| Overloading factor | 100%, 200% |
| MA signature random [fixed/random] | fixed |
| Channel estimation [ideal/realistic] | ideal, realistic |
| Timing offset [within CP/beyond CP] | within CP |
| Receiver | SIC |

(a)

(b)

Figure 2 BLER performance of NOMA and OMA in Case I with sum SE = 0.25 and 0.5

(a)

(b)

Figure 3 BLER performance of NOMA and OMA in Case 2 with sum SE = 0.25 and 0.5

Table 4 Sum spectral efficiency v.s. MCL

|  | 2UE in 4 PRB OMA Ideal CE |  |  |  | 2UE in 4PRB NOMA Ideal CE |  |  |  | 2 UE in 4 PRB OMA Real CE |  |  |  | 2 UE in 4PRB NOMA Real CE |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scheme | TBS=360; RU=2; Rep=1; SC=3 | TBS=360; RU=2; Rep=1; SC=12 | TBS=180; RU =2; Rep=1; SC=3 | TBS=180; RU=2; Rep=1; SC=12 | TBS=360; RU =2; Rep=2; SC=6 | TBS=360; RU =2; Rep=2; SC=12 | TBS=180; RU =2; Rep=2; SC=6 | TBS=180; RU =2; Rep=2; SC=12 | TBS=360; RU=2; Rep=1; SC=3 | TBS=360; RU=2; Rep=1; SC=12 | TBS=180; RU =2; Rep=1; SC=3 | TBS=180; RU=2; Rep=1; SC=12 | TBS=360; RU =2; Rep=2; SC=6 | TBS=360; RU =2; Rep=2; SC=12 | TBS=180; RU =2; Rep=2; SC=6 | TBS=180; RU =2; Rep=2; SC=12 |
| overloading | 100% | 100% | 100% | 100% | 200% | 200% | 200% | 200% | 100% | 100% | 100% | 100% | 200% | 200% | 200% | 200% |
| (sum) Spectrum Efficiency (bit/s/Hz) | 1 | 1 | 0.5 | 0.5 | 1 | 1 | 0.5 | 0.5 | 1 | 1 | 0.5 | 0.5 | 1 | 1 | 0.5 | 0.5 |
| Transmitter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (1) Tx Power (dBm) | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 | 23 |
| Receiver |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 | -174 |
| (3) eNB receiver noise figure (dB) | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
| (4) Interference margin (dB) | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| (5) Occupied channel bandwidth (Hz) | 45000 | 180000 | 45000 | 180000 | 90000 | 180000 | 90000 | 180000 | 45000 | 180000 | 45000 | 180000 | 90000 | 180000 | 90000 | 180000 |
| (6) Effective noise power = (2)+(3)+(4)+10 log ((5)) (dBm) | -124.5 | -118.4 | -124.5 | -118.4 | -121.5 | -118.4 | -121.5 | -118.4 | -124.5 | -118.4 | -124.5 | -118.4 | -121.5 | -118.4 | -121.5 | -118.4 |
| (7) Required SINR (dB) | 4.1 | 2.1 | 0.2 | -1.6 | -1.3 | -1.8 | -2.9 | -4.4 | 5.9 | 3.5 | 1.4 | -0.5 | 3.0 | 0.1 | -0.4 | -2.9 |
| (8) Receiver sensitity =(6)+(7) (dBm) | -120.3 | -116.4 | -124.3 | -120.1 | -122.8 | -120.3 | -124.3 | -122.8 | -118.6 | -115.0 | -123.1 | -119.0 | -118.4 | -118.3 | -121.8 | -121.3 |
| (9) Reciever processing gain | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| (10) MCL = (1) - (8) + (9) (dB) | 143.3 | 139.4 | 147.3 | 143.1 | 145.8 | 143.3 | 147.3 | 145.8 | 141.6 | 138.0 | 146.1 | 142.0 | 141.4 | 141.3 | 144.8 | 144.3 |