3GPP TSG RAN WG1 Meeting #87	R1-1611232

Reno, USA, November 14-18, 2016

Agenda Item:	7.1.6.1

Source:	Huawei, HiSilicon

Title:	Evaluation of flexible duplex in dense urban scenario

Document for:	Discussion and decision

Introduction

In RAN1 #86bis meeting, it was agreed that [1]

- Continue study considering some or all of the following aspects:

  - Deployment scenarios/bands, same-/cross-operator considerations

  - Resource assignments and rate adaptations

  - Frame structure and HARQ/scheduling timing

  - Measurements for cross-link interference management

  - Signalling (e.g., OTA, backhaul, UE capability, etc.)

  - Cross-link interference management (IC/IS, power control, etc.)

  - Centralized vs. distributed interference/resource management

  - Beamforming/MIMO

  - Duplex modes (e.g., FDD/TDD, FDM/TDM, etc.)

  - Latency reduction

  - Whether or not LTE interference/resource management can be used as a starting point (as applicable)

  - Sensing

  - RS design

  - Advanced receiver

  - Timing alignment between DL and UL

Flexible duplex allows flexible resource allocation among different transmission directions, for both paired and unpaired spectrum. To study the feasibility as well as to identify the challenges of flexible duplex on paired and un-paired spectrum in various deployment scenarios, comprehensive evaluations are needed. In RAN1 #86 meeting, some evaluation assumptions for flexible duplex were agreed [2][3][4]. Dense urban, urban macro and indoor hotspot would be considered.

In this contribution, some preliminary evaluation results for dense urban with beam coordination and advanced IRC receiver are provided and discussed.

Discussion on key simulation assumptions

In this section, some simulation scenarios and assumptions are discussed. These key assumptions are important for identifying the technical feasibility and challenges of flexible duplex. For detailed system-level simulation parameters, the parameters in Appendix A are used, which are aligned with RAN1#86 agreements [2][3][4].

Network Layout

In this contribution, the focus is on one operator scenario, and the two-layer layout is used where the Macro layer and Micro layer use different carrier frequencies. Such configuration is one of typical deployments that might be employed in two-layer scenario. The detailed layout set is as follows:

- Macro layer: Hex. Grid

- Micro layer:

  - Random drop (All micro TRPs are all outdoor)

  - 3 micro TRPs per macro TRP area

For carrier frequency, 4GHz and 30GHz with unpaired spectrum assumption are evaluated. It is assumed that 30GHz is only deployed on micro layer and 4GHz is only deployed on macro layer as follows.

- 4GHz: Macro layer only (assuming part of UEs accessed to 30GHz Micro layer)

- 30GHz: Micro layer only (assuming part of UEs accessed to 4GHz Macro layer)

Downlink and uplink resource allocation

The downlink and uplink resource allocation for unpaired spectrum is discussed in [5]. To study the potential benefit of flexible duplex for unpaired spectrum, the baseline employs a semi-static subframe allocation, and all TRPs share the same subframe ratio allocation.

In RAN1 #86 meetings [2], it was agreed that FTP traffic model 3 with packet size 0.5M bytes can be used and the ratio of DL/UL traffic load are {1:1}, {2:1}, {4:1}.

Table 1 provides the summary of the employed subframe allocation for baseline and flexible duplex.

Table 1: simulation setup parameters and resource allocations

|  | Baseline (Traditional TDD) | Flexible duplex |
| --- | --- | --- |
| DL/UL subframe ratio | semi-static subframe allocation - For DL:UL traffic ratio = 1:1 or 2:1, DL/UL subframe = 6:4 - For DL:UL traffic ratio = 4:1, DL/UL subframe = 4:1 | Flexible UL/DL subframe ratio allocation |
| DL/UL carrier bandwidth | 4GHz: 20MHz (DL+UL) 30GHz: 80MHz (DL+UL) |  |

Modeling of cross-link interference mitigation (CIM)

In flexible duplex, cross-link interference, e.g. TRP-to-TRP and UE-to-UE interference, exists in case that neighboring cells use different transmission directions on the same time-frequency resource. An example is shown in Figure 1.

Figure 1: Example of cross-link interference

In the simulation, the cross-link interference is modeled with appropriate propagation characteristics as given in [3] and [4]. It is noted that for dense urban scenario, 80% UEs are indoor UEs and 20% UEs are outdoor UEs, and the UE-to-UE cross-link refers to the link between UEs connected to different TRPs. In this case, it is very likely such cross-link is between UEs in different bulidings. In such cases, the  peneration loss would be large. The UE-to-UE cross link interference in  dense urban scenario is therefore not very severe. On ther other hand, all TRPs are outdoor, with large transmission powers. And the height of TRPs are higher than UE side, which results in higher probability of LOS channel betewen TRPs. All these factors lead to severe interference between  TRP-to-TRP cross-link and will severely degrade the uplink performance. To assure the performance gain, cross-link interference management schemes need to be considered [6].

In NR, MIMO is a key feature to improve the system throughput. For cross link interference mitigation, beamforming and beam nulling is also promising. For TRP-to-TRP links, it is noted that the angle spread is small in both arrival angle and departure angle according to the channel model, so beamforming and beam nulling is effective to mitigate a number of TRP-to-TRP links interference simultaneously. In LTE studies such as NAICS and MUST etc, advanced receivers are studied and are used for interference suppression. In [6], it is observed that advanced receiver IRC can significantly reduce the cross link interference level with low complexity.

In this contribution, massive MIMO is used, for the TRP-to-TRP and UE-to-UE interference mitigation, beam coordination and advanced IRC receiver are used to mitigate the influence of the cross-link interference [6] to assure the performance.

Simulation results

This section shows the preliminary evaluation results based on the above simulation assumptions.

4GHz

In this section, the evaluation results of flexible duplex for 4GHz un-paired spectrum case are presented in Table 2. The DL-UL subframe ratio of 6:4 and 4:1 are used in baseline, as discussed in section 2.2. The flexible duplex uses the dynamic subframe ratio allocation, which is according to the traffic load. Note that for 4GHz, only macro layer is considered.

Table 2: DL and UL Performance on 4GHz carrier

| Ratio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | DL Average UPT Gain | UL RU (%) | UL Average UPT (Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1:1 | Baseline | 6:4 | 7.36 | 55.23 |  | 24.09 | 18.54 |  |
|  | Flexible duplex | Dynamic change |  | 65.08 | 17.84% |  | 20.06 | 8.19% |
| 2:1 | Baseline | 6:4 | 6.54 | 55.16 |  | 11.42 | 20.11 |  |
|  | Flexible duplex | Dynamic change |  | 69.40 | 25.82% |  | 21.02 | 4.55% |
| 4:1 | Baseline | 4:1 | 5.93 | 75.84 |  | 10.95 | 10.34 |  |
|  | Flexible duplex | Dynamic change |  | 80.22 | 5.77% |  | 19.33 | 86.92% |

It is observed from the simulation that there are occasions when the network needs to serve DL packet only, or UL packet only. In this case, flexible duplex enjoys the flexibility of allocating DL/UL subframe ratio appropriately to match the instantaneous need of DL and UL traffic, e.g., to allocate a high DL resource ratio when there is dominant DL traffic. In this case, the DL radio resource is boosted compared to static DL/UL subframe ratio allocation, where the UL resource ratio is always reserved irrespective whether there is UL traffic. Such resource increase benefits the user perceived throughput.

Based on the above simulation results, we have the following observation.

Observation 1:

Flexible duplex in 4GHZ can significantly improve both the downlink and uplink performance. Preliminary evaluation results show that with cross-link interference mitigation, it achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 87%) and downlink (e.g., up to 26%) in dense urban scenario.

30GHz

In this section, the evaluation results of flexible duplex for 30GHz un-paired spectrum case are presented in Table 3. The DL-UL subframe ratio of 6:4 and 4:1 are used as the baseline, as discussed in section 2.2. The flexible duplex uses the dynamic subframe ratio allocation, which is according to the traffic load. Note that for 30GHz, only micro layer is considered.

Table 3: DL and UL Performance on 30GHz carrier

| Ratio of DL/UL traffic | Feature | DL/UL subframe ratio | DL RU (%) | DL Average UPT (Mbps) | DL Average UPT Gain | UL RU (%) | UL Average UPT (Mbps) | UL Average UPT Gain |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1:1 | Baseline | 6:4 | 8.41 | 208.48 |  | 24.37 | 64.52 |  |
|  | Flexible duplex | Dynamic |  | 268.94 | 29.00% |  | 100.78 | 56.19% |
| 2:1 | Baseline | 6:4 | 8.05 | 213.58 |  | 11.51 | 73.56 |  |
|  | Flexible duplex | Dynamic |  | 281.15 | 31.64% |  | 105.57 | 43.50% |
| 4:1 | Baseline | 4:1 | 9.50 | 274.90 |  | 17.57 | 36.09 |  |
|  | Flexible duplex | Dynamic |  | 291.71 | 6.12% |  | 93.35 | 158.62% |

Similar to 4GHz case, flexible duplex could bring the flexibility on subframe ratio allocation, which could adapts better to the instantaneous need of DL and UL traffic. Further the cross-link interference could be handled by interference mitigation schemes. Therefore the following observation is made.

Observation2:

Flexible duplex in 30GHz can significantly improve both the downlink and uplink performance. Preliminary evaluation results show that with cross-link interference mitigation, it achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 159%) and downlink (e.g., up to 32%) in dense urban scenario.

Conclusion

In this contribution, we provided some preliminary evaluation results of flexible duplex in dense urban scenario. The observation is summarized as follows.

Observation 1:

Flexible duplex in 4GHZ can significantly improve both the downlink and uplink performance. Preliminary evaluation results show that with cross-link interference mitigation, it achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 87%) and downlink (e.g., up to 26%) in dense urban scenario.

Observation2:

Flexible duplex in 30GHz can significantly improve both the downlink and uplink performance. Preliminary evaluation results show that with cross-link interference mitigation, it achieves considerable gain over TDD with static subframe ratio allocation in uplink (e.g., up to 159%) and downlink (e.g., up to 32%) in dense urban scenario.

- References

[1]	3GPP, Chairman's Notes, RAN1 #86bis, Oct 2016.

[2] R1-168053, “WF on scenarios and evaluation assumptions for flexible duplex,” RAN1#86, LG Electronics, Huawei, HiSilicon, Gothenburg, Sweden, Aug 22 - 26, 2016.

[3] R1-168372, “WF on channel model issues for flexible duplex evaluation,” RAN1#86, Huawei, HiSilicon, Intel, Gothenburg, Sweden, Aug 22 - 26, 2016.

[4] R1-168373, “WF on penetration issues for flexible duplex evaluation,” RAN1#86, Huawei, HiSilicon, LG Electronics, Gothenburg, Sweden, Aug 22 - 26, 2016.

[5] 3GPP TR 36.828 V2.0.0, “Further Enhancements to LTE TDD for DL-UL Interference Management and Traffic Adaptation,” June 2012.

[6] R1-1611225, “Benefits of cross-link interference mitigation,” RAN1#87, Huawei, HiSilicon, Reno, USA, 14-18 November, 2016.

Appendix A

| Parameters | Dense Urban |
| --- | --- |
| Layout | Two layers - Macro layer: Hex. Grid - Micro layer: Random drop (All micro TRPs are all outdoor) - 3 micro TRPs per macro TRP |
| Inter-BS distance | Macro-to-macro: 200m Macro-to-micro: 105m [TR36.897] Micro-to-micro: 57.9m |
| Minimum BS-UE (2D) distance | Macro-to-UE: 35m [TR36.897] Micro-to-UE: 10m [TR36.897] |
| Minimum UE-UE (2D) distance | 3m |
| Carrier frequency | 4GHz, 30GHz |
| Aggregated system bandwidth | 4GHz: Up to 200MHz (DL+UL) 30GHz: Up to 1GHz (DL+UL) |
| Simulation bandwidth | 4GHz: One carrier with 20MHz (TDD) 30GHz: One carrier with 80MHz (TDD) |
| Channel model | Below 6GHz: Macro-to-UE: 3D UMa Micro-to-UE: 3D UMi Macro-to-Macro: 3D UMa (h_UE=25m) Macro-to-Micro: 3D UMa (h_UE=10m) Micro-to-Micro: 3D UMi (h_UE=10m) UE-to-UE: A.2.1.2 in TR36.843 Above 6GHz: Macro-to-UE: 5GCM UMa Micro-to-UE: 5GCM UMi Macro-to-Macro: 5GCM UMa (h_UE=25m) Macro-to-Micro: 5GCM UMa (h_UE=10m) Micro-to-Micro: 5GCM UMi (h_UE=10m) UE-to-UE: 5GCM UMi (h_BS=1.5m ~ 22.5m) |
| Penetration loss | Follow [4] |
| Fast fading channel model | Below 6GHz: Macro-to-UE: 3D UMa Micro-to-UE: 3D Umi Macro to Macro: 3D UMa O-to-O (h_UE=25m); ASA and ZSA statistics* updated to be the same as ASD and ZSD; ZoD offset = 0 Macro to Micro: 3D Uma O-to-O Micro to Micro: 3D Umi O-to-O (h_UE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 UE to UE: InH for indoor to indoor, and 3D Umi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. Dual mobility support. |
| BS Tx power | Micro layer: 4 GHz: 33dBm for 20MHz system bandwidth Above 6GHz: 33 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 33 dBm. Macro layer: Below 6GHz: 44 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. Otherwise, 44 dBm |
| UE Tx power | Maximum 23 dBm |
| BS antenna configuration | Below 6GHz: Baseline: (M,N,P,Mg,Ng)=(8,8,2,1,1) (dH,dV)=(0.5,0.8)λ Above 6GHz: Baseline: (M,N,P,Mg,Ng)=(8,4,2,2,2) (dH,dV,dH,g,dV,g)=(0.5,0.5,4.0,2.0)λ |
| BS antenna configuration |  |
| BS antenna height | Macro: 25m Micro: 10m |
| BS antenna element gain + connector loss | 8 dBi |
| BS antenna tilt | Macro:108deg Micro:105deg |
| BS receiver noise figure | Below 6GHz: 5 dB Above 6GHz: 7 dB |
| UE antenna configuration |  |
| UE antenna elements | For 4GHz: 2Tx and 2Rx For around 30GHz: (M, N, P, Mg, Ng)=(2,4,2,1,2), the polarization angles are 0 and 90deg (dH, dV) = (0.5, 0.5)λInter-panel distance: 6.0λ |
| UE antenna | for outdoor UEs: 1 for indoor UEs: ~uniform(1, ) where ~uniform(4,8) |
| UE antenna gain | For below 6GHz: Follow the modeling of TR36.873 For 30GHz: 5dBi |
| UE receiver noise figure | Below 6GHz: 9 dB Above 6GHz: 13 dB |
| Traffic model | FTP traffic model 3 with packet size 0.5Mbytes Ratio of DL/UL traffic = {1:1}-the DL User arrival rate λ is 0.1 {2:1}-the DL User arrival rate λ is 0.2 {4:1}-the DL User arrival rate λ is 0.2 |
| UE distribution | For FTP traffic model 3: 2/3 users randomly and uniformly dropped within the clusters, 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area 80% indoor (3km/h) and 20% outdoor (30km/h) |
| UE receiver | MMSE-IRC |
| BS receiver | MMSE-IRC |
| UE association | UE connected to Macro or Micro layer, based on RSRP measurement |
| Transmission mode | SU-MIMO |