3GPP TSG RAN WG1 Meeting #87	R1-1611323

Reno, USA 14th - 18th November 2016

Source:	Ericsson

Title:	Improved LDPC Performance for Short Block Length

Agenda Item:	7.1.5.1

Document for:	Discussion

# Introduction

In RAN1#86bis, the following agreements on channel coding schemes for eMBB data were reached [1]:

Agreement:

- The channel coding scheme for eMBB data is LDPC, at least for information block size > X

- FFS until RAN1#87 one of Polar, LDPC, Turbo is supported for information block size of eMBB data <= X

  - The selection will focus on all categories of observation, including overall implementation complexity, regardless of the number of coding schemes in the resulting solution (except if other factors are generally roughly equal)

- The value of X is FFS until RAN1#87, 128 <= X <= 1024 bits, taking complexity into account

In this contribution, we propose to use a quasi-ML decoder, ordered statistics decoder (OSD) [2], for decoding short block lengths and show performance results for LDPC codes proposed in [3].

# Proposed LDPC Codes Under Quasi-ML Decoding

We demonstrate the performance of the proposed LDPC codes under ordered statistics decoding (OSD) algorithm, a realization of soft-decision nearly-ML-optimal decoding [2]. LDPC designs are based on a rate-compatible base graph (BG), designed for a certain range of information block lengths and code rates. The specification of the proposed BG for NR which is used in this contribution is summarized in Table 1, see also our contribution [3] for more details on the LDPC code designs.

Table 1 LDPC base graph

|  | Kb,max | Kb,min | Kmax | Kmin | Npunc | Rmax | Rmin | Available Z values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base graph 3 | 6 | 4 | 336 | 32 | 2xZ bits | 1/2 | 1/5 | Lift 1: 8, 10, 12, 14 |

For decoding, we employ the OSD scheme with different user-defined orders, determined by l=0,1,…, K and K being the number of information block length. In general, the OSD algorithm consists of two main steps:

- Permutation: it permutes the code’s generator matrix based on decreasing level of reliability measures for received codewords to determine the most reliable basis of the code.

- Re-encoding: Based on the permuted generator matrix, it checks all possible error patterns containing l bit errors (aka order l) by adding it to the hard-decision decoded information bits, and re-encodes the resulting codeword. Finally, it finds the minimum Euclidean distance between the re-encoded codewords and the received codeword.  The re-encoded codeword with the smallest Euclidean distance is designated as the estimated codeword.

The order l specifies a tradeoff between the degree of complexity and performance of the decoding algorithm. In short, the higher the degree is chosen, the better the performance becomes but the higher the complexity gets. The details of the OSD algorithm are described in [2]. It can be also verified that the decoder complexity grows at most like Ο(kl). Further refinements of the OSD algorithm, in terms of computation complexity, have been proposed in [4,5].

# Simulation Results

In this section, we demonstrate the performance of different coding schemes (in terms of FER vs. EsN0) for a short block length. As an investigation example, information block size of K=40 is used. Competing codes include LDPC codes under the proposed OSD and sum-product algorithm (SPA) with 30 iterations, Tail-biting Turbo Codes (TBTC) [6], Tail-biting Convolutional Codes (TBCC) [7] and Polar codes [8]. In this study, we used the best performing Polar codes that we could find. The simulated Polar code used an optimized info set, which is shown to achieve better performance than PC-Polar [9] in our study.

We choose information block length K=40, rate R=1/3 and QPSK modulation. Other parameters are summarized in Table 2 in Appendix. Observing the results of LDPC codes in Figure 1, under SP and OSD algorithms, we witness a strong potential inherent to LDPC codes -- at the expense of increasing decoding complexity we obtain significant gains. Employing the quasi-ML OSD algorithm instead of iterative-based SPA, the performance of LDPC codes can be improved considerably. For example, at a FER of 1%, we can obtain 0.8 dB gain by using order-2 OSD and 1.5 dB gain by using order-3 OSD instead of SPA.

Also, by comparing LDPC+OSD and Polar codes in Figure 1, it is observed that using order-3 and order-4 OSD together with LDPC codes, 0.4 dB and 0.6 dB improvements, respectively, can be obtained over Polar codes at a FER of 1%.

The results for block length K=40, rate R=1/2 are included in Figure 2. The same behavior described above can be also seen from the curves in Figure 2.

- For short block lengths, LDPC codes with OSD decoding algorithm outperforms LDPC with SPA.

- For short block block lengths, with quasi-ML decoding algorithms, LDPC codes have the potential to deliver comparable FER performance as other code types including TBCC, TBTC, and Polar codes.

Figure 1.	Performance comparison between LDPC+OSD, Polar, TBCC, TBTC, LDPC+SPA.  Code rate = 1/3, K = 40 and QPSK.

Figure 2.	Performance comparison between LDPC+OSD, Polar, TBCC, TBTC, LDPC+SPA.  Code rate = 1/2, K = 40 and QPSK.

There are other versions of OSD algorithm fitted for LDPC codes, see e.g., [5], as well as other refinements on it for reducing complexity, see e.g., [10], [11]. Also, in [12], comparisons among coding schemes for short block length have been made, showing significant gain achieved by using LDPC codes under the OSD algorithm.

Other low-complexity algorithms for LDPC codes, such as approximate min-sum (AMS), saturated min-sum (SMS), combination of AMS and SMS, and combination of OSD with the mentioned decoding algorithms can be also examined which could result in different performance-complexity tradeoffs.

# Conclusion

In this contribution, we have demonstrated the performance of our proposed LDPC codes under the quasi-ML OSD algorithm. Based on our results in Figure 1 and Figure 2, we have the following observations:

- For short block lengths, LDPC codes with OSD decoding algorithm outperforms LDPC with SPA.

- For short block lengths, with quasi-ML decoding algorithms, LDPC codes have the potential to deliver comparable FER performance as other code types including TBCC, TBTC, and Polar codes.

# Appendix

Table 2 Simulation parameters

| Channel | AWGN |  |  |  |
| --- | --- | --- | --- | --- |
| Modulation | QPSK |  |  |  |
| Coding Scheme | LDPC | TBTC | TBCC | Polar |
| Code rate (for evaluation purposes) | ½, 1/3 |  |  |  |
| Decoding algorithm | OSD, SPA (30 iterations) | Max-log-MAP (0.75 scaling factor, 8 iterations) | List-Viterbi | SC list |
| Info. block length | 40 |  |  |  |

# References

“Chairman's notes RAN1#86bis of AI 8.1.3 on channel coding and modulation for new radio interface”, October 2016.

Marc P. C. Fossorier and Shu Lin, “Soft-Decision decoding of linear block codes based on ordered statistics,” IEEE Transactions on Information Theory, Vol. 4, No. 5, September 1995.

R1-1611321, “Design of LDPC codes for NR”, Ericsson, November 2016.

Marc P. C. Fossorier and Shu Lin, “Computationally efficient soft-decision decoding of linear block codes based on ordered statistics,” IEEE Transactions on Information Theory, Vol. 42, No. 3, May 1996.

Marc P. C. Fossorier, “Iterative reliability-based decoding of low-density parity check codes,” IEEE Journal on Selected Areas in Communications, Vol. 19, No.5, May 2001.

R1-1611325, “Further Enhanced Turbo Codes for NR”, Ericsson, November 2016.

R1-166926, “Further Discussion on TBCC Complexity and Enhancements,” Ericsson, August 2016.

R1-167992, “ Performance Evaluation of Enhanced TBCC and Polar Codes, ” Ericsson, August 2016.

R1-1608862, “Polar Code Construction for NR, ” Huawei, HiSilicon, October 2016.

Wu, Yingquan, and Christoforos N. Hadjicostis. "Soft-decision decoding using ordered recodings on the most reliable basis." IEEE transactions on information theory, Vol 53, No. 2

Wu, Yingquan, and Marc Fossorier. "Soft-decision decoding using time and memory diversification."  IEEE International Symposium on Information Theory, 2008.

Van Wonterghem, J., Alloum, A., Boutros, J. J., & Moeneclaey, M., “Performance Comparison of Short-Length Error-Correcting Codes,” arXiv preprint, September 2016.