3GPP TSG RAN WG1 Meeting #87																R1-1611969

Reno, Nevada, U.S.A., 14-18 November 2016

Source: 	Intel Corporation

Title:	On NR initial access and mobility

Agenda item:	7.1.2.4

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss initial access and mobility aspects relevant to RAN1 based on agreements made in RAN1 as well as RAN2 from previous meetings.

The following agreement regarding mobility were made in RAN1 #86bis.

Agreements regarding NR cell - Detailed definition of NR cell FFS - Note: cell refers to NR cell which is tied to a same ID carried by NR-SS. Agreements regarding mobility in IDLE mode - Note: IDLE mode refers to a UE state similar to LTE IDLE state, whose exact definition is up to RAN2 - NR supports cell-level mobility based on DL cell-level measurement (e.g. RSRP for each cell) in IDLE mode UE - Study the following DL signals for IDLE mode RRM measurement - Option 1: Synchronization signal (e.g., NR-PSS, NR-SSS) - Option 2: RS for demodulating broadcast channel - Option 3: RS for mobility - FFS if and how to associate the cell ID with this RS - FFS this RS is for multi-beam and/or single-beam - Option 4: Any combinations of above - Other options are not precluded - FFS: QCL definition for DL signal for IDLE mode RRM measurement - FFS whether NR cell is defined only for “IDLE mode” or for both IDLE and CONNECTED mode Agreements regarding mobility in CONNECTED mode - Note: CONNECTED mode refers to a UE state similar to LTE CONNECTED state, whose exact definition is up to RAN2 - For L3 mobility based on DL measurement in CONNECTED mode UE: - At least non-UE-specific DL signals can be used for CONNECTED mode RRM measurement - FFS UE-specific DL signals for this purpose - Study the following DL signals for CONNECTED mode RRM measurement - Option 1: Cell related RS which is carrying Cell-ID (e.g. NR-PSS, NR-SSS) - Option 2: RS for mobility - FFS how to associate it with beam-ID and/or Cell-ID - Option 3: RS for demodulating broadcast channel - Option 4: A combination of option 1 and 2 - Other options are not precluded - At least one of cell-level and beam-level measurement quantities is supported for RRM reporting. - FFS which RRM measurement quantities to define, e.g., RSRP, RSRQ - Study the following options for RRM measurement quantities to be reported for L3 mobility: - Option 1: derived per cell (e.g., if multi-beam, as a function of multi-beam measurements) - Option 2: derived per beam - Option 3: A combination of option 1 and 2 - Other options are not precluded - FFS for other UE states (if introduced by RAN2)

The following agreements regarding new ‘state’ and UE mobility were made in RAN2 #95bis.

Agreements on new ‘state’ - Modelling will be concluded when functionality is understood (maybe in WI phase). For the scope of the study we will refer to RRC_INACTIVE. - A UE and at least 1 gNB should keep the AS context information in the RRC_INACTIVE state. - Limit number of radio network identifiers - In the RRC_INACTIVE state, a UE location can be known at the RAN based area level where that area may be a single cell or more than one cell. Area is determined by the network. Agreements on mobility in RRC_CONNECTED mode - Agreements for DL-based mobility in RRC_CONNECTED mode (optimized for data transmission, at least for network-controlled mobility) mobility with RRC involvement, concerning beams and the relation to the NR cell definition: - UE at least measures one or more individual beams and gNB should have mechanisms to consider those beams to perform HO. Note: This is necessary at least to trigger inter-gNB handovers and to optimize HO ping-pongs / HO failures. - FFS: whether UE report individual and/or combined quality of multiple beams - UE should be able to distinguish between the beams from its serving cell and beams from non-serving cells for RRM measurements in active mobility. UE should be able to determine if a beam is from its serving cell. - FFS whether serving/non serving cell may be termed 'serving/non serving set of beam) - FFS: whether the UE is informed via dedicated signalling or implicitly detected by the UE based on some broadcast signals. - FFS how the cell in connected relates to the cell in idle - Study how to derive a cell quality based on measurements from individual beams - In connected mode, intra-cell mobility can be handled by mobility without RRC involvement. - FFS whether there may be cases that do require RRC involvement. - UE should be able to identify a beam. FFS how beams are identified (to be defined by RAN1) Agreements on mobility in IDLE mode - In IDLE mode, UE performs cell selection and reselection on NR Cells. - Study how to derive a cell quality based on measurements.

The following agreements regarding UL-based mobility were made in RAN2 #95bis.

Agreement on further investigation of UL-based connected mode mobility - Concerning RRC driven UL-based connected mode mobility - For connected active state mobility, DL-based handover is supported, and UL based mobility can continue to be studied. - For connected inactive state, DL-based reselection is supported, and UL-based mobility can also be studied - Benefits of UL based mobility, compared to DL based mobility, should be studied with performance analysis.

# Mobility Support during Initial Access and Handover

In general scenarios for designing initial access and handover, UE speed needs to be considered. For instance in LTE, there was the requirements in [1] that the system shall support mobility across the cellular network for various mobile speeds up to 350km/h. Initial access is the earliest step before real data service, it is important to ensure the supportability in such high speed case. For instance, when PRACH was designed in LTE, a cyclic shift restriction (high speed flag) was introduced considering the high speed case covering up to 1.25kHz dopper frequency – later in Rel-14, the WI of performance enhancement of high speed scenario is being worked in [2] in order to support up to 750km/h, which may require to update the PRACH preamble sequence generation accordingly further.

For NR [3], the requirement for mobility is stated that the target for mobility target should be 500km/h. Therefore, the supportability to fulfil the NR requirement needs to be considered in initial access and handover. In addition to potential design impact as discussed in LTE example, beam tracking operation in initial access and handover also needs to be discussed together. For instance, when UE is close to eNB and UE is moving towards the direction perpendicularly to the eNB, the beam tracking may not be always available. Once again, initial access needs to be at least supportable since it is the earliest step before any data communication.

Figure 1 shows the high speed analysis according to the different UE speed assumption. We look at the elapsed time from peak main lobe of beamforming to 3dB loss. UE is assumed to move towards the direction perpendicularly to the eNB. X-axis represents the distance between eNB and UE, and Y-axis represents elapsed time to lose the signal by 3dB. NTX is the number Tx antennas for evaluating beamforming. It is noted that the starting point of the evaluation is quite optimistic by assuming the best beam selection at the time for eNB Tx beam detection, but in general the detected beam may not be corresponding to the peak lobe.

(a) UE speed = 30km/h                                                                 (b) UE speed = 120km/h

(c) UE speed 300km/h

Figure 1 High speed analysis – elapsed time from peak main lobe to 3dB loss of link budget according to different UE speed

Proposal 1: The design of initial access and handover should support high speed scenario up to 500km/h.

# Initial Access & Handover - NR Cell level vs. TRP level

The Cell Search for NR is the first step to acquire time/frequency synchronization and system information before data communication occurs. Typically, initial access procedure may comprise Cell Search and RACH procedures.

In LTE, on the given carrier frequency, a cell has been assumed as an individual entity for Tx/Rx although there has been no clear definition. For instance, an eNB may comprise multiple sectors and/or multiple TPs. The individual entity for each sector and/or TP is called a cell and a PCID (Physical Cell ID) is assigned for the individual entity (i.e. sector and/or TP). In consequence, the specification only describes ‘cell’ although the study assumed many practical deployment scenarios such as sectorization, small cell scenario (with the same/different PCIDs among the TPs within a macro cell area which are often referred by CoMP Scenario 3/4) and so on. In addition to the existing PCID, a further ID for CSI-RS has been configured to distinguish the different TPs in Rel-12 small cell enhancement WI and recently configurable ID for PRS has been agreed in Rel-14 indoor positioning WI. Both were mainly motivated by the same PCID scenarios within a macro cell area.

In NR, the same assumption can hold – on the given carrier frequency, an NR cell comprises one or multiple TRPs. NR DL synchronization may refer to the operation to find NR cell ID and further TRP-ID and/or Beam-ID (if defined) can be identified in the later stage than NR DL synchronization. Instead, the DL synchronization step may comprise to find both NR Cell ID and NR TRP-ID/Beam-ID.

We discuss two options for initial access and mobility – (1) TRP based approach and (2) NR Cell (which comprises one or multiple TRPs) based approach.

Figure 2 illustrates Option 1 (TRP level approach) for initial access and mobility. During the ‘Cell Search’ procedure, UE tries to find a TRP carrying individual TRP ID. The different NR PSS/SSS/PBCH/SIB1 (note: we simply shorten the other SI after the system information SIB1 for easier discussion) are carried from each TRP. The RRM measurement and handover can be based on each TRP. In high speed UE, the more frequent handover is expected and thus the higher layer signalling will be involved at every handover event. The overhead resulting from handover may be increased as such and in LTE Hetnet mobility study it shows that handover failure can more frequently happen when the target cell is a small cell. The beam sweeping can be considered during initial access on synchronization signals especially for >6GHz frequency – though, an independent RS needs to be supported for fine granularity of the beam sweeping (e.g. by TRP-specific RS). The UE specific or TRP specific data transmission can be based on TRP potentially by using beamforming operation. It is observed that with TRP level approach:

- All channels/signals rely on TRP specific transmission.

- Cell Search implies that UE tries to find individual TRP.

- The RRM measurement for handover is based on TRP level and the handover is performed across TRPs.

  - In high speed UE, more handover event may happen and the corresponding signalling overhead may be increased.

  - TRP specific beamforming/sweeping operation can be applied during initial access procedure.

- Due to independent signal/channel from different TRPs, SFN (Single Frequency Network) gain from multiple cells may not be expected.

- RSRP/RSRQ measurements for handover are based on TRP-specific RS possibly with beam sweeping operation (i.e. RSRP/RSRQ per beam).

- As TRP ID needs to be defined in a hardwired manner (like PCID in LTE) to support initial access and idle mode operation, the flexibility of reconfiguration for TRP ID is limited.

Figure 2 Option 1 – TRP level approach for initial access and mobility

Figure 3 illustrates Option 2 (NR Cell level approach) for initial access and mobility. An NR Cell comprises one or multiple TRPs. During the ‘Cell Search’ procedure, UE tries to find an NR Cell carrying NR Cell ID. As one example, a common NR PSS/SSS/PBCH/SIB1 are carried from all TRPs and UE would receive the signal/channel in a composite manner within the same NR Cell which can exploit SFN gain from different TRPs. The system information can be minimized as the very essential system information. The RRM measurement and handover can be based on each NR Cell, which could potentially minimize the handover event in high speed UE with heavy signalling overhead since the handover will only happen across the different NR Cells. In general, in order to leverage SFN gain due to the same data/signal transmission from the different TPs within the NR Cell, a relatively larger beamwidth might need to be used during initial access procedure. Likewise in Option 1, an independent RS needs to be supported for beam acquisition (e.g. by TRP-specific RS). The UE specific or TRP specific data transmission can be based on TRP potentially by using beamforming operation. Since handover is based on NR Cell level, some TRPs (e.g. in edge of NR Cells) may be shared between NR Cells (i.e. TRPs do not need to be exclusively a part of each NR Cell). It is observed that with NR Cell level approach:

- NR PSS/SSS/PBCH/SIB1 are based on NR Cell specific transmission while the other channels/signals can be TRP/Cell specific.

- Cell Search implies that UE tries to find the NR Cell.

- The RRM measurement for handover is based on NR Cell level and the handover is performed across different NR Cells.

  - In high speed UE, less handover event with corresponding less signalling overhead is expected.

  - In order to exploit SFN gain within an  NR Cell, a relatively larger beamwidth might need to be used during initial access procedure.

    - Such robust channel using SFN without beam operation can be used potentially for fall-back operation of beam based data transmission.

- Potentially more flexibility is expected by reconfiguring TRP ID per TRP.

Figure 3 Option 2 – NR Cell level approach for initial access and mobility

Table 1 summarizes the comparison between Option 1 (TRP level approach) and Option 2 (NR Cell level approach). In our companion contribution [4], the detection performance with single beam operation (with repetition) shows the good performance. Along with the advantages of Option 2 (NR Cell level approach), it is deemed to consider to take NR Cell level approach as a baseline and further TRP ID finding can be studied.

Table 1 Comparison between Option 1 (TRP level approach) and Option 2 (NR Cell level approach)

|  | Option 1: TRP level approach | Option 2: NR Cell level approach |
| --- | --- | --- |
| TRP based Tx | All channels/signals | All others than PSS/SSS/MIB/SIBx/Paging |
| NR Cell based Tx | N/A | PSS/SSS/MIB/SIBx, (potential) fall-back mode/schemes |
| Handover | TRP based | NR Cell based |
| Signaling overhead for measurement/HO in high speed | Potentially larger | Potentially smaller |
| HO frequencies in small cell | Potentially more often | Potentially less often |
| SFN gain | Not applicable | Applicable |
| TRP ID acquisition | Via NR PSS/SSS | Via TRP specific RS possibly with assistant information from PBCH/SIBx |
| TRP ID reconfiguration | Fixed | Flexible |

Proposal 2: NR DL Synchronization, system information acquisition and paging reception are based on NR Cell level.

Proposal 3: NR handover is based on NR Cell level (i.e. handover happens between NR Cells).

# Conclusions

In this contribution we discussed initial access framework and mobility aspects. Our views are summarized as below:

Proposal 1: The design of initial access and handover should support high speed scenario up to 500km/h.

Proposal 2: NR DL Synchronization, system information acquisition and paging reception are based on NR Cell level.

Proposal 3: NR handover is based on NR Cell level (i.e. handover happens between NR Cells).

# References

- 3GPP TR 36.913, “Requirements for further advancements for Evolved Universal Terrestrial Radio Access (E-UTRA) (LTE-Advanced)”

- WID: New WI proposal: Performance enhancements for high speed scenario, NTT DOCOMO, Huawei, HiSilicon

- 3GPP TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”

- R1-1610378, “Preliminary performance evaluation of different beamforming options for NR synchronization signals”, Intel