3GPP TSG-RAN WG1 #87								  			                             			R1-1611972

Reno, USA 14th - 18th November 2016

Source: 	Intel Corporation

Title:	Discussion on NR transmit diversity scheme

Agenda item:	    7.1.3.1

Document for:	Discussion and Decision

# Introduction

In RAN1#86bis the following agreement was made regarding multi antenna transmission schemes for NR [1]:

Agreements: - DL and UL transmission techniques should be studied to provide robustness against imperfect CSI and mobility - e.g.) The techniques using a subset of beams/precoders may include beam cycling, beam broadening, etc. - e.g.) In case of DL and FDD, this technical scope may include semi-open-loop MIMO technique being discussed in LTE eFD-MIMO. Agreements: - Support downlink transmission scheme(s) achieving diversity gain at least for some control information transmission - Exact scheme is for further study. - Specification support for such transmission schemes, i.e., the scheme(s) may or may not be implemented in spec-transparent manner - Note: The data/control channel transmission is at least based on DM-RS for demodulation.

In this contribution transmission schemes without spatial information and with partial spatial information are discussed.  Performance comparison using link-level and system-level simulation results are provided.

# Discussion

## Open-loop scheme without spatial information

In some cases spatial channel state information may be not reliable or available at the gNB transmitter. In this case gNB should apply open-loop transmission scheme that should achieve full spatial coverage. The simplest transmission scheme with full spatial coverage is single antenna transmission. However, in case of relatively large number of antenna elements at the gNB, it may be challenging to gNB to allocate all transmit power in a single antenna element. Another transmission scheme with full spatial coverage relies on the precoder cycling, where the precoding is applied to all antennas of gNB and changes in frequency domain every K PRBs. Comparing to single antenna transmission, the precoding cycling scheme can achieve full power utilization without any implementation limitation, and therefore, it should be considered as baseline for NR.

One of the important aspect of the precoding cycling transmission scheme is selection of the precoder set for the cycling. In this paper, we compared two different precoder sets, one with DFT vectors and the other with non DFT vectors.

Non DFT precoding vectors were obtained from DFT vectors W by using beam broadening operation, where each DFT vector is multiplied by beam broadening function as follows:

,

where Wb is resulting precoding corresponding to broaden beam, N1 or N2 is number of antennas in horizontal and vertical domain and {p,c} are parameters that determine the desired beam width of the precoding. The examples of beam patterns for different values of c are illustrated in Figure 1.

Figure 1 Examples of beam broadening with 4 antennas for different c values

To compare the performance of open-loop scheme with different precoding vectors sets, link-level simulations were carried out. For simulations, eight transmit antennas at the gNB were assumed. The channel with medium and high correlations were considered. LTE 8Tx rank-1 codebook with and without beam broadening function were used for precoding cycling, where the precoder was randomly chosen in every PRB. Other details of simulation assumptions can be found in Appendix.

Figure 2 Link level simulation results for precoder cycling with different c values and different MCS

From the simulation results in Figure 2, it can be seen that precoding cycling based on the non-DFT vectors with the beam broadening function provides noticeable gains over DFT precoding cycling. The performance gains are increasing for the channel with high correlation.  Based on the result the following observation and proposal can be made:

Observation: Beam broadening function provides noticeable gains over DFT precoding for open-loop transmission scheme with precoding cycling.

Proposal: Consider beam broadening function for open-loop schemes in NR

## Closed-loop scheme with partial spatial information

In case of high speed scenarios, some of CSI information provided by the UE to gNB may become outdated due to feedback delay and channel aging. In this case, it may be beneficial to rely only on the subset of the reported CSI information. For example, in LTE, semi-open loop transmission scheme were introduced, where gNB only relies on W1 feedback for downlink transmission by applying precoding cycling only for the precoding vectors indicated by i1 report.

To identify the CSI parameters that can be used for transmission in high speed scenarios, link-level evaluations were carried out with eight transmit antennas at the gNB. Channel model with high and medium spatial correlation were considered. The performance of the transmission schemes with polarization cycling, polarization and beam cycling were compared to each other. The performance results for closed-loop scheme is also provided for reference. In case of polarization cycling, the beamformer was selected according to the best reported beam in CSI report, while polarization was chosen randomly on each PRB. In case of polarization and beam cycling, the beamformer cycling was performed across all beams and polarizations within W1 report.

Figure 3 Link-level simulation results for semi-open loop and closed loop schemes

From the simulation results presented in Figure 3, it can be seen that the transmission scheme with polarization cycling provides better performance comparing to other evaluated schemes.

Observation: For high-speed scenario transmission scheme with polarization cycling provides better performance than transmission scheme with polarization and beamforming cycling.

SFBC precoding can also provide polarization diversity gain. However, since the number of transmission streams per subcarrier is two, compared to polarization cycling, SFBC reduces interference rejection capabilities of the interference aware receivers. To evaluate the performance impact, a system-level simulations were carried out for dense urban scenario with all outdoor UEs and 120 km/h speed. For evaluations FTP traffic model 1 was used. The system level performance for transmission scheme with polarization cycling and SFBC precoding were compared assuming MMSE-IRC receiver. LTE codebook was used for the precoding vectors.

Table 1: System level simulation results for different multi antenna schemes with MMSE-IRC receiver

| Traffic load (λ) |  | low (2.5) |  | medium (4.7) |  |
| --- | --- | --- | --- | --- | --- |
| MIMO scheme |  | Polarization cycling | Polarization SFBC | Polarization cycling | Polarization SFBC |
| UE average packet throughput, Mbps | Average | 46.80 | 44.30 (-5%) | 37.50 | 32.63 (-13%) |
|  | 5% of CDF | 22.72 | 19.36 (-15%) | 14.25 | 10.04 (-29%) |
|  | 50% of CDF | 55.25 | 53.34 (-4%) | 36.37 | 30.11 (-17%) |
|  | 95% of CDF | 55.85 | 55.84 (0%) | 55.78 | 55.73 (-1%) |
| Resource utilization |  | 18.1% | 19.8% | 37.5% | 44.4% |

From the simulation results presented in Table 1, it can be seen that SFBC precoding provides worse performance comparing to polarization cycling across PRBs.

Observation: For interference limited scenarios, transmission scheme with polarization cycling across PRBs provides better performance comparing to SFBC transmission scheme.

# Summary

In this contribution transmission schemes without spatial information and with partial spatial information are discussed.

The following observations were made:

- Beam broadening function provides noticeable gains over DFT precoding for open loop transmission scheme with precoding cycling.

- For high-speed scenario transmission scheme with polarization cycling provides better performance than transmission scheme with polarization and beamforming cycling.

- For interference limited scenarios, transmission scheme with polarization cycling across PRBs provides better performance comparing to SFBC transmission scheme.

Based on the observation the following proposal was made:

- Consider beam broadening function for open-loop schemes in NR

# References

- Chairman’s notes, RAN1#86bis, 3GPP TSG RAN WG1 Meeting #86bis

# Appendix

| Parameter for SLS | Value |
| --- | --- |
| Scenario | Dense urban |
| Layout | Single layer: Macro layer: Hex. Grid |
| Channel model | 3D UMa, ISD = 200 m |
| Carrier frequency | 4 GHz |
| Aggregated system bandwidth | 200 MHz (UL + DL) |
| Simulation bandwidth | 20 MHz per CC |
| Tx power | BS: 44 dBm PA scaled down with simulation BW when system BW is higher than simulation BW. |
| UE distribution | Uniform 100% outdoor (120 kmph) |
| BS antenna configuration | 8x8x2 URA X-pol, slants -45/+45 degree, 1x8x2 TXRU layout 0.5-wavelength horizontal spacing 0.8-wavelength vertical spacing |
| UE antenna configuration | 2 Rx X-pol, slant 0/90 degrees |
| Traffic model | FTP 1 |
| TRP association | RSRP based Handover margin = 3dB |
| Channel estimation | Ideal |
| Interference covariance estimation | Ideal |
| CSI feedback | LTE codebook; (N1,N2) = (8,1); (O1,O2) = (4,1); Codebook Config. 1. i1 feedback only |
| Transmission mode | DM-RS based SU-MIMO Maximum rank of transmission is 1 |
| Scheduling | Proportional Fair |
| OLLA | 10% BLER target |
| Elevation beamforming | One vertical beam per TXRU electrically down-tilted to 100 degrees |
| UE receiver | MMSE-IRC |
| Max HARQ transmissions | 4 |

| Parameter for LLS | Value |
| --- | --- |
| Channel model | EVA; 300 Hz maximum Doppler frequency |
| Allocation size | 50 PRB |
| BS antenna configuration | 8 Tx with 0.9 (high) and 0.3 (medium) correlation between two adjacent antenna elements with one polarization. Medium correlation (0.3) between collocated antennas with different polarization. |
| UE antenna configuration | 2 Rx with no correlation between elements |
| Channel estimation | Ideal |
| Noise estimation | Ideal |
| CSI feedback periodicity | 5 subframes |
| CSI feedback delay | 8 subframes |
| UE receiver | MMSE |
| MCS | QPSK 1/2; 16-QAM 1/2. |
| Max HARQ transmissions | 0 |