3GPP TSG-RAN WG1#87	R1-1612668

Reno, USA, 14-18 November, 2016

Source:	Ericsson

Title:	On the Remaining Details of Non-precoded CSI-RS Codebook Design for 20 to 32 Ports

Agenda Item:	6.2.2.1.3

Document for:	Discussion and Decision

# Introduction

In RAN1#86bis, the following agreement and conclusions were reached on the non-precoded CSI-RS codebooks for 20 - 32 ports [1]:

Agreement:

- Agree on all (N1, N2) combinations on slide 4 of R1-1010560

- Agree on the following table for (O1, O2) combinations (Alt1 of the left table on slide 4 of R1-1610560)

| Number of CSI-RS antenna ports, P | (N1,N2) | (O1,O2) |
| --- | --- | --- |
| 20 | (2,5) | (8,4) |
|  | (5,2) | (4,4) |
|  | (10,1) | (4,-) |
| 24 | (2,6) | (8,4) |
|  | (3,4) | (8,4) |
|  | (4,3) | (4,4) |
|  | (6,2) | (4,4) |
|  | (12,1) | (4,-) |
| 28 | (2,7) | (8,4) |
|  | (7,2) | (4,4) |
|  | (14,1) | (4,-) |
| 32 | (2,8) | (8,4) |
|  | (4,4) | (8,4) |
|  | (8,2) | (4,4) |
|  | (16,1) | (4,-) |

Conclusions:

- The need for extending design principles of Rel.13 Class A codebooks with {20, 24, 28, 32} ports is to be decided in RAN1#87.

In this contribution, we discuss Class A codebook designs for 20,24,28 and 32 ports in Rel-14.

# Non-precoded CSI-RS codebook design for 20-32 ports

## Extending of Rel-13 Class A codebook

With the agreed port layouts for 20 to 32 ports, it is straight forward to extend the Rel-13 class A codebook to support 20 to 32 ports.  The same set of codebook parameters, i.e. {N1, N2, O1, O2, config#}, should be used for the non-precoded CSI-RS codebooks in Rel-14. The same codebook configs 1-4 also should be used in Rel-14. Only the parameter ranges of {N1, N2} need to be extended. Therefore, the same set of RRC parameters can be used with the following new parameter ranges:

- codebookConfigig-N1:  range (2,3,4,5,6,7,8,10,12,16)

- codebookConfig-N2: 	range (1,2,3,4,5,6,7,8)

- codebook-Over-Sampling-RateConfig-O1: range unchanged

- codebook-Over-Sampling-RateConfig-O2: range unchanged

- CodebookConfig:  range (1,2,3,4) unchanged

The allowed {N1, N2, O1, O2} combinations should follow the agreed port layouts and oversampling combinations. With this extension, there is only limited spec changes required in 36.213 and 36.331.

Observation 1:

- The same class A codebook configuration in Rel-13 can be used for non-precoded CSI-RS codebook in Rel-14. Only the range of N1 and N2 need to be extended.

Proposal 1:

- Reuse the Rel-13 class A codebook configuration parameters for Rel-14 non-precoded CSI-RS codebook

## Support of codebook enhancements

In the previous RAN1 meetings, some concerns were raised on the potential lack of angular coverage with 4 beams per beam group in W1 in Configs 2-4 for more than 16 ports and some codebook enhancements were proposed. However, there are no simulation results showing that there is indeed a coverage problem. Our simulation results [2] with the following three port layouts and codebook configurations show negligible differences between 4 and 8 beams in W1 in 3D UMi.

- 1x16 (vertical x horizontal) port layout:  Config4 with 4 and 8 beams in W1

- 2x8 (vertical x horizontal) port layout: 	Config2 with 4 with 8 beams in W1 (i.e. 2x2 and 2x4 beam arrangement)

- 8x2 (vertical x horizontal) port layout: 	Config2 with 4 and 8 beams in W1 (i.e. 2x2 beams and 4x2 beam arrangement)

Note that these antenna port layouts have the narrowest beamwidths among the Rel-14 port layouts and if there is an angular coverage issue, these would represent the worst case scenarios.   The results, however, show no big performance difference with 4 and 8 beams in W1.

Observation 2:

- Similar SU-MIMO performance is observed for W1 with 4 beams per beam group and W1 with 8 beams per beam group in Configs 2 & 4 for 32 ports

In [3], it was proposed to enhance Class A rank1-2 codebook with two new configs, i.e. “codebook1” and “codebook2” as shown Figure 1.

- Codebook 1

- Codebook 2

Figure 1: codebook enhancements proposed in [3]

According to [4],  for both codebook1 and codebook2, there seems to be some small performance gains comparing to Class A codebook with config2.  However, when compared to a Class A codebook with config3, both codebook1 and codebook 2 have either very small  cell edge gains (<2%)  or losses as shown in Table 1 and Table 2 below, where the data were extracted from Table 4 and Table 5 in [4], respectively.

Table 1: Performance comparison with various codebooks in 3D-UMi with (N1, N2, O1, O2) = (4,4,4,4) [4].

|  | Mean UE Throughput (bps/Hz) | Mean UE Throughput Gain | 5% UE Throughput (bps/Hz) | 5% UE Throughput Gain | Resource Utilization | FTP load, λ (UEs/s/sector) |
| --- | --- | --- | --- | --- | --- | --- |
| Config 3 | 2.9976 | 0.00% | 0.8316 | 0.00% | 0.27 | 2 |
| Codebook 2 | 2.9939 | -0.12% | 0.8475 | 1.91% | 0.27 |  |
| Codebook 1 | 2.996 | -0.05% | 0.8333 | 0.20% | 0.27 |  |

Table 2: Performance comparison with various codebooks in 3D-UMa with (N1, N2, O1,O2) = (4,4,4,4) [4].

|  | Mean UE Throughput (bps/Hz) | Mean UE Throughput Gain | 5% UE Throughput (bps/Hz) | 5% UE Throughput Gain | Resource Utilization | FTP load, λ (UEs/s/sector) |
| --- | --- | --- | --- | --- | --- | --- |
| Config 3 | 2.3394 | 0.00% | 0.5556 | 0.00% | 0.27 | 2 |
| Codebook 2 | 2.3181 | -0.91% | 0.5391 | -2.97% | 0.27 |  |
| Codebook 1 | 2.3428 | 0.15% | 0.5658 | 1.84% | 0.34 |  |

Observation 3:

- When compared to config3 results in [4], negligible performance gain or even loss were observed with codebook1 and codebook2 as proposed in [3]

Given observations 2 and 3 above, we have the following proposal:

Proposal 2:

- The codebook Configs 1-4 of Rel-13 are used also in Rel-14 for 20,24,28 and 32 ports, unless a new config can be justified by performance gains

# Conclusions

In this contribution, we have discussed extending of Rel-13 class A codebook to support 20 to 32 ports in Rel-14 non-precoded CSI-RS. Based on the discussions, we have the following observations and proposals:

Observation 1:

- The same class A codebook configuration in Rel-13 can be used for non-precoded CSI-RS codebook in Rel-14. Only the range of N1 and N2 need to be extended.

Observation 2:

- Similar SU-MIMO performance is observed for W1 with 4 beams per beam group and W1 with 8 beams per beam group in Configs 2 & 4 for 32 ports

Observation 3:

- When compared to config3 results in [4], negligible performance gain or even loss were observed with codebook1 and codebook2 as proposed in [3]

Proposal 1:

- Reuse Rel-13 class A codebook configuration parameters for Rel-14 non-precoded CSI-RS codebook

Proposal 2:

- The codebook Configs 1-4 of Rel-13 are used also in Rel-14 for 20,24,28 and 32 ports, unless a new config can be justified by performance gains

# References

- Chairman’s notes RAN1#85

- R1-165097, “Class A codebook design for up to 32 ports”, Ericsson, RAN1#85

- R1-1610942, “WF on Class A codebook enhancements”, Qualcomm, LG Electronics, Xinwei, RAN1#86bis

- R1-1609195, “Discussion on Class A codebook for eFD-MIMO”, LG Electronics, RAN1#86bis

- R1-1609967, “Non-precoded CSI-RS Codebooks for 20-32 ports”, Qualcomm, RAN1#86bis