TSG-RAN WG1 #87	R1-1612925

Reno, NV, USA, November 14 – 18, 2016

Source:	Ericsson

Title:	On high-level considerations on URLLC design

Agenda Item:	7.1.4.4

Document for:	Discussion and Decision

# Introduction

URLLC requires low latency and high reliability, where high reliability means that the packet should be successfully delivered within the given latency budget, i.e. a packet delivered after the latency limit is of no value and treated as a lost packet. The requirements in terms of latency and reliability is highly use-case dependent and not all use cases require 1 ms latency and/or 10-5 in reliability.

In this contribution we provide some high-level views on URLLC design.

# Discussion

Design for URLLC need to consider latency and reliability.

## General design principle

Low latency is extensively discussed for NR and has been a large part of the frame structure (including mini-slots) and numerology discussion as well as various other aspects such as designing signals for pipelining-friendly processing in the UE and ‘immediate’ hybrid-ARQ acknowledgement in the same slot. Hence, from a latency perspective there seems to be no need for a separate URLLC discussion but rather take it as an integrated part in the overall NR design.

Reliability requires the data channels as well as the control channels to be designed to allow high reliability. This calls for efficient coding schemes, exploitation of diversity in frequency and spatial domains and (if the latency budget allows) time domain using e.g. as hybrid-ARQ retransmissions. It may also have implications on e.g. the CRC sizes for control channels in order to avoid too many false alarms. These design choices need to be part of the respective discussion (control channel design, data transmission, multi-antenna schemes, etc) and not in a separate URLLC discussion. If this is not the case there is a risk we may design two different channel designs when it would be sufficient with a single one.

A separate URLLC agenda item (if there is such an agenda item for future meetings) could instead focus in evaluating the NR design from a URLLC perspective to see if there is a need for complementary solutions or enhancements to the general NR design. Before there is some form of NR design in place, e.g. for control channels, scheduling mechanism, etc, it is difficult to discuss URLLC for NR in detail.

## Scheduling for URLLC

As an example of the above, consider scheduling. NR will support dynamic scheduling using a scheduling request, followed by a scheduling grant. In addition, it will most likely also support some form of semi-persistent scheduling which could be useful in case the scheduling request/scheduling grant signaling is not reliable enough for some use cases. In LTE, semi-persistent scheduling implies that the UE need to transmit whenever a scheduling occasion occurs (i.e. padding will be applied to fill up the semi-persistently scheduled resources if needed) although in the instant uplink access work in RAN2 this has been enhanced such that a UE can remain silent if there is no data to transmit in a semi-persistently scheduled occasion. A similar approach could easily be applied to NR.

The basis for URLLC traffic should be dynamic and/or semi-persistent scheduling. Once the properties of these mechanism in NR are better known, e.g. in terms of latency and reliability, it can be evaluated if additional means to support URLLC are required. Only if there is a clear need which cannot be fulfilled by other, already existing features, should such additional mechanisms be introduced. For example, dynamically broadcasting available resources not used for eMBB to URLLC terminals (as suggested at RAN1#86bis) seem not to provide any benefits compared to semi-static resource allocation to the URLLC users and dynamically schedule eMBB on the remaining resources.

# Conclusion

URLLC should be an integral part of the overall NR design. Only if the general mechanism cannot support URLLC and there is a clear need should other, complementary mechanisms be introduced.