3GPP TSG RAN WG1 Meeting #87		R1-1612936

Reno, Nevada, USA, 14 - 18 Nov 2016

Source: 	Ericsson

Title:	Contents of PSBCH for V2X

Agenda Item:	6.2.1.7

Document for:	Discussion and Decision

# Introduction

In this paper we discuss the outstanding details concerning PSBCH for V2X. Background of our discussion is captured in the following agreements:

Agreements in RAN1 #85:

- Working assumption: PSBCH content size is the same as Rel-12.

Agreements in RAN1 #86bis:

- Rel-12 mechanism is re-used to determine the subframe used for transmissions of SLSS/PSBCH from out-coverage UEs synchronized to SLSS, i.e., two SLSS resources are pre-configured for an out-coverage UE, one is used for SLSS of UE’s syncRef and the other is used for SLSS transmission of the UE.

- Working assumption: An out-coverage UE directly synchronized to GNSS transmits SLSS/PSBCH on the pre-configured sync resource which is one of the two preconfigured sync resources.

  - FFS: Out-coverage UE indirectly synchronized to GNSS

  - In the other resource, the UE receives PSBCH potentially transmitted from an in-coverage UE.

- FFS whether a UE that selected SLSS ID = 0 as its sync reference transmits SLSS = X (FFS 0 or 168) and PSBCH in a different resource.

  - FFS PSBCH content if transmits

# Contents of PSBCH

We propose the following contents for the V2X PSBCH:

- DFN: 14 bits

  - direct frame number: 10 bits

  - direct subframe number: 4 bits

- TDD UL-DL configuration (tdd_ConfigSL-r12): 3 bits:

  - In case of FDD, this field is set to 000.

  - The UE is assumed to know a priori the duplex mode of the carrier

- In-coverage indicator: 1 bit

- Sidelink system bandwidth: 3 bits

- Reserved field: FFS bits set to a SIB-signalled or preconfigured value in V2X PSBCH.

The contents are identical to Rel-12 except for the length of the reserved field. Due to the reduction in REs available to PSBCH in V2X compared to Rel-12 we suggest to re-evaluate the length of the reserved field in order to meet a reasonable link budget requirement.

Proposal:

- V2X PSBCH content is the same as Rel-12 PSBCH.

- PSBCH does not signal any other parameter that is not mandatory for protecting the network.

- The length of the reserved field should be adjusted according to the link budget target.

# Conclusion

In this contribution we discuss outstanding details of PSBCH:

Proposal:

- V2X PSBCH content is the same as Rel-12 PSBCH.

- PSBCH does not signal any other parameter that is not mandatory for protecting the network.

- The length of the reserved field should be adjusted according to the link budget target.