3GPP TSG-RAN WG1 #87	R1-1613490

Reno, NV, USA 14th - 18th November 2016

Title:	[DRAFT] LS regarding RAN1 agreements on FeMBMS

Release:	14

Work Item:	MBMS_LTE_enh2-Core

Source:	Intel Corporation [TSG RAN WG1]

To:	TSG RAN WG2

Cc:	-

Contact Person:

#### Name:	Jeongho Jeon

E-mail:	jeongho.jeon@intel.com

1. Overall Description:

RAN1 has discussed eMBMS enhancements for LTE and made the following agreements:

Agreement (supersedes the Working Assumption made in RAN1#86bis):

- For 100% MBSFN subframe allocation FeMBMS carrier transmits a periodic subframe, CAS Cell Acquisition Subframe

- CAS supports PSS, SSS, CRS, PBCH, PDCCH, PDSCH (SI)

- CAS is always transmitted  in subframe #0  with a period of 40ms

- MIB is provided by PBCH in every CAS

- The MIB is transmitted in SFN mod 4 = 0 and can change only in SFN mod 16 = 0 and contains systemFrameNumber equal to the 6 most significant bits of the SFN

- SI can be provided by PDSCH in CAS

- A first SI that may also contain scheduling of further SI can be transmitted in SFN mod 8 = 0  and can change only in SFN mod 16 = 0

- This first SI may be a combination of SIBs, up to RAN2 agreements.

- MCCH change notification and SI modification notification are sent in PDCCH region of the CAS.

- From RAN1 perspective, it is feasible to transmit multiple SI messages in the same subframe by using different RNTIs. RAN2 may consider if this may be used.

- Inform RAN2 that SI modification notification can be conveyed to the UE with Direct Indication signalling.

Agreements:

- For a Rel-14 FeMBMS carrier, the current DCI format 1-C used to indicate MCCH change, is extended to also indicate SI change.

- PBCH in the CAS uses a different scrambling sequence initialization than for legacy PBCH.

- Cross-carrier scheduling is always supported (regardless of presence of unicast control region)

  - PHICH on the scheduling cell is used

- If a UE is configured with self-carrier scheduling, the UE shall assume that unicast control region (including CRS) is always present in all MBSFN subframes.

- For an FeMBMS carrier

  - A UE shall not assume that unicast control region (including CRS) is always present in an MBSFN subframe not assumed to be used for PMCH.

  - A UE can assume that unicast control region (including CRS) is always present in an MBSFN subframe assumed to be used for PMCH with 15kHz numerology.

  - A UE can assume that unicast control region (including CRS) is never present in an MBSFN subframe assumed to be used for PMCH with 1.25kHz numerology.

- Subframes using the new numerology do not need to be allocated in pairs

  - The RS pattern is defined for even and odd subframes independently

Agreements:

- In MIB, two reserved bits are used to signal additional subframes other than CAS which follow CAS immediately for SI transmission. The values are 0, 1, 2, 3.

  - From RAN1 perspective, 1.4MHz does not support a value of 0.

  - Note: The exact signaling for such indication is up to RAN2.

  - Note: It is up to RAN 2 if the first SI for dedicated FeMBMS carrier needs to indicate the presence of a further set of additional non-MBSFN subframes.

2. Actions:

ACTION: 	RAN1 would like to respectfully ask RAN2 to take the above information into account for their specification work.

3. Date of Next TSG-RAN1 Meetings:

RAN1 #88	February 13 – 17, 2017	Athens, Greece

RAN1 #88bis	April 3 – 7, 2017	US