3GPP TSG RAN WG1 Meeting #88                                                                           R1-1701809

Athens, Greece, 13th - 17th February 2017

# Source: 	ZTE, ZTE Microelectronics

Title:	Linear combination based CSI feedback design for NR MIMO

Agenda Item:	8.1.2.3.3

Document for:	Discussion and Decision

- Introduction

In last RAN1 NR Ad Hoc meeting, it has been agreed that at least one of the following linear combination based feedback schemes need to be supported in NR [1].

# Agreements:

- Support at least one scheme taken from Category 1, 2, and/or 3 for Type II CSI

  - Possible down selection can be performed throughout Phase I WI

    - If more than one schemes is supported, these schemes should be complementary

    - This includes further refinement within each category

    - Note: other schemes within each category are not precluded

  - Descriptions for Category 1 and 2 are given in the following slides

    - For the purpose of summary in TR38.802

- Category 1: precoder feedback based on linear combination codebook

- Dual-stage W = W1W2 codebook

  - W1 consists of a set of L orthogonal beams, e.g. 2D DFT beams

    - The set of L beams is selected out of a basis, e.g. oversampled 2D DFT beams

    - Beam selection is wideband

  - W2: L beams are combined in W2 with common W1

    - Subband reporting of phase quantization of beam combining coefficients

  - Beam amplitude scaling quantization can be configured for wideband or subband reporting

- Category 2: covariance matrix feedback

      - A quantized/compressed version of covariance matrix is reported by the UE

    - Quantization/compression is based on a set of M orthogonal basis vectors

    - Reporting can include indicators of the M basis vectors along with a set of coefficients

    - FFS: basis set

- Category 3: Hybrid CSI feedback

  - Type II Category 1 or 2 CSI codebook can be used in conjunction with LTE-Class-B-type-like CSI feedback (e.g. based on port selection/combination codebook)

    - The LTE-Class-B-type-like CSI feedback can be based on either Type I or Type II CSI codebook

Moreover, several detailed alternatives are given for each category. In this contribution, we discuss some detailed design aspects on linear combination codebook in terms of beam selection and coefficient design for the first category.

- Discussion on linear combination based CSI feedback

Linear combination based CSI feedback has drawn much recent attention due to its potential to enhance the beamforming gain. The linear combination of K beams for l-th layer can be represented as

fl=a1,1,lv1,1,l+…+aN,1,lvN,1,la1,2,lv1,1,l+…+aN,2,lvN,2,l

where vn,r,l=un,r,lh⊗un,r,lv denotes the n-th 2D-beam selected for l-th layer and r-th polarization direction, and an,r,l=λn,r,lejαn,r,l denotes the coefficients associated with vn,r,l, including both amplitude and phase. From the analysis on channel property, the beams selected for different layers and pols in the same panel can be identical to reduce feedback overhead. Then the precoder for L layers is

F=f1⋯fL=v1⋯vN00v1⋯vNa1,1⋯a1,La2,1⋯a2,L=W1W2

where W1 contains the selected beams, and in W2, ar,l=a1,r,l⋯aN,r,lT denotes the vector containing coefficients for each lay and pol. In this section, we discuss the design of linear codebook based on the above structure.

2.1 General design principles for LC codebook in NR

In LTE Rel-14, linear combination codebook is agreed with constrained orthogonal beam combination supporting up to 2 layers. However, due to the limit of CSI overhead in LTE PUCCH, only two beams are combined. Since LC codebook shows good gain over single-beam codebook especially in the scenarios with rich scatters, more beams combined would lead to better performance. Hence 2-beam combination does not make full use of the potential of linear combination. Moreover, as the advanced CSI in Rel-14 is targeted on MU CSI, linear combination codebook only support up to 2 layers. Hence the Rel-14 LC codebook cannot support high-rank transmission as well. This also limits the performance of linear combination codebook, especially in the case of SU transmission. Since 4-antenna commercial UEs have been more and more common, and NR evaluation assumptions target to support UE up to 32 antennas, high-rank SU transmission is also an important use case in NR. Hence we have the following proposal giving general design principles for LC codebook in NR considering the above aspects.

Proposal 1: NR should support linear combination codebook based on constrained orthogonal beam combination in LTE Rel-14 with at least the following enhancements:

- Enable the combination of more beams.

- Support at least four-layer SU CSI.

2.2 Beam selection

For beam selection, a direct simple way is to select those vectors indicated by Rel-13 W1. It is not necessary to introduce new signal to feed back the beam selection information through this method. However, for linear space, orthogonal basis can provide the highest quantization efficiency to recover arbitrary vector in the space. Further, when the numbers of antennas and oversampling factors are large, a tremendous number of orthogonal beam groups can be identified in the whole 2D DFT vector space. It is impossible to use all orthogonal 2D DFT beams to get the target precoding because of extremely large feedback overhead. Therefore down-selection from these beams should be done in order to reduce overhead. Theoretically, we should choose the best orthogonal beams. However, this approach may also cause feedback overhead problem, as for N1*N2 antenna array, totally CN1N2K possible combinations need to be quantized. One approach balancing both feedback overhead and performance is to pre-define several beam selection patterns for different scenarios. The beam selection patterns can be configured through high-level parameters, and each pattern indicates M orthogonal beam groups to be selected. UE only needs to dynamically chose and feed back the best beam group out of M. Following this approach, the extra overhead compared with the beam selection based on legacy W1 is only log2M bits. UE’s procedure of selecting orthogonal beams is given as follows.

- Step 1: UE calculates the best beam and the corresponding PMI.

- Step 2: Based on the selected beam and configured M orthogonal beam group, UE selects the best beam group out of M.

- Step 3: UE calculates the coefficients based on the selected beam group.

An example of orthogonal beam pattern design, targeting at 4-beam combination, is shown in Fig. 3 and Table 1 for 3D-Umi. In this design, the beam groups contain the adjacent orthogonal beams around the selected beam in Step 1.

Fig. 1 Orthogonal beam pattern in (N1, N2) = (4, 4) for 3D-Umi

Table 1 Orthogonal beam index in each group for 3D-Umi

| (N1, N2) | Beam index in each group |
| --- | --- |
| (4, 4) | {0, 4, 12, 1}, {0, 4, 12, 3}, {0, 4, 1, 3}, {0, 12, 1, 3} |

For the beam selection approach using resource-level and/or port-level beam information feedback, this beam group pattern configuration can be performed by beam grouping based spatial restriction. Specifically, gNB indicates the UE how to group the ports/resources, and UE selects beams based on the best beam and beam grouping. Then UE also needs to calculate the associated phase and amplitude of the coefficients. The details on beam grouping and spatial restriction are illustrated in our companion contribution [3].

Based on the orthogonal beam pattern design, we show the simulation results for 32 ports and up to 2 layers in Fig. 4. Amplitude coefficient and phase coefficient are quantized with 2-bit and 3-bit respectively and fed back in sub-band. Other system simulation parameters are attached in Appendix I. It is seen from the simulation results that, with only 2 extra bits for beam selection, the proposed scheme provides large gain over the scheme based on legacy W1.

Fig.2 Simulations results for different LC feedback schemes

Based on the analysis and simulation results, we have the following observation and proposal.

Observation 1: Orthogonal basis with restricted pattern outperforms non-orthogonal basis for quantized LC codebook in NR.

Proposal 2: NR should consider both performance and feedback overhead for linear combination feedback.

    - Grouping-based restricted orthogonal beam selection is a good candidate.

2.3 Coefficient design

In LTE Rel.14, as mentioned above, LC codebook is designed to enhance MU CSI. Hence Rel.14 LC codebook supports up to two layers. Moreover, since MU algorithm can help to diagonalize the channel, orthogonality among layers is not achieved in Rel.14 LC codebook. However, if LC codebook is supported for high-rank SU transmission in NR, orthogonality among layers need to be studied for LC codebook as it impacts the resulting performance.

For LC codebook, the typical implementation algorithm is to search the best beam group and then to calculate and quantize the coefficients with signal processing algorithm such as LS. Compared with exhaust search of the codebook, this approach can find the near-optimal precoder with lower complexity. However, as the coefficients are usually computed to minimize the square error, it’s difficult to guarantee the orthogonality among layers, which helps to optimize the inter-layer interference. Hence in order to achieve orthogonality among layers, some specified constraints need to be imposed on the coefficients. Moreover, coefficients with constraints would cost smaller overhead than unrestricted design. From the perspective of performance, the constraints would hurt the optimality in terms of least square error, but may also have some positive impact in terms of inter-layer interference minimization. Hence the constraint has to be designed carefully to trade-off square error and inter-layer interference. It cannot be too strong to degrade the performance due to the break on least square error. In the remainder of this subsection, we give a specific coefficient design to achieve inter-layer orthogonality.

For arbitrary two layers, l1 and l2, from L layers, the linear combination precoders can be expressed as follows.

fl1= a1,1,l1v1+…+aN,1,l1vNa1,2,l1v1+…+aN,2,l1vN

fl2=a1,1,l2v1+…+aN,1,l2vNa1,2,l2v1+…+aN,2,l2vN

In order to achieve inter-layer orthogonality, we have fl2Hfl1=0. Since the selected beams v1, …, vN are mutually orthogonal, it yields

m,nam,n,l1am,n,l2*=0

Then it can be concluded that the sufficient and necessary condition to achieve inter-layer orthogonality is to achieve orthogonality among columns for the following 2N*L matrix

A0=a1,1,1…a1,1,L⋮⋱⋮aN,2,1⋯aN,2,L

Based on the theory of unitary space parameterization, any matrix with orthogonal columns can be parameterized with Givens rotations. Hence to achieve inter-layer orthogonality, A can be quantized with the following expression

A=m=1LPmn=m+12NUn,m⋅I(:,1 :L)                                                       (1)

where I(:,1:L) denotes the first L columns of the unit matrix, and Pm is a 2N*2N diagonal matrix as follows.

Pm=diag1,…,1,exp⁡(jϕm+1,m,…,exp⁡(jϕ2N,m))

Un,m is a matrix transferred from the unit matrix by converting its (n,n)-th, (m,m)-th, (n,m)-th and (m,n)-th entries into the following submatrix

cos⁡(θn,m)sin(θn,m)sin(θn,m)-cos⁡(θn,m)

In the above expressions, 2N*L matrix A is expressed by L(4N-L-1) parameters θn,m and ϕn,m to achieve inter-layer orthogonality. In fact, for unrestricted coefficient feedback, totally 4NL parameters are required for both amplitudes and phases of entries in A, which means CSI overhead for L(L+1) parameters can be saved. Further, L(4N-L-1)/2 parameters θn,m∈[0,π/2] can be regarded as angle rotations of orthogonal basis I(:,1:L), which are related to the amplitudes of the entries in A. L(4N-L-1)/2 parameters ϕn,m∈[0,2π] can be regarded as phase shifts of I(:,1:L), which are related to phases of the entries in A. Hence θn,m can be wideband report, whereas ϕn,m can be subband report. Additionally, as the structure in equation (1) can be regarded as sufficient and necessary condition of inter-layer orthogonality, it is not a too strong restriction. Thus good performance trade-off between square error and inter-layer interference can be foreseen. Therefore, compared with unrestricted feedback on the amplitudes and phases of the entries in A, feedback constructed by equation (1) has the following advantages

  - Inter-layer orthogonality is achieved.

  - Good performance trade-off between square error and inter-layer interference.

  - CSI feedback overhead is reduced.

  - Flexibility of configuring WB and SB feedback for different parameters is retained.

From the perspective of UE complexity, exhaustive search for θ and ϕ may bring too much complexity. In fact, the following procedure can be used to calculated θ and ϕ with quite low complexity.

Step 1: Calculate P1(ϕ1,1,ϕ2,1,…ϕ2N,1). Firstly UE calculates the original coefficient matrix A0 via selected beams and channel eigenvectors V. Creat a diagonal matrix P1’ where diagonal elements are corresponding to the phase of elements in the first column of A0. Then transform the first column of A0  into real by equation (2). As only the relative phase shifts information of beams will be useful, P1  is obtained by equation (3)：

(2)

(3)

Step 2: Iteratively Calculate θn,1,n=2,3…2N：Transform the second element in the first column of A1 into zero by U2,1 as mentioned before, where θ2,1 is expressed through equation(4).

(4)

(5)

Then recursively calculate Un,1 from  and transform its nth element in the first column to 0 until the first column of coefficient matrix becomes a unitary vector. Thus 2(2N-1) parameters θn,1 and ϕn,1 for the first layer are obtained.

Step 3: Iteratively perform the procedure described by step 1and step2 to other layers. Finally all the 2L(4N-L-1)parameters θn,m and ϕn,mare acquired. We conduct system-level simulations to evaluate the performance of the proposed structure in (1). We compare the proposed scheme with legacy Class A and unrestricted coefficient feedback. For beam selection in LC codebook, 4 beams are selected according to Rel.14 beam selection approach. In the proposed scheme, 3 bits are used to quantize  ϕn,m, and 2 bits are used to quantize θn,m. In unrestricted coefficient feedback, 3 bits are used to quantize phase, and 2 bits are used to quantize amplitude. Up to 4 layer SU transmission is used for all cases. Moreover, wideband θ and wideband amplitudes are also evaluated for both schemes. The other simulation assumptions are shown in Appendix. Simulation results are shown in Table 2.

Table 2 Simulation results for different codebook schemes

| Schemes |  | Mean Gain | Edge Gain | RU |
| --- | --- | --- | --- | --- |
| 16 ports | Proposed, wideband θ | 8.5% | 26.4% | 0.27 |
|  | Unrestricted coefficients, wideband amplitude | 8.3% | 28.4% | 0.27 |
|  | Proposed, subband θ | 11.5% | 28.9% | 0.27 |
|  | Unrestricted coefficients, subband amplitude | 11.8% | 29.5% | 0.26 |
|  | Legacy Class A Config 1 | 0.0% | 0.0% | 0.30 |
| 32 ports | Proposed, wideband θ | 4.2% | 21.8% | 0.18 |
|  | Unrestricted coefficients, wideband amplitude | 1.7% | 17.6% | 0.19 |
|  | Proposed, subband θ | 6.2% | 21.5% | 0.18 |
|  | Unrestricted coefficients, subband amplitude | 4.2% | 19.6% | 0.19 |
|  | Legacy Class A Config 1 | 0.0% | 0.0% | 0.21 |

From Table 2, it can be observed that the proposed scheme achieve similar performance in the case of 16 ports and better performance in the case of 32 ports compared with unrestricted coefficient design. When amplitudes and θ are wideband, the proposed scheme performs better than unrestricted coefficients. Hence the constructed inter-layer orthogonality is helpful in terms of performance. Moreover, the proposed scheme saves 31.25% CSI overhead compared with unrestricted coefficient feedback. The observations can be summarized as follows.

Observation 2: Compared with unrestricted coefficient feedback, the proposed inter-layer orthogonality construction

  - is helpful in term of performance

  - saves CSI overhead

Based on the above observation, we have the following proposal

Proposal 3: NR should support inter-layer construction on the coefficient design in linear combination codebook.

- Conclusions

In this contribution, we discuss the aspects involved in LC based CSI feedback.  Based on the above discussion and simulations, we have the following observations and proposals.

Proposal 1: NR should support linear combination codebook based on constrained orthogonal beam combination with at least the following enhancements:

- Enable the combination of more beams.

- Support at least four-layer SU CSI.

Observation 1: Orthogonal basis with restricted pattern outperforms non-orthogonal basis for quantized LC codebook in NR.

Proposal 2: NR should consider both performance and feedback overhead for linear combination feedback.

    - Grouping-based restricted orthogonal beam selection is a good candidate.

Observation 2: Compared with unrestricted coefficient feedback, the proposed inter-layer orthogonality construction

  - is helpful in term of performance

  - saves CSI overhead

Proposal 3: NR should support inter-layer construction on the coefficient design in linear combination codebook.

- References

[1] Chairman’s notes, 3GPP RAN1#87

[2] Draft meeting report, 3GPP RAN#74

[3] R1-1700147, Beam selection and CSI acquisition for NR MIMO, ZTE, ZTE Microelectronics

Appendix

| System level simulation parameters |  |
| --- | --- |
| Scenarios | 3D-Uma 200m ISD |
| Antenna Configurations | 2x1 virtualization, with 130°tilt |
| Antenna Spacing | (dV,dH)=( 0.8λ, 0.5λ) |
| Number of UE antenna | 4Rx cross-polarized antenna |
| Traffic model | FTP 3 with packet size 0.5M byte |
| OLLA | Target at 10% BLER |
| CSI-RS | Period is 5 ms and overhead is accounted. |
| HARQ | Max 4 retransmissions or |
| Transmission rank | Adaptation between rank-1~4 or 1~2 |
| SU/MU pre-coding | BD |
| Scheduling | SU, up to 4 layers or Proportional fair, up to 2 UEs, up to 2 layers |
| CQI/PMI reporting interval and frequency granularity | 5ms for CSI, 6RB |
| Feedback scheme | Rel-12 enhanced CSI feedback, PUSCH mode 3-2, Ideal channel covariance /PMI feedback |
| Delay for scheduling and AMC | 6ms |
| Receiver | MMSE-IRC. With non-ideal interference covariance matrix estimation by using complex Wishart distribution with 12 degrees of freedom (Model in TR36.829 with DMRS based sample covariance matrix) |
| HARQ Scheme | Chase Combining |
| Maximum number of retransmissions | 4 |
| Traffic model | FTP3 model with 0.5Mbyte |
| Feedback Assumption | Non-ideal modeling of channel estimation error modeling is used, based on DMRS for data demodulation, based on IMR for interference measurement |
| Handover margin | 3dB |