# 3GPP TSG-RAN1#88	R1-1701889

# 13th – 17th April 2017

# Athens, Greece

Agenda Item:	6.2.4.3

Source: 	Ericsson

Title:  	On Release 14 DCI formats for the support of a single HARQ process

Document for:	Decision

# Introduction

One of the objectives of the Release 14 NB-IoT enhancements work item is to reduce power consumption and latency. At RAN1#87 it was agreed to introduce a larger TBS size to facilitate this reduction:

- There is a new NB-IoT UE category with max UL and max DL TBS of 2536 bits

- Introduce the {1 HARQ + 2536 bits UL/DL TBS} TBS tables in R1-1613508

This contribution takes the above agreement into account and proposes to reuse the Release 13 DCI formats N0 and N1 for the scheduling of a Release 14 UE supporting a single HARQ process with a maximum TBS of 2536 bits.

# Discussion

In Release 13 the DCI formats N0 and N1 are used to schedule NPDSCH and NPUSCH block flows. Each of these formats supports a four-bit Modulation and coding scheme (IMCS) parameter. It is used to identify the scheduled uplink or downlink TBS (ITBS). Release 13 supports a set of 13 different TBS values, while it in Release 14 has been agreed to expand the TBS table to support 14 different values. Table 1 and Table 2 present the agreed Release 14 TBS tables.

Table 1 Agreed NPDSCH TBS for a single HARQ process [2].

| ITBS | ISF |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1032 |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 | 968 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1352 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1032 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 904 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1128 | 1256 | 1544 | 2024 | 2536 |

Table 2 Agreed NPUSCH TBS for a single HARQ process [2].

| ITBS | IRU |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1128 | 1256 | 1544 | 2024 | 2536 |

A consequence of this minor update of the TBS tables is that the existing DCI Formats N0 and N1 will also be capable of supporting scheduling of NPDSCH and NPUSCH single HARQ blocks flows to UEs supporting up to 2536 TBS. This is achieved by using one of the spare code points in the Modulation and coding scheme (IMCS) parameter. It is therefore proposed that the DCI Formats N0 and N1 are reused for the scheduling of Release 14 NB-IoT UEs using a single HARQ process with a max TBS of 2536 bits.

Proposal 1: RAN1 agrees to reuse the DCI Formats N0 and N1 for the scheduling of Release 14 NB-IoT UEs using a single HARQ process with a max TBS of 2536 bits.

# Summary

In this contribution we propose to reuse the DCI Formats N0 and N1 for the scheduling of Release 14 NB-IoT UEs using a single HARQ process with a max TBS of 2536 bits.

Proposal 1: RAN1 agrees to reuse the DCI Formats N0 and N1 for the scheduling of Release 14 NB-IoT UEs using a single HARQ process with a max TBS of 2536 bits.

# References

- RP-161901, “Revised work item proposal: Enhancements of NB-IoT,” RAN#73, source Huawei, HiSilicon, 19 – 22 September, 2016.

- R1-1613508, “WF to achieve NB-IoT power consumption and latency reduction” source Ericsson et al, RAN1#87, 14th-18th Nov, 2016.