3GPP TSG RAN WG1 Meeting #88                                                                        R1-1702077

Athens, Greece 13th - 17th February 2017

Source:		CATT

Title:			Discussion on beam reporting

Agenda Item:		8.1.2.2.1

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1#86bis meeting, UE reporting for beam management procedures were discussed and following agreements were made [1]:

Agreements:

- For downlink, based on RS (used for beam management) transmitted by TRP, UE reports information associated with N selected Tx beams

  - Study how the N Tx beams can be selected

  - Study the case where N comprises of all Tx beams

  - Study UE reporting information

  - Note: N can be equal to 1

In 3GPP RAN1 NR Adhoc meeting, further agreements on beam reporting were reached [2]:

Agreements:

- UE measurement based on RS for beam management (at least CSI-RS)  composed of K (= total number of configured beams) beams and reporting measurement results of N selected beams:

  - N is not necessarily fixed number

  - FFS: whether/how to configure and/or indicate the values of N

  - Note: The above procedure based on RS for mobility purpose is not precluded.

- Reporting information at least include

  - Measurement quantities for N beam (s)

    - FFS: Detailed reporting contents, e.g., CSI, RSRP or both

    - FFS: How to select N beam(s)

    - FFS: how to identify the subset

  - Information indicating N DL Tx beam(s), if N < K

    - FFS: the details on this information, e.g., CSI-RS resource IDs, antenna port index, a combination of antenna port index and a time index, sequence index, etc.

It was also agreed to study group based beam management procedures in RAN1 #86bis meeting [1]:

Agreements:

- Group based beam management is to be further studied:

  - Definition of beam grouping:

    - Beam grouping = for TRP(s) or UE to group multiple Tx and/or Rx beam(s) and/or beam pair(s) into one subset of beams

  - FFS detailed mechanisms for beam grouping, reporting, beam-group based indication for beam measurement, beam-based transmission or beam switching, etc.

In this contribution, further details on UE beam reporting will be discussed.

Discussion

TX beam and RX beam are sensitive to UE movement, rotation and blockage, etc. To improve robustness of beamforming against these factors, multi-beam transmission was proposed, e.g., multi-beam diversity, or multi-beam multiplexing. With these schemes, a single data layer is transmitted from multiple spatial beams to improve robustness against blockage, rotation, etc.

To facilitate multi-beam transmission, in beam management procedure, UE needs to select and report more than one TX beams. CSI measured on these beams perhaps is also needed. Based on recommended TX beams, TX beam used for data transmission can be selected or even dynamically switched, e.g., using one selected beam for transmission, or using multiple beams for diversity transmission to overcome blockage.

The number of selected beams N can be configured by TRP. UE selects those TX beams that have the highest RSRP/ RSRQ. Alternatively, the number can be determined by UE. TRP can configure UE a threshold of RSRP/RSRQ, UE then selects those TX beams with RSRP/RSRP greater than the threshold.

Proposal 1:

- The number of selected beams N can be configured by TRP or determined by UE.

Measurement quantities for the N selected beams can be CSI or RSRP. With CSI measurement, the preferred beams and CSI over the beam are acquired at gNB in a single step. Data transmission can follow the beam reporting immediately utilizing complete CSI. However, computational complexity of deriving CSI would be a great burden for UE especially when a huge number of beams are involved. On the other hand, calculating RSRP is relative easy from complexity perspective. Another step is needed for UE to report CSI on the preferred beams if only RSRP is reported. As a result, measurement and reporting RSRP is suitable for initial beam reporting where large number of beams are to be measured. CSI can be employed in the procedure of fine beam tuning, e.g., during procedure P-2. Therefore, the measurement and reporting quantities is preferred to be configurable depending on the procedure of beam management. In some cases, reporting both CSI and RSRP may be beneficial to facilitate the comparison between beams reported during different procedures. It is straightforward to extend the agreed framework for CSI acquisition to beam management reporting: whether CSI or RSRP (or both) is to be reported is configured in the CSI reporting setting.

Proposal 2:

- Measurement quantities for beam reporting is configurable between RSRP, CSI, and both RSRP and CSI.

The N selected Tx beams can be indicated by CSI-RS resource IDs. CSI-RS for one candidate Tx beam is transmitted over one CSI-RS resource. The number of antenna ports for the CSI-RS resource can be configured depending on whether CSI or RSRP is reported. If only RSRP is reported, the port number could be 1 or 2. If CSI is reported, larger port number can be configured. CSI, including PMI, RI, CQI, etc, is measured and reported based on a proper codebook. Antenna port index indication is similar to CSI-RS resource ID, if antenna port number of CSI-RS resource is 1. Time index is also not necessary as long as the reference resource for reporting is clearly defined in the spec.

Proposal 3:

- CSI-RS resource IDs are reported to indicate selected beams.

If there is no constraint on the TX beams that UE selects, it is likely that those selected TX beams are highly correlated. That is, the directions of those selected beams are close to each other. This is especially the case when the candidate TX beams are of high spatial resolution. When one of the selected TX beam is blocked, it is highly possible all those selected beams are blocked. Thus the transmission reliability or robustness cannot be guaranteed as expected. The situation is illustrated in Figure 1. Two TX beams are selected by UE, and unfortunately the two TX beams travel along the same path. If one beam pair fails due to sudden blockage, it is highly probable that the other beam pair also fails.

Figure 1: Illustration of correlation between selected TX/RX beams

To overcome this problem, the TX beams can be grouped according to spatial correlation. For example, TX beams with correlation greater than a threshold are put into one group. Then UE is required to select TX beams from different groups, that is, at most one TX beam can be recommended from one TX beam group. An example is shown in Figure 2. Two TX beam groups are shown in Figure 2 (Other TX beams are omitted for simplicity). UE selects one TX beam from one group. With proper grouping, TX beams with sufficient spatial separation can be selected to guarantee robustness or reliability.

Figure 2: TX beam selection with TX beam group constraint

Proposal 4:

- Beam grouping based TX beam selection is supported to guarantee independence of selected TX beams.

  - UE is required to select TX beams from different beam groups.

Another usage of beam grouping is to unify intra-TRP and inter-TRP beam management. Inter-TRP beam management gives us more opportunity of overcoming blockage. For example, when beams from one TRP are blocked, the transmission could be switched to another TRP. Even more aggressively, inter-TRP joint transmission could be supported to further boost user throughput. Both above usages require UE to select TX beams for multiple TRPs. By beam grouping, selection of TX beams for multiple TRPs could be supported in the same framework as single TRP beam selection. To be specific, TX beams of different TRPs are put into different beam groups and UE is required to report beam-related information for those beam groups. Whether the beam groups belong to one or multiple TRPs could be made transparent to UE. The scenario is illustrated in Figure 3. Two beam groups from two TRPs are configured to UE. UE selects one TX beam from each group which is equivalent to select one TX beam from each TRP.

Figure 3: Beam grouping for multiple-TRP scenario

Beam grouping is carried out at TRP. TX beams with correlation greater than a threshold are put into one group, or TX beams pointing to adjacent spatial directions are put into one group. For a planar antenna array, oversampled 2D DFT beams are natural choice of beamforming weight. Assume the number of antenna elements in the first and second dimension of one polarization is  and , respectively. Oversampling factors of each dimension is  and . The oversampled 2D DFT beams have  beams:

(1)

,                          (2)

For example, one simple way of implement beam grouping is to put adjacent DFT beams into one group. A beam grouping results for  is illustrated in Figure 4.

Figure 4: Beam grouping illustration

The grouping information, i.e., information regarding which beams belong to the same group, shall be signaled to UE together with the RS configuration for beam management.

Proposal 5:

- Beam grouping is carried out at TRP, and the grouping information is signaled to UE together with the RS setting for beam management.

Conclusions

In this contribution, further details on UE beam reporting are discussed, and the following proposals are given:

Proposal 1:

- The number of selected beams N can be configured by TRP or determined by UE.

Proposal 2:

- Measurement quantities for beam reporting is configurable between RSRP, CSI, and both RSRP and CSI.

Proposal 3:

- CSI-RS resource IDs are reported to indicate selected beams.

Proposal 4:

- Beam grouping based TX beam selection is supported to guarantee independence of selected TX beams.

  - UE is required to select TX beams from different beam groups.

Proposal 5:

- Beam grouping is carried out at TRP, and the grouping information is signaled to UE together with the RS setting for beam management.

References

- RAN1 #86bis Chairman’s Notes.

- RAN1 NR Adhoc Chairman’s Notes, Jan. 2017.