3GPP TSG RAN WG1 Meeting RAN1 #88														R1-1702183

Athens, Greece 13th - 17th February 2017

Source: 	Intel Corporation

Title:	Paging for NR

Agenda item:	8.1.1.1.3

Document for:	Discussion and Decision

# Introduction

Following agreements were made regarding paging in RAN1 NR Ad-hoc #1.

Agreements: - Companies are encouraged to compare the following options for channel design for paging, taking the necessity of beam sweeping into consideration - Opt-1: Paging message is scheduled by DCI carried by NR-PDCCH and is transmitted over PCH carried by NR-PDSCH - Opt-2: Paging message is transmitted in a non-scheduled physical channel - Where the paging indication may be carried by NR-PBCH or some other channel(s) - Opt-3: Paging message is transmitted over PCH carried by NR-PDSCH without DCI. The resource is semi-statically configured - Opt-4: Paging message (e.g., only for SI change indication) is transmitted over NR-PDCCH without NR-PDSCH - Opt-5: Paging message is transmitted by PDSCH and paging indication is transmitted non-scheduled physical channel

Companies were encouraged to provide their views on definitions of paging occasion and paging periodicity, and potential contents of the paging message and its payload size.

# Discussion

As discussed in our companion contribution [1], the minimum paging message size is expected to around 44 bits in a nominal situations. Furthermore, the specification may need to support from up to 8192 UEs in a regular DRX cycle of 1.28s. This results in average of 64 UEs per 10 ms duration. For a carrier that is operating in 15 kHz subcarrier spacing, paging channel that could potentially support up to 64 UEs per radio frame (assuming 10 slots per radio frame) should provide sufficient signaling for paging. Taking the above information into account, we discuss the each of the paging channel options from NR Ad-hoc #1 meeting.

Option 1: Paging message is scheduled by DCI carried by NR-PDCCH and is transmitted over PCH carried by NR-PDSCH

This is similar to the paging channel for LTE. The benefits of this proposal is that a single paging channel can aggregate any number of paging messages with any paging size. It provides the most flexibility in terms of scheduling and allocating resources for paging. However, this is at the drawback of consuming common search spacing control signaling resources which is shared with other control information of other system information and broadcast information. Furthermore, this requires the UE to constantly monitor the common search space of the PDCCH, which can be a significant portion of UE power consumption during IDLE mode operations.

Option 2: Paging message is transmitted in a non-scheduled physical channel, where the paging indication may be carried by NR-PBCH or some other channel(s)

This option is to define a dedicated paging channel that may be configured by the system information. Although this proposal may not change the resource for paging as dynamically as option 1, the configured resources for paging can still be reclaimed for other purposes (e.g. data transmission) by the network if it is not used for paging. Therefore, the resources allocated for the paging channel are not necessary wasted.

In order to avoid blind detection of the payload size of the paging message, we assume that each paging message intended for each UE is encoded separately with a predetermined paging message size. If each paging message intended for each UE is encoded separately and mapped to different resources, and paging message is determined to be around 44 bits, the amount of blind decodes that needs to be performed for detection of the paging may be similar to blind decoding of PDCCH in option 1. However, this approach does not take up control signaling within the common search space.

Option 3: Paging message is transmitted over PCH carried by NR-PDSCH without DCI. The resource is semi-statically configured

Option 3 is conceptually similar to option 2. The only difference would be that dedicated paging channel would be defined in the form of NR-PDSCH. For example, paging message for one UE may simply occupy one PRB of a slot. The paging message would be encoded using NR-PDSCH mechanisms. The main drawback of this is that multiple slots is needed in mmWave where beam sweeping might be necessary. This is because the network may not be aware of the best Tx beams for IDLE mode UEs that is under mobility.

Option 4: Paging message (e.g., only for SI change indication) is transmitted over NR-PDCCH without NR-PDSCH

Option 4 is only available for SI change indication, and can be considered separately from rest of the options. In general, use of NR-PDCCH should be limited to only system critical signaling and careful analysis should be performed. For each feature that requires usage of NR-PDCCH will impact introduction of another feature that also require usage of NR-PDCCH in the future. For NR to be viable system that can continuously evolve and support new concepts and features in the future, careful planning of limited system resources such as NR-PDCCH should be considered.

Option 5: Paging message is transmitted by PDSCH and paging indication is transmitted non-scheduled physical channel

Option 5 is similar to Option 3 in that paging message is conveyed through PDSCH. However, it further required paging indication which informs whether paging message exists and for which set of UEs. For operation in mmWave, having a separate paging indication requires the paging indication to be sent in beam sweeping operation prior to the paging message. This may create additional latency in the obtaining the paging message.

For scenarios where beam sweeping operation is required to transmit broadcast information, having intermediate steps in obtain paging message and content should be avoided. Furthermore, usage of NR-PDCCH for paging does require some careful planning, such that it does not shrink the available common resource for NR-PDCCH. From these observations, our preference is to investigate further between option 2 and 3, where it seems to provide a good trade-off between paging resource flexibility and overhead incurred to support paging channels.

Proposal:

- NR should down-select further between Option 2: Paging message is transmitted in a non-scheduled physical channel and Option 3: Paging message is transmitted over PCH carried by semi-statically configured NR-PDSCH without DCI.

# Summary

This document presented our view on paging channel. The following is a summary of our proposal.

Proposal:

- NR should down-select further between Option 2: Paging message is transmitted in a non-scheduled physical channel and Option 3: Paging message is transmitted over PCH carried by semi-statically configured NR-PDSCH without DCI.

# References

- R2-1701746, “Minimum content size of a single paging message and number of UEs to be paged within a certain time period,” 3GPP TSG RAN WG2#97, Intel Corp.