3GPP TSG RAN WG1 Meeting #88                                              	R1-1702271

Athens, Greece 13th - 17th February 2017

Agenda item:	8.1.2.3.5

Source: 	AT&T

Title: 	Interference Measurement Framework for NR

Document for:	Discussion/Decision

# Introduction

In RAN1 NR Adhoc, interference measurement has been discussed with the following agreement.

Agreements:

- For NR, support at least two types of resources used for interference measurement in CSI configuration based on the following candidates:

  - ZP CSI-RS, NZP CSI-RS, DMRS

    - Including independent or joint usage of any combination of the above three candidates

  - Selection is to be done in RAN1#88

In addition, CRI reporting has been discussed and the following agreement was made.

Agreements:

- When a UE is configured with K>1 NZP CSI-RS resources, a UE can report a set of N UE-selected CSI-RS-resource-related indices

  - FFS: The name of the corresponding CSI reporting parameter

  - FFS: Maximum value of N (support of N=1 only is not precluded)

  - FFS: What other CSI parameter(s) are reported together with the CSI-RS-resource-related indices, if any, and joint reporting mechanism (in case of multiple reporting parameters)

  - FFS: Extension of this feature to other types of Resources

In this paper, we give our view on the resource to be used for interference measurement.

# Resources for interference measurement

ZP CSI-RS resource has been used for interference measurement since LTE Rel.11. The benefit of using ZP-CSI-RS for interference has been extensively discussed at that time: Since all the received power on ZP CSI-RS resource REs is interference, there is no need to extract the useful signal out (in comparison to any NZP RS- based interference measurement). Such operation increases the measurement accuracy with an even simpler UE implementation. It is straight forward to adopt ZP CSI-RS for interference measurement in NR.

Then the question is which other RSs can be used as additional resource for interference measurement. Both NZP CSI-RS and DMRS are being considered based on the agreement in RAN1 NR AdHoc.

There are pros and cons to considering DMRS and NZP CSI-RS. DMRS has higher density which means a better channel estimation (Although the detailed RS pattern is not yet decided, it’s very likely that DMRS will have higher density than NZP CSI-RS). On the other hand, DMRS is transmitted along with PDSCH thus unscheduled UEs can’t use it to estimate interference. More importantly, the transmission of NZP CSI-RS can be easily coordinated across multiple gNBs in a semi-static fashion while the coordination of DMRS/PDSCH transmission requires dynamic coordination. Considering that, we propose to use NZP CSI-RS for interference measurement in NR.

Proposal: Support ZP CSI-RS and NZP CSI-RS based interference measurement in NR

# Using CRI to select interference source

In RAN1 NR Adhoc, CSI acquisition framework has been refined: RS and IM settings are merged into resource settings. Furthermore, CRI is agreed to select multiple resources for the CSI report. The combination of these two agreements open a door to use CRI to select both the signal and the interference resources for the CSI report.

Figure 1 illustrates an example use case of using CRI to select both signal and interference sources. When configured by multiple NZP CSI-RS resources as interference measurement sources, the UE can use CRI to select a NZP CSI-RS resource for interference measurement. The selection is based on UE’s measurement on all the interference resources, e.g. UE may find I3 >> I2, therefore, UE selects to report the interference from I3. The detailed feedback of the selected interference source can be further decided in the work item, e.g. PMI of the NZP CSI-RS resource (for interference) can be fed back as a best/worst companion PMI.

NZP CSI-RS Resource-1

NZP CSI-RS Resource-2

NZP CSI-RS Resource-3

S1

I2

I3

ZP CSI-RS Resource

I_out

CQI=S1I2+Iout

CRI to select resources

Figure.1 using CRI to select the signal and interference sources for CSI report

Proposal: Consider to use CRI to select the NZP CSI-RS resources for interference measurement.

# Conclusion

In this paper, we give our view on the beam recovery mechanism for connected UE. In particular, we propose:

Proposal: Support ZP CSI-RS and NZP CSI-RS based interference measurement in NR

Proposal: Consider to use CRI to select the NZP CSI-RS resources for interference measurement.

# Reference

- RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO.