3GPP TSG-RAN WG1 #88	R1-1702362

Athens, Greece, 13th – 17th February, 2017

Source:	Ericsson

Title:	Performance of Advanced CSI with Power Limitation

Agenda Item:	7.2.2.8

Document for:	Discussion and Decision

# Introduction

In RAN1#87, a working assumption was made regarding the advanced CSI codebook proposed in [1].

In this contribution, we present evaluation results comparing the Rel-14 advanced CSI codebook, according to the working assumption, with a Rel-13 baseline. As the advanced CSI codebook contains non-constant modulus precoders, we also assess the impact of per-TXRU power limitation.

# Discussion

In the advanced CSI codebook, precoders are created by linearly combining two DFT vectors as for example:

W(1)=W1W2(1)= BISPc0,0c1,0=w0,0w1,0

wr,0=i=01bIS(i)⋅pi⋅cr,0,i, r=0,1.

This means that the constant modulus property of the Class A DFT codebook is broken for advanced CSI and, for a given TFRE, the antenna ports (and correspondingly the PAs) may have an unequal power loading. To avoid clipping each individual PA, the eNB should make a power backoff so that the power of the strongest PA does not overshoot, i.e. a per-TXRU rather than a sum-TXRU power constraint should be applied. The power constraint per port/PA of course applies in the time domain and is the sum over all frequencies and layers, so even if an individual per-layer precoder has a certain unfavorable power loading, it is not certain that the average power loading summed across frequencies and layers will be unfavorable. In fact, for larger bandwidths and MU ranks, the power loading across ports should become similar due to the central limit theorem.

Another thing to consider is that the power utilization of the PAs is not perfect even if constant modulus codebook is applied, as frequency-selective precoding and high rank transmission (due to MU-MIMO) is used, and so, it is not clear if the additional unequal power loading introduced by a non-constant modulus codebook will significantly impact performance. Furthermore, most gains with MU-MIMO is found in dense urban scenarios, which are generally interference-limited rather than power-limited. Thus, the additional power backoff introduced may not necessarily have a significant impact on performance.

Observations:

- The non-constant modulus property of the Rel-14 codebook may lead to unequal power loading of antenna ports, requiring eNB to make a power backoff to avoid PA clipping

  - However, it is not certain that this will have a significant performance impact

# Simulation results

In this section we present evaluation results comparing sum-TXRU and per-TXRU power constraint when using the Rel-14 advanced CSI codebook. The simulations are performed in 3GPP 3D UMi with an 8x4 cross-polarized antenna array with a 4x1 subarray virtualization for 16 antenna ports using the 3GPP FTP-1 traffic model with 100 kB packet size. The UEs are equipped with 2 RX antennas and dynamic rank adaptation as well as dynamic SU/MU-switching is used. Additional SLNR processing of the reported precoders are used to suppress MU interference. Remaining simulation assumptions are presented in the Appendix. The baseline system uses the Class A codebook.

Table 1: Evaluation results of Rel-14 advanced CSI codebook performance over Rel-13 codebook, with and without per-TXRU power constraint, measured at 70% baseline RU

| Sum-power constraint |  |  |
| --- | --- | --- |
| Number of antenna ports | Average gain | Cell edge gain |
| 16 | 24% | 52% |
| Per-TXRU power constraint |  |  |
| Number of antenna ports | Average gain | Cell edge gain |
| 16 | 22% | 47% |

Observations:

- Adding a per-TXRU power constraint reduces cell-edge throughput gains by ~0-5%

- For 16TX, substantial UTP gains over Rel-13 is still observed with per-TXRU power constraint

# Conclusion

In this contribution, we have discussed the impact of per-TXRU power constraint on Rel-14 advanced CSI codebook performance and presented corresponding evaluation results. The following observations have been made:

Observations:

- The non-constant modulus property of the Rel-14 codebook may lead to unequal power loading of antenna ports, requiring eNB to make a power backoff to avoid PA clipping

  - However, it is not certain that this will have a significant performance impact

- Adding a per-TXRU power constraint reduces cell-edge throughput gains by ~0-5%

- For 16TX, substantial UTP gains over Rel-13 is still observed with per-TXRU power constraint

# References

- R1-1613647 WF on Advanced CSI Codebook, Ericsson, et.al., 3GPP TSG RAN WG1 Meeting #87 Reno, US, November, 2016

# Appendix

## Simulation parameters

| Simulation Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD |
| Antenna Configurations | 16 TX: 8x4 with 4x1 virt., UMi (108° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Advanced CSI codebook (when used) | According to working assumption Number of beams 2, with beam restriction for 32TX Beam space rotation hypotheses per dimension: 4 Beam power: Wideband, 4 States Co-phasing: Q-PSK |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) |
| Traffic model | FTP Model 1, 100 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency Max 8 MU layers |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for. Channel estimation error modeled. |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS MU-MIMO |