3GPP TSG RAN WG1 Meeting #88	R1-1702913

# Athens, Greece 13th - 17th February 2017

Agenda item:	8.1.2.1

Source: 	Samsung

Title: 	Continuous precoding of NR DMRS in time domain

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1 NR Ad-Hoc [3], following agreements on PRB bundling are agreed:

Agreements:

- For the DL, NR supports configurable PRG size for data DMRS

  - FFS: Signaling for the configuration of PRG size (semi-static or dynamic)

  - FFS: available options/schemes.

- FFS: continuous precoding in time domain for DMRS channel estimation with the following examples:

  - Semi-static measurement window

  - Dynamic indication of precoding sets

  - Support of continuous precoding when gNB supports slot aggregation

  - Contiguous time allocations

- FFS: multiplexing of DMRS ports using time domain OCC between aggregated slots

  - Note: Consider impact of DMRS pattern

  - Note: Other code is not precluded.

In this contribution, we discuss continuous precoding of NR DMRS in time domain.

Discussions on precoding granularity in time domain

In LTE, DMRS precoding granularity in time domain is fixed to one subframe regardless of used precoding in data transmission. In Figure 1, DMRS channel estimation procedure in LTE is depicted as shown below:

Figure 1 Hybrid CSI-RS transmission for multiplexed services in NR

When reciprocity between uplink and downlink channel is not supported, eNB can realize preferred transmission layer and precoding by using UE’s periodic and/or triggered feedback. For periodic CSI reporting, it should be noted that information on precoding in eNB side does not change between CSI reporting instances. Considering such aspects, it is quite natural to use identical precoding between the CSI reporting instances for a given UE if UE is scheduled. As well as periodic CSI reporting case, UE which uses aperiodic CSI reporting experiences similar situation. Since information on precoding in eNB side only changes when eNB triggers, using identical precoding between UE reports would be natural. The situation is similar for channel reciprocity case since eNB can acquire channel information only when UE transmits SRS to eNB.

In order to support joint estimation of DMRS, specification support can be considered in NR. One possible alternative would be semi-static configuration of measurement window for DMRS channel estimation. In Figure 2, the example of joint DMRS estimation by using measurement window is provided.

Figure 2 DMRS joint estimation based on the semi-static measurement window

As shown in Figure 2, eNB can configure semi-static measurement window of DMRS to UE by using higher layer. Based on the configuration, UE can jointly estimate DMRS and enhance estimation performance. In this scheme, the performance can be degraded due to the restriction.

Another method to support joint estimation of DMRS would be dynamic indication of DMRS precoding set to UE. Figure 3 shows the exemplary operation of dynamic DMRS precoding set indication.

Figure 3 DMRS joint estimation based on the dynamic indication of precoding set

Moreover, joint estimation of DMRS can be considered when UE is scheduled on multiple slots. Figure 4 shows exemplary operation.

Figure 4 DMRS joint estimation in multi slot scheduling

In 3GPP RAN1 #86bis [2], support of slot aggregation is agreed. When eNB and UE support slot aggregation, data transmission can be scheduled to span multiple slots. In this case, supporting identical precoding on multiple slot can be implicitly or explicitly indicated to UE to support joint estimation of DMRS.

Observations:

- For both channel reciprocity and non-reciprocity case, CSI acquisition may not be done in every slots.

  - For periodic CSI reporting, acquisition of CSI is limited to UE reporting instance.

  - For aperiodic CSI reporting, CSI can be provided only when eNB triggered CSI.

  - For channel reciprocity case, channel information can be done only when UE transmits SRS.

- Considering availability of CSI, support of continuous precoding on DMRS in time domain can bring channel estimation accuracy enhancement.

Proposals:

- Support continuous precoding on DMRS in time domain.

- Consider following alternatives to support continuous precoding on DMRS in time domain:

  - Semi-static measurement window.

  - Dynamic indication of precoding sets.

  - Support of continuous precoding when eNB supports slot aggregation.

Evaluation results

will be updated..

Conclusions

In this contribution, continuous precoding of NR DMRS in time domain is discussed. Based on the discussions, the following observations and proposals are provided:

Observations:

- For both channel reciprocity and non-reciprocity case, CSI acquisition may not be done in every slots.

  - For periodic CSI reporting, acquisition of CSI is limited to UE reporting instance.

  - For aperiodic CSI reporting, CSI can be provided only when eNB triggered CSI.

  - For channel reciprocity case, channel information can be done only when UE transmits SRS.

- Considering availability of CSI, support of continuous precoding on DMRS in time domain can bring channel estimation accuracy enhancement.

Proposals:

- Support continuous precoding on DMRS in time domain.

- Consider following alternatives to support continuous precoding on DMRS in time domain:

  - Semi-static measurement window.

  - Dynamic indication of precoding sets.

  - Support of continuous precoding when eNB supports slot aggregation.

- References

- RP-160671, “New SID Proposal: Study on New Radio Access Technology”, NTT DOCOMO.

- 3GPP TSG RAN1#86bis, Chairman’s Note

- 3GPP TSG RAN1 NR Ad-Hoc, Chairman’s Note

- R1-1702943, “CSI acquisition framework for DL NR MIMO”, Samsung.