3GPP TSG-RAN WG1 Meeting #88bis	R1-1704792

Spokane, US, 3 - 7 April 2017

#### Agenda item:		7. 2.1.2.3.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:					sPUCCH Design for shortened TTI

Document for:		Discussion and Decision

# 1. Introduction

The SI on Latency reduction techniques for LTE [1] was closed at RAN#72 and based on the outcome documented in the TR [2], a follow-up WI was approved in [3]. The main objectives of the WI in [3] are given by:

The objective of this work item is to specify shortened TTI operation and shortened processing time for both legacy (1ms) TTI and shortened TTI. The specified solution should cover the case of carrier aggregation and non-carrier aggregation. Aim for a similar design as possible independent of frame structure.

…

For Frame structure type 1: [RAN1, RAN2, RAN4]

- Specify support for a transmission duration based on 2-symbol sTTI and 1-slot sTTI for sPDSCH/sPDCCH

- Specify support for a transmission duration based on 2-symbol sTTI, 4-symbol sTTI, and 1-slot sTTI for sPUCCH/sPUSCH

  - Down-selection is not precluded

- Study any impact on CSI feedback and processing time, and if needed, specify necessary modifications (not before RAN1 #86bis)

For Frame structure type 2: [RAN1, RAN2, RAN4]

- Specify support for a transmission duration based on 1-slot sTTI for sPDSCH/sPDCCH/sPUSCH/sPUCCH

- Study any impact on CSI feedback and processing time, and if needed, specify necessary modifications (not before RAN1 #86bis)

Follow the recommendation made in [2] when specifying for support of transmission duration based on 2-symbol sTTI, 4-symbol sTTI, and 1-slot sTTI.

In RAN1#87 following agreements were made:

Agreements:

- At least the following formats should be designed for sPUCCH:

  - For sPUCCH supporting up to 2-bit HARQ-ACK and/or SR (if any)

    - DMRS based demodulation for 7-symbol sTTI

    - FFS on the formats and DMRS design for 2-symbol sTTI

  - For sPUCCH supporting more than 2-bit HARQ-ACK and SR (if any)

    - DMRS based demodulation for all sTTI lengths

- FFS on encoding methods

- FFS on sPUCCH with channel selection for up to 4-bit HARQ-ACK.

- FFS on support of frequency hopping

In RAN1#88 the summary of candidate designs for 2-OS PUCCH with 1-2 bits agreed in R1-1704037, and 2-OS Design for more than 2 bits is agreed in R1-1703744. For 7-symbol sPUCCH the following was agreed:

Agreements:

- Intra-sTTI frequency hopping is supported for at least one slot-based sPUCCH format

- No intra-sTTI frequency hopping is supported for at least one slot-based sPUCCH format

- No dynamic switch between intra-sTTI frequency hopping and no intra-sTTI frequency hopping for the same slot-based sPUCCH format

- FFS: whether the same slot-based sPUCCH format can support both intra-sTTI frequency hopping and no intra-sTTI frequency hopping

In this document, we discuss the principles of sPUCCH design. sPUCCH resource management related aspects are discussed in [4]

# 2. Design Targets for sPUCCH

PUCCH design is one of the key aspects in facilitating latency reduction. In order to be able to reduce HARQ round-trip time, the UL control channel conveying HARQ-ACK will need to be shortened similarly as the related PDSCH carrying the data transport block(s). Therefore it seems apparent that a shortened PUCCH will need to be specified to carry at least HARQ-ACK. In principle, it should be straight forward to apply the same design for conveying Scheduling Request as well, which in turn may provide a minor decrease in the observed latency.

Observation #1: Primary contents for sPUCCH are HARQ-ACK (+SR).

Other content for shortened PUCCH could be CSI, but its need is far less clear to us. In LTE, periodic wideband CSI reports are transmitted on PUCCH when there is no simultaneous PUSCH transmission. However, firstly, periodic CSI reporting involves inherently relatively large latency depending on the reporting periodicity and/or availability of measurement resource, and the length of PUCCH – 1 ms or less – does not have a significant impact on the overall latency. Secondly, the wideband nature (due to limited capacity) of the report implies that the precision of the feedback is rather coarse and cannot be improved significantly by more frequent reporting. Thirdly, faster and more frequent ACK/NACK transmitted on sPUCCH improves the link adaptation already by means of OLLA without any specification effort. Finally, it should be possible for the network to trigger Aperiodic CSI report to be transmitted using shortened PUSCH, which further reduces the need for transmitting periodic CSI over shortened PUCCH. Therefore we think that no CSI enhancements for periodic reporting on shortened PUCCH are needed.

Observation #2: No CSI enhancement for periodic CSI reporting on sPUCCH are needed.

When it comes to sPUCCH design, a few requirements need to be considered. In LTE, the coverage of the whole system is often limited by the PUCCH. In order to mitigate this, various means have been specified including e.g. PUCCH repetition to enhance HARQ-ACK coverage. Obviously, for shorter TTI operation to make sense, the impact on coverage should be minimized. On the other hand, shorter TTI means that the energy per bit for e.g. HARQ-ACK transmission is inevitable reduced, and hence there will be some penalty in terms of cell coverage. This issue is more pronounced with TTI lengths less than 0.5 ms and may easily end up limiting the coverage of sTTI feature as a whole.

Observation #3: PUCCH coverage may easily become a bottleneck for sTTI, especially for sTTI length of less than one slot.

In order to make shorter TTI operation an attractive feature, special attention needs to be put on sPUCCH coverage. We see that the as a starting point, one needs to keep the basic design properties of 1-ms PUCCH, namely frequency hopping to achieve frequency diversity. Furthermore, sPUCCH waveform should have low cubic metric to guarantee uncompromised operation at maximum TX power.

Proposal #1): sPUCCH should aim at as large coverage as possible, and

  - Retain support for frequency diversity (i.e. hopping across band edges)

  - Have low cubic metric similar or close to that of SC-FDMA

Another key design target for LTE PUCCH has been low overhead, which has been achieved with high multiplexing capability. Depending on the PUCCH format (and correspondingly payload), more than 10 UEs can share the same PUCCH PRB. We see that the same design target is equally important with shorter TTIs as well, and efficient multiplexing of different sPUCCH UEs, as well as both sPUCCH and legacy PUCCH UEs should be supported within a PRB.

Observation #4: Increase in UL control overhead due to sTTI should be minimized.

Proposal #2: Multiplexing of sPUCCHs should be supported within the same PRB.

Proposal #3: Multiplexing of sPUCCH and legacy PUCCH should be supported within the same PRB.

# 3. sPUCCH Design Candidates

In the following we provide performance analysis for the sPUCCH design candidates for 2-symbols and 7-symbol cases.

## 3.1 2-symbol sPUCCH design for up to 2-bit HARQ-ACK and/or SR

The 1 or 2-bit design options for 2-symbol sPUCCH considered in R1-1704037 are:

- Option 1: sequence based

  - Option 1a: ACK/NACK are indicated by different combinations of cyclic shifts on symbols, with the cyclic shifts on all symbols in each combination are NOT all the same.

  - Option 1b: ACK/NACK are indicated by different cyclic shifts, with the same cyclic shift on all symbols.

- Option 2: DMRS based

  - Option 2a: TDM of 1 DMRS symbol and 1 or 2 data symbols

  - Option 2b: DMRS sequence and data sequence are transmitted on the same symbol with different cyclic shifts.

The key differences between the above options are two-fold: options 1a, 1b, and 2b apply frequency hopping between the sPUCCH symbols, whereas in option 2a both the DMRS as well as the data symbols are transmitted on the same PRB. Another key different is in overhead, i.e. the number of cyclic shifts required: Options 1a and 1b consume 4 cyclic shifts, Option 2a uses a single CS, and Option 2b require two cyclic shifts, one for data and one for DMRS.

## 3.2 Performance evaluation for 2-symbol sPUCCH

In this section, we provide the BLER vs SNR performance for the different options for 2-symbol sPUCCH as discussed in previous section. In general, option 1a, 1b and 2b provide better performance in comparison to option 2a by utilizing frequency hopping. Sequence based option 1a and 1b perform better than DMRS-based option 2b. However, the sequence based options use 4 sequences (4 cyclic shifts) for transmitting 2 bits and therefore reduce the multiplexing capacity. On the other hand, option 2b use only 2 cyclic shifts and hence allow for greater multiplexing capacity.

Observation #5: DMRS-based option 2b provides a good trade-off between the performance and multiplexing in comparison to sequence-based options 1a and 1b.

Proposal #4: Option 2b is selected for 2-symbol sPUCCH transmission with up to 2 bits.

Figure 1: BLER vs SNR for 2-symbol sPUCCH

## 3.3 7-symbol sPUCCH design supporting more than 2-bits

Figure 2. illustrates our preferred candidate design for 7-symbol sPUCCH with moderate UCI payloads of up to 11 bits (“sPUCCH Format 2”). Rel-8 length-12 UL DM-RS sequences are applied, with a distinct cyclic shift assigned for each sequence. In the DM-RS symbols (i.e. symbol #1, #4), one unmodulated DM-RS sequence is transmitted (e.g. CS_0), whereas in data symbols two QPSK-modulated sequences are transmitted in parallel (e.g. M-CS_0, M-CS_1), separated with cyclic shifts. Frequency hopping can be applied between “short slots”, or alternatively all 7 symbols can be transmitted on a single PRB depending on the configuration. The described arrangement results in having 20 uncoded bits in a 0.5 ms TTI, i.e. the same number as with the current LTE PUCCH format 2. This allows for directly reusing the PUCCH Format 2 Reed-Muller channel coding scheme. Similarly as with the 1/ 2 bit design, also in this case legacy PUCCH format 2 users - as well as sPUCCH Format 1 users – can be multiplexed on the same PRB(s).

Figure 2. sPUCCH Format 2 design supporting up to 11 bit payload with 0.5 ms TTI.

## 3.4 Performance evaluation for 7-symbol sPUCCH

In this section we present performance evaluation for 7-symbol “sPUCCH format 2” and “sPUCCH format 3” (larger format which does not apply spreading). The key properties of these formats are summarized in Table 1, while the BLER performance is depicted in Figure 3. Based on the analysis it can be concluded that sPUCCH format 2 provide solid performance while allowing for multiplexing with legacy PUCCH, hence minimizing the associated overhead.

Observation #6: 7-symbol sPUCCH design (sPUCCH format 2) with Reed-Muller coding provide solid performance for supporting up to 10-bit payload while allowing for multiplexing with legacy PUCCH.

Proposal #5: The 7-symbol sPUCCH format supporting more than 2 bits is based on QPSK modulated, cyclically shifted CAZAC sequences, where multiple cyclic shifts can be assigned for a single user.

| PUCCH format | HARQ-ACK payload (bits) | TTI length | # of DMRS | Spreading | Frequency Hopping | Coding |
| --- | --- | --- | --- | --- | --- | --- |
| sPUCCH Format 2 | 2 - 10 | 7 OS | 2 | Yes | Yes | Reed-Muller |
| sPUCCH Format 3 | 10 - 40 | 7 OS | 2 | No | Yes | TBCC |

Table1. 7-symbol sPUCCH Formats

Figure 3. BLER vs SNR for 7-symbol sPUCCH

# 4. Summary

In this contribution we consider aspects impacting PUCCH design for sTTI. Based on the discussion, we make the following observations and proposals related to PUCCH design:

Observation #1: Primary contents for sPUCCH are HARQ-ACK (+SR).

Observation #2: No CSI enhancement for periodic CSI reporting on sPUCCH are needed.

Observation #3: PUCCH coverage may easily become a bottleneck for sTTI, especially for sTTI length of less than one slot.

Proposal #1): sPUCCH should aim at as large coverage as possible, and

  - Retain support for frequency diversity (i.e. hopping across band edges)

  - Have low cubic metric similar or close to that of SC-FDMA

Observation #4: Increase in UL control overhead due to sTTI should be minimized.

Proposal #2: Multiplexing of sPUCCHs should be supported within the same PRB.

Proposal #3: Multiplexing of sPUCCH and legacy PUCCH should be supported within the same PRB.

Observation #5: DMRS-based option 2b provides a good trade-off between the performance and multiplexing in comparison to sequence-based options 1a and 1b.

Proposal #4: Option 2b is selected for 2-symbol sPUCCH transmission with up to 2 bits.

Observation #6: 7-symbol sPUCCH design (sPUCCH format 2) with Reed-Muller coding provide solid performance for supporting up to 10-bit payload while allowing for multiplexing with legacy PUCCH.

Proposal #5: The 7-symbol sPUCCH format supporting more than 2 bits is based on QPSK modulated, cyclically shifted CAZAC sequences, where multiple cyclic shifts can be assigned for a single user.

# 5. References

- RP-150465, New SI proposal Study on Latency reduction techniques for LTE, Ericsson

- TR 36.881, Study on latency reduction techniques for LTE

- RP-161922. New Work Item on shortened TTI and processing time for LTE, Ericsson

- R1-1704799,	sPUCCH resource management for shortened TTI, Nokia, Alcatel-Lucent Shanghai Bell, RAN1#88bis, April 2017

# Appendix A	Link level simulation assumptions

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| TTI length | 2/7 symbols |
| Channel model | ETU |
| UE speed | 3km/h, 60km/h |
| Antenna configuration | 1Tx(UE), 2Rx(eNB) |
| CP length | Normal |
| Receiver type | MMSE |
| Channel estimation | Practical |