3GPP TSG RAN WG1 Meeting #88bis                                    	R1-1705012

Spokane, Washington, USA, 3rd – 7th April 2017

Agenda item:    7.2.6.2

Source:             Qualcomm Incorporated

Title:                  Efficient monitoring of DL control channels

Document for:  Discussion and Decision

- Introduction

- Introduction

In RAN#75 a new work item (WI) named even further enhanced MTC for LTE [1] was introduced. The objectives of the WI is to study and, if found beneficial for idle mode paging and/or connected mode DRX, specify physical signal/channel that can be efficiently decoded or detected prior to decoding the physical downlink control/data channel.

In this contribution, we provide our views on introducing a new physical channel (“wake up signal”) that UE can monitor prior to MPDCCH which informs UE whether to monitor MPDCCH or not. We cover use cases, analysis of power savings, as well as some PHY/MAC design aspects of the wake-up signal. In this contribution, we focus on the paging use case. However, the techniques do apply to connected mode DRX as well.

- Wake-up Receiver Architecture

Figure 1: Typical idle mode processing (no paging case)

Typical processing at UE when it is in idle mode is shown in Fig. 1. The UE monitors for page in its paging occasions which are configured periodically with a period equal to the DRX cycle. The UE is in a sleep mode prior to its paging occasion. It wakes up slightly earlier than its paging occasion to obtain timing/frequency sync and reconfirm the serving cell. Once it has timing/frequency sync, it goes on to monitor the MPDCCH. If there is no page (which is the case in Fig. 1), the UE then goes back to sleep until the next paging occasion. The amount of time UE is awake includes the time for sync, time for monitoring MPDCCH, as well as some wake up and ramp down time. Since the MPDCCH payload contains several bits, the time to decode MPDCCH can span several 10s of subframes.

The awake time of the UE can be substantially reduced by sending, for example, a “1-bit” wake up signal that lets the UE know whether it should monitor the MPDCCH or not. As shown in Fig. 2, if UE doesn’t detect the wake-up signal (or it decodes it and the wake-up signal indicates no MPDCCH present) the UE does not monitor the MPDCCH. Since the length of the wake-up signal is expected to be much smaller than that of MPDCCH (as it effectively conveys much fewer bits) the UE awake time is significantly reduced and hence the corresponding power consumption.

Figure 2 : Idle mode processing with wake-up signal (no paging case)

Fig. 3 shows the processing when a page is sent to the UE. The UE is expected to detect / decodes the wake up signal, go on to monitor MPDCCH, and then associated PDSCH.

Figure 3 : Idle mode processing with wake-up signal when UE is paged

The wake-up signal also enables use of much more power efficient HW architectures. One example is shown in Fig. 4. MPDCCH monitoring involves complex basedband processing but detection of wake-up signal may be possible by new low power wake up receiver (e.g. that maybe does just correlations). The wake-up receiver runs only to detect the wake-up signal. The full baseband modem is turned on only when the wake-up signal is detected. This enables extracting even more gains than just those obtained by reducing the awake time. Note that the wake-up receiver and baseband modem block-level partitioning shown in the Fig. 4 is conceptual and there are a number of ways to realize in actual HW implementation, including operating WUS detection as a low power mode functionality of the modem.

Figure 4 : Wake up receiver architecture

- Use Cases and Power Analysis

  - Use Case

The wake-up signal / receiver is primarily targeted towards MT data traffic use case where page monitoring power is a dominant source of power consumption. The wearables use case from [2] can probably be considered to study the potential benefits of the wake-up signal. The details for this use case is summarized in Table 1.

Table 1: Use Case

| Use case | Wearable [2] |
| --- | --- |
| Traffic Characteristic Description | MT 100 bytes + 20 byte ACK. 1 message per 2 hours |
| Latency (paging cycle) | 5 seconds |
| MCL | 144 dB |
| Battery life | Coin Cell : 1 year Rechargeable : 2 weeks |

  - Power Models

We consider the power models proposed in [2],[4] to evaluate the benefits of the wake-up receiver. The details of the power models are shown in Table 2 and Table 3. Note that these models are used to obtain a rough estimate of power savings and may not represent the actual power consumption/savings. Since the model is [2] was for NB-IoT we assume a 20% higher active power for eMTC.

Table 2 : Model l - UE power consumption model for mMTC battery life evaluation [2]

| Operating mode |  | Value | Notes |
| --- | --- | --- | --- |
| Transmit | Loss from PA to radiated power | FD-FDD: 3dB HD-FDD / TDD: 1.5dB | Accounts for switches, duplexers, insertion loss |
|  | PA efficiency | Integrated PA: 45% External PA: 50% |  |
|  |  | 60mW | Other RF and baseband circuitry |
| Receive |  | 60mW | RF and baseband circuitry |
| Sleep |  | 3mW | Corresponds to maintaining accurate timing by keeping RF frequency reference active. |
| Standby |  | 0.015mW | Common assumption. (Deep sleep mode) |
| Sleep to awake switching |  |  | Not modelled |

Table 3 : Model 2 - UE power consumption [4]

| Power consumption: | Value | Units |
| --- | --- | --- |
| Rx | 50 | mA |
| Tx (good conditions) | 50 | mA |
| Tx (bad conditions) | 250 | mA |
| Sleep mode (legacy DRX) | 0.5 | mA |
| Deep sleep mode (PSM/eDRX) | 0.05 (– 0.005) | mA |
| Wake-up / ramp-down (sleep mode and deep sleep mode) | 25 | mA |
| DRX time (legacy DRX with 1.28 sec cycle) | 1.4 | mA |
| Latencies: |  |  |
| PICH monitoring | 10 | msec |
| DRX time (4 x 1.28 sec) | 5120 | msec |
| Wake-up (deep sleep mode) | 150 | msec |
| Wake-up (sleep mode) | 14 | msec |
| Ramp-down (sleep mode and deep sleep mode) | 6 | msec |

Note that in both these models the light sleep mode power and the standby/deep sleep mode power are substantially different. The wake-up time from the stand-by/deep sleep mode to active mode can be very significant (e.g. in Model 2 it is 150ms) and hence UE is expected to move to deep sleep mode only when the savings in sleep power is more than the power spent on wake-up every DRX cycle. For example, corresponding to Model 2, UE would use deep sleep mode only when the sleep duration T ms is such that T x difference in sleep current (0.5-0.05) (mA) is larger than 150ms (wake up time) x wake up current (25mA), i.e., T >= 8 seconds.

  - Power Analysis

In this section, we analyze the power savings with a wake-up signal assuming a simplistic model of UE only monitoring paging but not receiving any page. We assume no UL data transfers as well. Since in wearables use case from Table 1, a small amount of data is sent once every 2 hours, we don’t expect the trend in the results with more accurate modelling to differ significantly from the trend shown here.  The savings are studied as a function of different MPDCCH Rmax and DRX cycles. The other assumptions made are as follows. We assume that the sync duration is 10ms. For the wake-up signal we assume the length is equal to length of time needed for SYNC+ Rmax/16. The inherent assumption here is that the wake-up signal has only one bit but MPDCCH has 30+ bits payload including CRC and hence Rmax/16 number of subframes should be sufficient for the wake-up signal. The detailed assumptions are also summarized below:-

| Parameter | Value |
| --- | --- |
| Rmax | [8,16,32,64,128,256] |
| DRX | 128,256,512,1024 frames |
| SYNC duration | 10ms |
| Wake up signal length | SYNC duration + Rmax/16 |
| FA rate for WU signal detection | 2% |

The resulting power savings computed due to savings in awake time is shown in the Fig. 5 below. The results are assuming the sleep mode is light sleep mode (i.e. not the deep sleep/standby mode). The savings of course will be higher (50% or more across all cases in the plot) if we can assume UE can go to deep sleep mode without significant switching time.

Figure 5: Power savings with wake-up signal due to reduced awake time

With a new hardware architecture as discussed in Section 2 it may be possible for the baseband hardware to go into a deep sleep mode while the power consumption of the new wake up receiver HW is lower than the baseband hardware power in light sleep mode. The power savings in that case with a wake-up receiver can be very significantly higher. For example, Fig. 6 shows power saving for model 1 if we assume that the overall sleep power can also be reduced by 50%.

Figure 6: Power savings with wake-up signal assuming new low power wake up receiver hardware saves 50% of both sleep and active power of sync/wake-up signal monitoring

The results shown above are sensitive to the power model but as is clear, there are significant gains for small DRX cycles or for large PDCCH repetition factors.

Observation 1: In the paging monitoring dominated use cases, the power savings with wake up receiver signalling are substantial especially for small DRX cycles / large Rmax. Savings can be significantly larger with power efficient hardware optimized for wake-up receiver for DRX cycles where deep sleep mode is not efficient.

- Power consumption as function of Rmax

In Fig. 7 we plot the power consumed for a given DRX cycle as function of Rmax corresponding to power model 1. As can be seen, since the WUR signal is much smaller in length than the MPDCCH, the impact of Rmax on the power consumption is significantly reduced. Hence, use case such as wearables can be enabled for deeper coverage without significantly impacting the overall power consumption and battery life of the device.

Figure 7: Idle mode power consumption as function of Rmax

Observation 2: Use of wake-up signal makes the power consumption less sensitive to MCL enabling the use cases to operate at higher MCL without significantly impacting the battery life.

Based on observation 1 and 2, we do see clear benefits of introducing wake up signal. We hence make the following proposal.

Proposal 1: A new physical channel that carries the wake-up signal is introduced for eFeMTC. UE is required to decode the control channel in idle/connected mode DRX only when the wake-up signal is detected.

- Wake-up signal resource configuration

In this section, we present our views on configuration of the wake-up signal resource (WUSR) and mapping to UE groups/MPDCCH search space to it. The proposed operation is shown in Fig. 8. We propose that a group of WUS resource occasions is configured with a certain periodicity. In each occasion, there are one or more WUS resources configured. UE determines the WUS resource to monitor based on UE-ID/RNTI/Rmax etc. The WUS indicates the presence or absence of MPDCCH for a group of subframes. For example, in Fig. 8, the WUS resources shown in a particular colour indicate the presence / absence of MPDCCH on subframes of the same colour.

The motivation for having multiple WUS resources per occasion is as follows. If the paging probability system wide itself is very low (e.g. only 1 occasion out of 10 have any paging present), we can just configure 1 WUS resource in each occasion. All UEs would wake up in the occasion if any UE got paged. However, since this is infrequent UE would still save power. On the other hand, if every occasion has at least one UE being paged, with just 1 WUS resource in each occasion, all UEs will be on all the time – no power saving (actually extra power consumed for WUS resource monitoring). It is hence important to split UEs into different WUS resources.

Figure 8: Configuration of wake-up signal resource

Proposal 2: Wake up signal resource occasions are configured with certain periodicity. In each occasion, multiple wake up signal resources can be configured which all correspond to the same set of subframes. Different UEs may monitor different wake up signal resources within the same occasion.

- Summary

The observations and proposals made in this contribution are summarized below.

Observation 1: In the paging monitoring dominated use cases, the power savings with wake up receiver signalling are substantial especially for small DRX cycles / large Rmax. Savings can be significantly larger with power efficient hardware optimized for wake-up receiver for DRX cycles where deep sleep mode is not efficient.

Observation 2: Use of wake-up signal makes the power consumption less sensitive to MCL enabling the use cases to operate at higher MCL without significantly impacting the battery life.

Proposal 1: A new physical channel that carries the wake-up signal is introduced for eFeMTC. UE is required to decode the control channel in idle/connected mode DRX only when the wake-up signal is detected.

Proposal 2: Wake up signal resource occasions are configured with certain periodicity. In each occasion, multiple wake up signal resources can be configured which all correspond to the same set of subframes. Different UEs may monitor different wake up signal resources within the same occasion.

References

[1] RP-170732, “New WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm, 3GPP TSG RAN Meeting #75, Dubrovnik, Croatia, March 6 - 9, 2017.

[2] R1-168485, “Battery Lifetime Model for NR”, Sony,  u-blox, Sierra Wireless, 3GPP TSG RAN WG1 Meeting #86, Gothenburg, Sweden 22nd - 26th August 2016.

[3] 3GPP Technical Report 45.820, “Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)”, December 2015.

[4] R2-151142, “eDRX performance”, Ericsson, 3GPP TSG RAN WG2 #89bis, Bratislava, Slovakia, April 20-24, 2015