3GPP TSG-RAN WG1 #88bis Meeting	R1-1705856

Spokane, WA, USA, 3rd - 7th April, 2017

Agenda item:		8.1.4.1.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Segmentation principles for LDPC

Document for:		Discussion and Decision

# 1 	Introduction

During RAN1 #86 meeting, the following agreements were made on the flexibility of channel coding techniques [1].

Agreement:

- Channel coding techniques for NR, should support the following:

  - Info block size K flexibility:

    - Granularity at lower end of range of K = [D1] bits

      - D1 may be different for control and data channels

      - FFS whether D1 may be different for different code rates

    - FFS whether the granularity is coarser at higher values of K

    - Shortening (i.e. assigning info bits to known values, e.g. 0) may be used to provide info block size flexibility

  - Codeword size flexibility:

    - Basic code design with rate matching (i.e., puncturing and/or repetition) supports 1-bit granularity in codeword size

In this contribution, we share considerations on code segmentation mechanism based on LDPC code for eMBB data channel.

# 2 	Code segmentation in LTE system

In LTE system, only Turbo code is used for data channel. Quadratic Polynomial Permutation (QPP) interleaver is used as the internal interleaver for Turbo code. The bit number space for QPP interleaver size may be 8/16/32/64, which is related to the input information block size. When the input bit number is not aligned with the size of QPP interleaver, the padding bits is used. To guarantee the realization complexity, the maximum code block size (CBS) is 6144. Elaborate code segmentation mechanism is specified when input bits for Turbo code are larger than 6144 [2]. The number of CB is determined by the quotient of total information bit number and the maximum CBS. Then, CBS K+ and K- are used for code segmentation. Here, K+ is the smallest size of Quadratic Polynomial Permutation (QPP) interleaver to contain the information bits of each CB, which is determined by equal division for different CBs. K- is the maximum size of QPP interleaver smaller than the previously determined CBS K+. Finally, the code block number for K+ and K- is determined by the specified formula [2] to reduce the padding bit number. Based on LTE code segmentation mechanism, we have the following observation:

Observation 1: Padding technique can be used to align transmit information block to the supported block sizes at the encoder.

Observation 2: Two code block sizes are used to reduce padding overhead, which improves transmission efficiency.

# 3 	Considerations for code segmentation in 5G NR

In Ran1 #87 meeting, LDPC was agreed to serve as channel coding scheme for eMBB data channel because of the good performance and high realization efficiency. Figure 1 shows a simple example of the transmitter chain with LDPC codes. First, a TB is segmented equally into multiple CBs. The number of CB equals to the ceil of the quotient between input bit number, and maximum supported input bit number for LDPC encoder. In many occasions, the size of the CB does not match the required information bits for encoding. Therefore, as the second step, padding bits are added at the end of each CB to perform encoding process. Finally, coded bits are ready for the rate matching and transmission. Here, the same CBS is used and information bits are almost equally divided into these CBs. It can serve as a start point for discussion of code segmentation scheme with LDPC codes.

Figure1: Example of procedure of LDPC encoding

Based on the previous scheme, the following aspects need further consideration.

Transmission efficiency and power efficiency

In general, LDPC codes are constructed for fixed dimensions. Quasi-cyclic like LDPC codes is agreed to use for the purpose of study and comparison [3-4]. The parity check matrix (PCM), H, is often defined as (N-K)×N matrix,

H= P1,1P2,1.....PN-K,1P1,2P2,2.....PN-K,2P1,1P2,1.....PN-K,3……..P1,NP2,N.....PN-K,N

where Pi,j is a cyclic-permutation matrix obtained from the zero matrix and the z by z cyclically shifted identity matrix to the right; N*z, K*z and (N-K)*z are the coded block sizes, information block size and the parity size, respectively. The supported information block size is often limited to K*z or some other limited set if the z is allowed to adjust. On account of hardware realization and robust performance for PCM design, just limited set of z can be used [5]. For example, a set of lifts is given by  a1,a2,…, aK×2i, 0≤i≤M. Each subset j∈1:K is described by its leader aj and its powers of 2 until the maximum power M. Thus, altogether there are M+1K lifting values. On account of maximal information bit size 8192, the lifting value will be limited by Zmax. Thus, the number of lifting value will be limited.

In many occasions, zero padding bits are added at the end of each CB to match the information bits for encoding to the CBS. For small information bit number, e.g. Kmin-512, 8 bit granularity may be used for LDPC design. It cannot be a multiple of information bit number in H, e.g. 32, 16. For large information bit number, padding bits are also needed when exact z value is not in the set of  a1,a2,…, aK×2i. Thus, zero padding bits has to be used for LDPC encoding for these two cases. The spectral efficiency and power efficiency will be reduced by too much padding bits. For the larger TBS or packet transmission in 5G systems, the padding overhead will increase on account of large space between larger lifting value z. Moreover, padding bits causes rate loss not only in the information bits part but also in the parity bits part. That could be a critical issue for codes with lower code rates.

During code segmentation, the information bits are divided into CBs. Therefore, multiple CBS, similar as LTE code segmentation mechanism, can be considered for well matching between information bits for LDPC encoder and supported CBS. If the zero padding can be thought of as segment fragment, this fragment can be reduced with multiple CBS, which can be thought of the container with different resolution. On account of realization efficiency, the subset of z with the same leading element can be used for one code segmentation. Let’s take an example for information bit number 9216. According to equal size segmentation scheme, the CBS is 4608 and has to add ZP to 5120 bits on account of limit Z size. Then, 512 zero padding bits are needed for each code block. If multiple CBS scheme is used, it can be divided into two code block with size 8192 and 1024 and no zero padding bits are needed. With this code segmentation of multiple CBS, transmission efficiency and power efficiency can be improved. On the other hand, code segmentation with multiple granularity, i.e. multiple CBS, will increase realization complexity. Therefore, further consideration is needed for the trade-off between transmission efficiency, power efficiency, and realization complexity.

Proposal 1: Transmission efficiency shall be considered for designing code segmentation scheme.

Hardware structure and decoding latency

Latency is one of important KPIs for 5G NR. Decoding latency is one of important parts for total latency. For LDPC, parallel processing helps to avoid increasing buffer requirements when supporting higher throughputs. The pipelined encoding and decoding can be used to improve implement efficiency and reduce the encoding/decoding delay. This is critical when supporting self-contained sub-frame structures where symbol level processing is allowed at both transmitter and receiver sides.

For the code block segmentation scheme shown in Figure 1, the number of CBs, denoted as C, is determined by the input bit number B and the maximum supported info bit number for one CB, i.e.. This type of segmentation principle does not have any relation to the hardware structure of the encoder and decoder. Some pipelines will be wasted when the number of CB is not multiple of pipeline number. Therefore, hardware structure for the transmission can be considered in code block segmentation to reduce decoding delay as much as possible.

Proposal 2: Hardware structure shall be considered in code block segmentation to reduce decoding latency.

Maximum code block size of eMBB

For the code segmentation shown in Figure 1, the number of CBs is determined by the TBS and the maximum CBS. Thus, the code segmentation scheme is closely related to the maximum CBS. For Turbo code, 6144 is specified as the maximum CBS. For LDPC, the following agreement was made on maximum block size during RAN1 88 meeting [6].

Agreement:

- The largest info block size supported by LDPC encoder Kmax and the largest shift size Zmax defined for a H matrix are selected from the following set of {Kmax, Zmax} pairs:

  - {8192, 256}, {8192, 512}, {FFS near 8192, 320}

It is highly likely that we finalize 8192 as the NR eMBB maximum CBS. However, we should carefully investigate on segmentation principle according to the following aspects.

- UE specific maximum CBS, when supporting lower code rates

With 8192 or near maximum CBS, the complexity of encoding and decoding of LDPC can be larger for lower code rates. Sometimes UE categories may operate only with lower code rates, where limiting maximum CBS to some other value may be desired.

- Support URLLC puncturing and symbol-wise processing,

It's good to have single or multiple CBs within a symbol duration of the allocated bandwidth. Possible info block sizes can be determined to support such feature considering bandwidth, code rate and modulation scheme supported in eMBB.

Proposal 3: Maximum code block size can be selected as the 8192 for NR eMBB data channel. However, depending on the use case, code segmentation may assume much lower operating maximum CBS than 8192.

# 4 	Conclusions

Based on above discussion. We also make the following proposals and observations:

Observation 1: Padding technique can be used to align transmit information block to the supported block sizes at the encoder.

Observation2: Two code block sizes are used to reduce padding overhead, which improves transmission efficiency.

Proposal 1: Transmission efficiency shall be considered for designing code segmentation scheme.

Proposal 2: Hardware structure shall be considered in code block segmentation to reduce decoding latency.

Proposal 3: Maximum code block size can be selected as the 8192 for NR eMBB data channel. However, depending on the use case, code segmentation may assume much lower operating maximum CBS than 8192.

# References

- 3GPP RAN1 #86 Chairman Notes

- TS 36.212-d20, Section 5.1.2

- 3GPP RAN1 #85 Chairman Notes

- R1-165643, “WF on LDPC schemes for NR”, ZTE, ZTE Microelectronics, Nokia, Qualcomm, InterDigital, MediaTek, Intel, Samsung, Sharp

- R1-1610137, “LDPC rate compatible design overview”, Qualcomm Incorporated

- 3GPP RAN1 #88 Chairman Notes