3GPP TSG-RAN WG1 #89s	R1-1706895

Hangzhou, China, 15th – 19th May, 2017

Source:	Ericsson

Title:	Early data transmission for NB-IoT

Agenda Item:	6.2.7.6

Document for:	Discussion and Decision

# Introduction

At RAN#75 it was agreed to start the Release 15 work item on Further NB-IoT enhancements. In this paper we address the objective on Support for early data transmission [1]:

- Evaluate power consumption/latency gain and specify necessary support for DL/UL data transmission on a dedicated resource during the Random Access procedure after NPRACH transmission and before the RRC connection setup is completed. [RAN2, RAN1, RAN3]

The scope of this paper is twofold:

- In section 2 we present reference performance in terms of uplink service latency and power consumption for a NB-IoT device. This may serve as a starting point for the evaluations of the gains of early data transmission.

- In section 3 we present methods for transmitting data in Message 3 and 4.

# NB-IoT reference performance

## Latency

Part of the objective on supporting for early data transmission is to evaluate latency gain. To evaluate the gains for user plane data transmission the latency achieved using the RRC Resume procedure may serve as a relevant base line. This does not preclude other baselines e.g. based on the regular connection setup procedure.

Figure 1 depicts the NB-IoT RRC Resume procedure for transmitting a mobile originating (MO) report including the following basic steps:

- Synchronizing to the NPSS/NSSS after waking up from (e)DRX or PSM to achieve time and frequency synchronization, and to acquire subframe timing and physical cell identity.

- Setting up a connection, including:

  - Reading MIB to acquire the System Frame Number and SIB1-NB scheduling information.

  - Reading SIB1-NB to acquire the system information value tag and access barring status.

  - Performing the system access procedure.

  - Configuring radio bearers.

- Transmitting the uplink report.

Figure 1: Data and signalling flow used to model RRC Resume latency performance.

To support latency calculations based on the time from a MO event is triggered until the data is delivered as illustrated in the figure above. In Annex A and B we present detailed assumptions on:

- Application level protocol overheads (Table 7).

- The core network protocol overheads and RRC message sizes defining the required signaling radio bearer (SRB) transport block sizes used in Figure 1 (Table 4).

- Radio related assumptions used when deriving the link level performance at 144, 154 and 164 dB coupling loss (CL) (Table 5).

- Link level performance in terms of transmission time intervals (TTI) at 144, 154 and 164 dB (Table 6).

Further details about the background to these evaluation assumptions are given in [3].

The latencies based on the assumptions to deliver an 85-byte packet (including 65 bytes overhead, see Table 7) is presented in Table 1. For a coupling loss of 144 dB it can be noted that ~50% of the reported latency is due to the initial phase of the connection setup where NPSS/NSSS and NPBCH acquisition is performed and the device is assumed to wait 40 ms for a NPRACH opportunity.

Table 1: Latency reference performance.

| Coupling loss [dB] | Standalone | Guardband | Inband |
| --- | --- | --- | --- |
| 144 | 0.3 | 0.3 | 0.3 |
| 154 | 0.7 | 0.9 | 1.1 |
| 164 | 5.1 | 8.0 | 8.3 |

The performance presented in Table 1 may serve as the base line performance for evaluating the latency gains for early transmission of data.

It should be noticed that already in Release 13 the data can be transmitted multiplexed with the RRC Connection Resume Complete message in the MAC layer and not after the RRC Connection Resume Complete message as assumed here. So a more optimized reference performance should be possible to achieve and is for further study.

Also for mobile terminated data transfer a reference model should be developed to show the gains of early data transmission in the downlink. In addition to adding a downlink data transfer to the message flow in Figure 1, the time to address a device with a page should be considered.

## Battery life

In addition to latency, the gain in power consumption, or battery life, from the introduction of an early data transmission feature is to be quantified. As for latency the performance achieved for the RRC Resume procedure may serve as baseline for evaluating the achievable gains, but in terms of battery life.

Figure 2 depicts the procedure used to evaluate the battery life. It includes the modelling of four levels of power consumption; TX, RX, light sleep, and deep sleep. One second waiting time between uplink and downlink transmission is included in the model as well as a 20-second Active timer after the connection has been released. Further details on the background to these evaluation assumptions are given in [2].

Figure 2: Data and signalling flow used to model RRC Resume battery performance.

Based on the assumed traffic model (Table 8, Table 9) and assumed power consumption levels (Table 10) presented in Annex C, the battery life presented in Table 2 and Table 3 are achieved. These may serve as base line performance for evaluating the battery life gains for early transmission of data.

The performance presented in Table 2 and Table 3 may serve as base line performance for evaluating the battery life gains for early transmission of data.

Table 2: Battery life according to Release 13 scenarios according to TR 45.820 [5].

| Reporting interval | DL Packet size | UL Packet size | Battery life [Years] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 144 dB CL |  |  | 154 dB CL |  |  | 164 dB MCL |  |  |
|  |  |  | S | G | I | S | G | I | S | G | I |
| 2 hours | 65 bytes | 50 bytes | 22.2 | 22.1 | 22.1 | 13 | 12.6 | 12.3 | 3.0 | 2.7 | 2.6 |
|  |  | 200 bytes | 20 | 20.0 | 20 | 7.9 | 7.8 | 7.7 | 1.4 | 1.3 | 1.3 |
| 24 hours |  | 50 bytes | 36.2 | 36.1 | 36.1 | 33 | 32.8 | 32.6 | 19.3 | 18.4 | 18.0 |
|  |  | 200 bytes | 35.6 | 35.6 | 35.6 | 29 | 28.9 | 28.7 | 11.8 | 11.5 | 11.3 |

Table 3: Battery life according to Release 14 scenario according to TR 38.913 [6].

| Reporting interval | DL Packet size | UL Packet size | Battery life [Years] |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 144 dB MCL |  |  | 154 dB MCL |  |  | 164 dB MCL |  |  |
|  |  |  | S | G | I | S | G | I | S | G | I |
| 24 hours | 20 bytes | 200 bytes | 35.6 | 35.6 | 35.6 | 29 | 28.9 | 28.8 | 11.8 | 11.6 | 11.4 |

It should be noted that the power consumption levels used here was derived for NB-IoT during the feasibility study in Release 13. Now when devices are getting ready for the market we would welcome feedback if these levels are still considered applicable. The transmitted power consumption is e.g. based on 45% PA efficiency. Figure 3 in Annex C shows the dependency between battery life and PA efficiency, and indicates the importance of making correct assumptions.

The power levels for evaluating NB-IoT battery life are reused from the Release 13 NB-IoT feasibility study. The reliability of the estimated battery life is highly dependent on the accuracy of this assumption.

# Early data: Data transmission in Msg3 and Msg 4

## Data in Msg3

A promising solution for early data transmission is to append uplink data to Msg3.

In terms of the RRC Resume solution this means that uplink data can be transmitted together with the RRC Connection Resume Request in Msg3 instead of together with the RRC Connection Resume Complete in Msg5. This would reduce the latency by the time it takes to transmit the NPDCCH, NPDSCH, NPUSCH F1 and NPUSCH F2 associated with the RRC Connection Resume (Msg4) and the RRC Connection Resume Complete (Msg5) messages. The transmission times indicated in Table 6 for these physical channels suggest that the absolute gains are most significant at the MCL of 164 dB..

To facilitate flexible use of this feature, RAN1 should review the suitability of the uplink grant in the Random Access Response to support data transmission in the RRC Connection Resume Request. Currently the NB-IoT Random Access Response grant supports a fixed transfer block size (TBS) of 88 bits, which likely needs to be extended to make data in Msg3 an attractive feature.

RAN1 should review the suitability of the uplink grant in the Random Access Response to support data transmission in Msg3

- To make efficient use of an early data transmission feature, i.e. the assignment of a Msg3 TBS larger than 88 bits the device may use the NPRACH for indicating its capability and interest of early data transmission. A reserved range of NPRACH preambles dedicated to indicate support for early data transmission may be used to support this early indication. The drawback of this option is that partitioning of the NPRACH will lead to loss in NPRACH capacity due to so called trunking loss.

Early indication of the support for data transmission in Msg3 may be supported by a reserved range on NPRACH preambles.

- An alternative to NPRACH partitioning is to transmit a dual random access response grant to devices setting up an connection. The first grant follows the Release 13 88 bit format while the second grant allows for the transmission of a larger TBS including data. The device autonomously determines which of the two grants to use. A device not being capable of the feature only decodes the Release 13 grant and transmits the 88 bit Msg3. A device capable of early data transmission may select to use the second grant supporting a larger TBS for Msg3 to also include UL data. The advantage of this option compared to NPRACH partitioning is that the NPRACH trunking loss is avoided. The potential drawback is that the eNB needs to blindly detect if Msg3 is transmitted in accordance to the first or second grant.

As an alternative to the NPRACH early indication the eNB may transmit dual grants, the first for devices not capable of Msg3 data transmission and the second that is large enough to support Msg3 data.

## Data in Msg4

Downlink data can be appended to the RRC Connection Resume message (Msg4), instead of transmitted after the RRC Connection Resume Complete (Msg5). This would reduce the latency by the time it takes to transmit the NPUSCH, NPDCCH and NPDSCH related to the RRC Connection Resume Complete. The most promising gains are foreseen achievable at the MCL of 164 dB..

The RRC Connection Resume message is scheduled using DCI format N1 scrambled by the temporary C-RNTI. The available set of TBSs are the same as when scheduling the NPDSCH on a data radio bearer and should be sufficient to cater for early data in Msg4. RAN1 can hence assume that RAN2 will drive the work on early data in Msg4.

The supported set of Msg4 transfer block sizes are sufficient to cater for early data.

# Conclusions

In this paper we have presented latency and battery life performance for NB-IoT with the RRC Resume procedure. This may serve as base line performance for evaluating the gains obtained by early transmission of data. We have also presented a method for transmitting data in Msg3 and Msg4 to further improve the presented performances. We have also made the following set of observations:

- The performance presented in Table 1 may serve as the base line performance for evaluating the latency gains for early transmission of data.

- The performance presented in Table 2 and Table 3 may serve as base line performance for evaluating the battery life gains for early transmission of data.

- The power levels for evaluating NB-IoT battery life are reused from the Release 13 NB-IoT feasibility study. The reliability of the estimated battery life is highly dependent on the accuracy of this assumption.

- RAN1 should review the suitability of the uplink grant in the Random Access Response to support data transmission in Msg3

- Early indication of the support for data transmission in Msg3 may be supported by a reserved range on NPRACH preambles.

- As an alternative to the NPRACH early indication the eNB may transmit dual grants, the first for devices not capable of Msg3 data transmission and the second that is large enough to support Msg3 data.

- The supported set of Msg4 transfer block sizes are sufficient to cater for early data.

# References

RP-170836, “Further NB-IoT enhancements”, RAN#75, source Huawei

R1-1703112, “On mMTC, NB-IoT and eMTC battery life evaluation”, source Ericsson, RAN1#88

R1-1703113, “On mMTC, NB-IoT and eMTC latency evaluation”, source Ericsson, RAN1#88

RP-170511, “On eMTC 5G mMTC requirement fulfillment”, source Ericsson, RAN#75

3GPP TR 45.820

3GPP TR 38.913

# Annex A: General assumptions for early data evaluations

Table 4 presents the higher layer assumptions used for evaluation of latency and battery life performance.

Table 4: Assumptions on core network protocol overheads and RRC message sizes.

| Parameter | Assumption |
| --- | --- |
| MIB | 34 bits |
| Higher layer procedure | RRC Resume |
| PDCP | 5 bytes |
| RLC | 2 bytes |
| MAC | 2 bytes |
| Random Access Response | 7 bytes |
| RRC Connection Resume Request PDU | 9 bytes |
| RRC Connection Resume PDU | 2 bytes |
| RRC Connection Resume Complete PDU | 2 bytes |
| RRC Connection Release PDU | 7 bytes |

Table 5 and Table 6 presents the radio related assumptions used for evaluation of latency and battery life performance.

Table 5: Radio related assumptions.

| Parameter | Assumption |
| --- | --- |
| Propagation condition | ETU |
| Fading | Rayleigh, 1 Hz |
| Mobile NF | 5 dB |
| Base station NF | 3 dB |
| Device power class | 23 dBm |
| LTE system BW (inband case) | 10 MHz |
| Base station power class | Inband, guardband: 46 dBm, Standalone: 43 dBm |
| Power boosting (inband case) | 6 dB on anchor |
| Coupling loss | 144, 154, 164 dB |
| Targeted link level performance | PSS/SSS, PBCH: 90th percentile acquisition time PDSCH, PUSCH F1: 10% BLER NPDCCH, PUSCH F2, PRACH: 1% BLER |
| Link level scenario | Sensitivity limited |

Table 6: Assumed transmission times in ms at 144, 154 and 164 dB coupling losses.

| Inband |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coupling loss | NPDSCH 10% BLER 680 bit TBS | NPUSCH 10% BLER 1000 bit TBS | NPDCCH 1% BLER | NPUSCH F2 1% BLER | NPRACH 1% BLER | NPSS/NSSS 90th percentile | NB-MIB 90th percentile |
| 144 | 10 | 32 | 4 | 2 | 14 | 84 | 10 |
| 154 | 128 | 320 | 32 | 4 | 52 | 124 | 60 |
| 164 | 1024 | 2560 | 256 | 64 | 205 | 1284 | 640 |
| Guardband |  |  |  |  |  |  |  |
| Coupling loss | NPDSCH 10% BLER 680 bit TBS | NPUSCH 10% BLER 1000 bit TBS | NPDCCH 1% BLER | NPUSCH F2 1% BLER | NPRACH 1% BLER | NPSS/NSSS 90th percentile | NB-MIB 90th percentile |
| 144 | 8 | 32 | 2 | 2 | 14 | 84 | 10 |
| 154 | 80 | 320 | 16 | 4 | 52 | 124 | 60 |
| 164 | 760 | 2560 | 256 | 64 | 205 | 1014 | 640 |
| Standalone |  |  |  |  |  |  |  |
| Coupling loss | NPDSCH 10% BLER 680 bit TBS | NPUSCH 10% BLER 1000 bit TBS | NPDCCH 1% BLER | NPUSCH F2 1% BLER | NPRACH 1% BLER | NPSS/NSSS 90th percentile | NB-MIB 90th percentile |
| 144 | 4 | 32 | 2 | 2 | 14 | 84 | 10 |
| 154 | 24 | 320 | 8 | 4 | 52 | 104 | 20 |
| 164 | 192 | 2560 | 128 | 64 | 205 | 264 | 170 |

# Annex B: Assumptions for latency evaluations

Table 7 presents the application layer assumptions used for evaluation of latency performance.

Table 7: Packet model for latency evaluations.

| Protocol layer | Overhead [bytes] |
| --- | --- |
| Application data | 20 bytes |
| COAP | 4 bytes |
| DTLS | 13 bytes |
| UDP | 8 bytes |
| IP | 40 bytes |
| Total | 85 bytes |

# Annex C: Assumptions for battery life evaluations

Table 8 and Table 9 presents the application layer assumptions used for evaluation of battery life performance.

Table 8: Release 13 battery life evaluation scenarios from TR 45.820.

| Message type | UL report |  | DL Application Acknowledgment |
| --- | --- | --- | --- |
| Size | 200 bytes | 50 bytes | 65 bytes |
| Arrival rate | Once every 2 h or once every 24 h |  |  |

Table 9: Release 14 battery life evaluation scenarios from TR 38.913.

| Message type | UL report | DL Application Acknowledgment |
| --- | --- | --- |
| Size | 200 bytes | 20 bytes |
| Arrival rate | Once every 24 h |  |

Table 10 presents the power consumption assumptions used for evaluation of battery life performance taken from the Rel-13 NB-IoT evaluations. Figure 3 shows the battery life for a 23 dBm device for a range of PA efficiencies. The evaluation was performed for the 24 hour report triggering interval, and 200 bytes uplink followed by 65 bytes downlink at 164 dB coupling loss.

Table 10: Release 13 power consumption levels.

| Device power consumption |  |
| --- | --- |
| TX (23 dBm, integrated PA: 45% eff.) | 500 mW (incl. 60 mW support circuity) |
| RX | 80 mW |
| Idle – Light sleep | 3 mW |
| Idle – Deep sleep | 0.015 mW |

Figure 3: Battery life as function of PA efficiency for 23 dBm device with 60 mW power consumption from support circuits.