3GPP TSG RAN WG1 Meeting #89	R1-1707015

Hangzhou, China, 15-19 May, 2017

Agenda Item:	6.2.8.1

Source:	Huawei, HiSilicon

Title:	             Consideration on channel model for LTE Aerial

Document for:	Discussion and decision

Introduction

At RAN#75 meeting, it was agreed to start a SI on Enhanced Support for Aerial Vehicles [1]. Channel modeling is one of the main objectives, and it is aimed to select appropriate models applicable to Air-to-ground (ATG) channels. Reusing an existing channel model, if applicable, should be prioritized.

In the chairman’s notes of RAN1#88bis, it’s agreed that four options are considered for channel modeling of aerial UEs in three deployment scenarios should be considered, which is UMi, UMa and RMa, and the aerial UT height is between 1.5m and 150m.

Agreement

- For channel modelling of Aerial UEs, at least the following can be different from the terrestrial UEs

  - Pathloss, shadowing, LOS probability and fast fading

Conclusion:

- Companies are encouraged to provide measurement/simulation/evaluation results for deciding the channel modelling of Aerial UEs in RAN1#89 meeting by considering the options below.

In this contribution, we discuss general aspects on ATG channels and scenarios, and propose field measurements and simulation results for deciding channel modeling of pathloss and fast fading.

Scenarios

During RAN1#88bis meeting in Spokane, the following scenarios were agreed [3], and channel model related items are highlighted as yellow.

Table 1. Channel model related items in AV Scenarios.

| Parameters |  | UMi AV | UMa AV | RMa AV |
| --- | --- | --- | --- | --- |
| BS antenna height |  | 10m | 25m | 35m |
| Carrier frequency |  | 2GHz | 2 GHz | 700 MHz; optionally 800 MHz |
| UT location | LOS/NLOS (aerial) | FFS | FFS | FFS |
|  | Height hUT (aerial) | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m |

The key question is how the flying device will affect the channel model, and how much the existing channel models can be reused. We discuss it briefly here.

Urban macro-cell

Traditional urban macro-cell is defined as a base station above rooftop and UE below rooftop (Figure 1). In the case of aerial vehicles, the UE may be either below rooftop (Figure 2) or above rooftop (Figure 3). This coarse breakdown is obviously a simplification since in real-world the building height may be inhomogeneous. However, it follows the simplifications done in 3GPP many times earlier (e.g. macro- vs. micro-cell, LOS vs. NLOS). Therefore, we think this breakdown is sufficient for categorization of channel models. If the UE is below rooftop (Figure 2), we can use the existing 3D-UMa channel model from TR36.873. If the UE is above rooftop (Figure 3), we can assume LOS channel with a few ground reflections and rooftop reflections.

Figure 1. Traditional urban macro-cell.

Figure 2. Urban macro-cell for aerial vehicle, UE below rooftop.

Figure 3. Urban macro-cell for aerial vehicle, UE above rooftop.

Urban micro-cell

Traditional urban micro-cell is depicted in Figure 4. The upper part is a side view, and the lower part a top view. In the NLOS case, the UE is behind an obstacle (e.g. building), and the main propagation mechanisms are reflection, scattering and diffraction. In the case of an aerial device, the user can be either below rooftop (Figure 5) or above rooftop (Figure 6). The latter case is quite similar to the traditional macro-cell (Figure 1), but the BTS and UE are swapped. It gives us an opportunity to reuse the existing 3D-UMa model.

For LOS probability calculation in the case of aerial user being above rooftop, it is also similar to the traditional macro-cell case with BTS and UE swapped. In this case, the “BS in UMi AV scenario” can be viewed as “UT in 3D UMa channel model”, and “Aerial UT in UMi AV scenario” can be viewed as “BTS in 3D UMa channel model”. So LOS probability can be calculated by employing 3D UMa LOS probability formula by setting “hUT” equal to 10m, which is BS height in UMi AV scenario (see Table 1). It is noted that in this case, 3D UMa LOS probability equals to 3D UMi LOS probability. Therefore 3D UMi LOS probability could be re-used.

Figure 4. Traditional urban micro-cell.

Figure 5. Urban micro-cell for aerial vehicle, UE below rooftop.

Figure 6. Urban micro-cell for aerial vehicle, UE above rooftop.

Rural macro-cell

Traditional rural macro-cell scenario is depicted in Figure 7. For aerial device below rooftop, we can use the same model as with the terrestrial UE since the average rooftop is low, and there is no big difference between these two situations. For the case where UE is above rooftop, the channel is LOS, see Figure 8 for UE below the BTS height and Figure 9 for the UE above the BTS height.

Figure 7. Traditional rural macro-cell.

Figure 8. Rural macro-cell for aerial vehicle, UE below BTS, but above rooftop.

Figure 9. Rural macro-cell for aerial vehicle, UE above BTS.

Summary on scenarios and sub-scenarios

Table 2 summarizes the propagation scenarios for aerial devices. Based on the observation of the propagation characteristics in each of the scenario, it is seen that some of the existing models can be re-used (at least as starting point) for AV UTs below rooftop, and these models can be extrapolated to cover higher altitudes, although the existing models have some limitations on the UE height range. The proposed use of channel model is shown in Table 2.

Table 2. Proposed model for Propagation scenarios for Aerial device

| Scenario | Sub-scenario | LOS/NLOS | Proposed models (Pathloss, Shadowing, LOS probability, fast fading) |
| --- | --- | --- | --- |
| UMa AV | UE below rooftop (hAerial_UT<25m) | LOS probability | 3D-UMa, extend applicability of hUT to 25m |
|  |  | Pathloss & shadowing | LOS: 3D-UMa LOS NLOS: 3D-UMa NLOS |
|  |  | Fast fading | LOS: 3D-UMa NLOS NLOS: 3D-UMa NLOS |
|  | UE above rooftop (hAerial_UT>25m) | LOS probability | PLOS=1 |
|  |  | Pathloss & shadowing | LOS: See Table 3 below |
|  |  | Fast fading | To be updated |
| UMi AV | UE below rooftop (hAerial_UT<25m) | LOS probability | 3D-UMi |
|  |  | Pathloss & shadowing | LOS: 3D-UMi LOS NLOS: 3D-UMi NLOS |
|  |  | Fast fading | LOS: 3D-UMi LOS NLOS: 3D-UMi NLOS |
|  | UE above rooftop (hAerial_UT>25m) | LOS probability | 3D-UMi |
|  |  | Pathloss & shadowing | LOS: 3D-UMa LOS swapped (i.e., BS in UMi AV scenario =”UE” in 3D-UMa, AV UE in UMi AV scenario=”BS” in 3D-UMa), check fast fading. NLOS: 3D-UMa NLOS swapped (i.e., BS in UMi AV scenario =”UE” in 3D-UMa, AV UE in UMi AV scenario =”BS” in 3D-UMa) |
|  |  | Fast fading | LOS: 3D UMi LOS NLOS: 3D UMi NLOS |
| RMa AV | UE below rooftop (hAerial_UT<10m) | LOS probability | 3D RMa |
|  |  | Pathloss & shadowing | LOS: 3D-RMa LOS NLOS: 3D-RMa NLOS |
|  |  | Fast fading | LOS: 3D-RMa LOS NLOS: 3D-RMa NLOS |
|  | UE above rooftop(hAerial_UT>10m) | LOS probability | PLOS=1 |
|  |  | Pathloss & shadowing | To be updated. |
|  |  | Fast fading | To be updated. |

Proposed model for UMa AV above rooftop

Pathloss

Table 3 summarizes the proposed path loss and shadow fading models for urban macro-cell.

Table 3. BS-to-Aerial channel – Pathloss

| Applicability range, antenna height default values | LOS/ NLOS | Pathloss [dB], fc is in GHz and d is in meters (6) | Shadow fading std [dB] |
| --- | --- | --- | --- |
| hAerial_UT>25 m | LOS |  |  |

Measurement of path loss

For the UMa scenario, we have measured LTE aerial channels in typical UMa LOS cases when the AV UT height is above rooftop.

For UMa LOS case, we choose street canyon environments, and part of them are depicted as shown in the figures below. In the figure, the base station is located on the rooftop of a 5-story office building, and the height of the antenna was about 25m above the ground.

The altitudes of the user equipment included 50 m, 75 m, 100 m, 125 m, and 150 m. At each altitude, the UE traveled horizontally over the distances of 420m and 320m along the street. The flying routes are shown in the figures. The LOS channels between the transceivers were measured.

UMa LOS case

In the UMa scenario measurement, the transmitter on the UAV was a WiFi AP working at the frequency of 2.4 GHz. An amplifier was employed such that the transmission power was 30 dBm. At the BS side, the PCTEL fast frequency scanning and protocol analysis receiver was adopted to collect the data. It could identify the WiFi channel, extract the target signal, and estimate the received power. Thus the interference in the frequency band was avoided. The receiver recorded the received power () every 500ms while the UAV was flying in the air. Two omni-directional antennas () were used on the UAV and one omni-directional antenna () on the BS, with the calibrated gains of 6 dBi at both sides.

The pathloss can be computed by the function:

where  is the pathloss, dBm, dBi, dBi, and  is the receive signal power.

We compute the shadowing by the function:

where  is the path loss measurement and  is the path loss predicted by the model, and  is the number of the path loss measurement samples.

The PCTEL receiver instrument is depicted in Figure 10. The four-rotor UAV with the WiFi AP and two omnidirectional antennas on board are depicted in Figure 11. An illustration of the entire measurement system is depicted in Figure 12.

Figure 10.  The BS in the field measurement

Figure 11. Aerial UT

Figure 12. UMa channel measurement system

Results of path loss in UMa scenario

It is found that when aerial UE fly above 25 m, the channel between the aerial UE and terrestrial BS is LOS dominated, so we propose 25 m as a threshold in vertical height.  When aerial UE fly below 25 m, LOS channel and NLOS channel both exist.

Note that in our field measurements, the horizontal distance is limited to several hundred meters and located within break point distance, so the modified model is based on PL1 defined in TR38.901/36.873, and we extrapolate the UE height dependent adjustment parameter for PL2.

The path loss model for the LOS UMa scenario defined in 3GPP TR38.901/36.873 is (before break point)

In this measurement campaign, the measurement results of the path loss have the same tendency as the 3GPP model, for all the measured UE horizontal distances and vertical altitudes. Therefore, we propose to revise the 3GPP model by adding a UE height dependent adjustment parameter. The modified PL model is:

By fitting the measurement results and calculating the least sum of squared residuals, we get the value of adjustment parameter. The results are shown in Figure 13 below.

For UMa LOS with aerial UE’s height above 25m, it is observed that

.

Figure 13. Adjustment depending on AV UE height

The shadowing standard derivation is derived from

Figure 14. Shadowing of UMa for AV UE height > 25m

Conclusion

In this contribution, we propose channel models for aerial UEs based on field measurements. The proposal includes pathloss, shadowing, LOS probability, and fast fading for different scenarios, and we make the following proposal.

Proposal: Use Table 2 and Table 3 in the channel model for communication with aerial devices.

- References

RP-170779, “SID on Enhanced LTE Support for Aerial Vehicles”, NTT DOCOMO INC, Ericsson, 3GPP TSG RAN Meeting #75, Dubrovnik, March 2017.

R1-1704285, “Considerations on evaluation scenarios and channel models for drones”, Huawei, HiSilicon, 3GPP TSG-RAN WG1 Meeting #88bis, Spokane, April 2017.

R1-1706794, “Chairman’s notes of AI 7.2.8 Enhanced Support for Aerial Vehicles”, Ad-Hoc chair (Ericsson), 3GPP TSG RAN1 meeting #88bis, Spokane, April 2017.