3GPP TSG RAN WG1 Meeting RAN1 #89													R1-1707345

Hangzhou, P.R. China 15th – 19th May 2017

Source: 	Intel Corporation

Title:	CSI-RS design for L3 Mobility

Agenda item:	7.1.1.5.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss CSI-RS configuration for L3 mobility based on agreements made from previous meetings. The following agreement regarding mobility were made in RAN1 #88bis.

Agreements: - For CONNECTED mode RRM measurement for L3 mobility based on CSI-RS, NR supports following targets for CSI-RS design and configuration: - Design: - Reuse CSI-RS design for beam management as baseline - Identify additional requirements on the CSI-RS to support L3 mobility - Configuration: - Support measurement of a large number of beams - Minimize configuration overhead, reporting overhead, and UE complexity - FFS: Detailed signaling or format Agreements: - For Connected mode, CSI-RS are supported to be configured using at least dedicated RRC signaling for DL based RRM measurement for L3 mobility. - Note that signalling other than dedicated RRC signalling is not precluded Agreements: - The time synchronization reference for a CSI-RS for L3 mobility is the frame/slot/symbol timing of a cell. - Note: The frame/slot/symbol timing of the cell can be obtained from an SS block - FFS: Note: timing synchronization between CSI-RS and SS block of the cell is assured. Timing synchronization refers to frame/slot/symbol timing. - NR cell ID for time reference of CSI-RS(s) is informed to the UE

# CSI-RS for L3 Mobility Measurements

The main motivation behind support of CSI-RS for CONNECTED mode RRM measurement for L3 mobility was that CSI-RS configured for P-1 beam management can re-utilized for L3 mobility purposes. The P-1 beam management is used to enable UE measurements on different TRP Tx beams to support the selection of TRP Tx beam and UE Rx beams. Basically, the P-1 beam management is set of procedures utilizing link quality measurement from set of reference signals and determining the best links. The basic structure should be able to provide necessary measurements for L3 mobility.

The only difference between P-1 beam management and L3 mobility is that the UE is required to obtain the timing (i.e. slot and radio frame boundary) of CSI-RS that it needs to measure. In RAN1 #88, it was agreed that the detection of neighbor cells for L3 measurement is based on NR-SS, and in RAN1 #88bis, it was agreed that NR cell ID for time reference of CSI-RS(s) is informed to the UE. However, it was left FFS whether timing synchronization between CSI-RS and SS block of the cell is assured, where the timing synchronization refers to frame/slot/symbol timing.

In this section, we provide 2 different deployments cases and discussed the timing requirements for CSI-RS such that UE can perform useful measurements.

Case 1) Fully synchronous network (all cells and TRPs are synchronized, including SFN synchronization)

Figure 1. Illustration of CSI-RS measurement timing for fully synchronous network

In a fully synchronous network, where all TRPs from all cells are synchronized, the UE only requires to know the frame boundary timing of the serving cell for any CSI-RS measurements for L3 mobility. Here we assume synchronization refers to synchronous to the receiver within a fraction of the CP. For synchronous deployments with extremely large cells, where the propagation delay can be larger and in the orders of CP lengths, we classify them as asynchronous for the purpose of this discussion.

In such scenario, the network does not necessarily need to associate different cell ID to different CSI-RS configurations. All CSI-RS configuration could potentially be associated with the serving cell ID. Of course, there may be situations where the network wishes to group or classify CSI-RSs such that it can receive RSRP reporting from each of the CSI-RS groups. It should be noted that if the network wishes to receive RSRP reporting for different TRPs within a cell, this CSI-RS grouping functionality may be needed even for the serving cell to enable RSRP reporting per TRP.

Case 2) Asynchronous network (all cells are not synchronized, TRPs within a cell are synchronized)

Figure 2. Illustration of CSI-RS measurement timing for asynchronous network

In an asynchronous network, the UE requires to know the frame boundary timing of the cell that is transmitting CSI-RS for CSI-RS RSRP measurements. In such scenario, the network need to provide associated cell ID to each CSI-RS configurations. Similar to the synchronous network case, the network may wish to further group CSI-RS from a cell. Such functionality should be provided for L3 mobility measurements.

Proposal 1:

- Mechanism to group of CSI-RS for L3 mobility measurement for a cell should be supported.

- UE will be expected to report at least one CSI-RS RSRP for one CSI-RS within each group. FFS, how the RSRP triggering events will be applied to grouped CSI-RS configuration.

One of key aspect for multi-TRP cells is that the frame boundary timing from any one of the SS blocks should provide accurate frame boundary timing for any of the CSI-RS that is being transmitted from that cell. For example, if two TRPs from the same cell have some transmission delay, such that SS blocks being sent from different TRPs result in different frame timing, just providing an associated cell ID to the CSI-RS does not provide all the information necessary for the UE to perform CSI-RS measurements. Therefore, any of the SS blocks from a cell must provide the exact the same frame boundary timing, and the CSI-RS that is being transmitted from any one of TRPs should be bounded to the frame boundary timing.

Proposal 2:

- Remove the FFS in the agreement note and agree that

  - Timing synchronization between CSI-RS and SS block of the cell is assured

# Conclusions

In this contribution, we discussed CSI-RS configuration for L3 mobility. Our proposals are summarized as below:

Proposal 1:

- Mechanism to group of CSI-RS for L2 mobility measurement for a cell should be supported.

- UE will be expected to report at least one CSI-RS RSRP for one CSI-RS within each group. FFS, how the RSRP triggering events will be applied to grouped CSI-RS configuration.

Proposal 2:

- Remove the FFS in the agreement note and agree that

  - Timing synchronization between CSI-RS and SS block of the cell is assured