3GPP TSG-RAN WG1 #89                                                R1-1707366

Hangzhou, China, 15th – 19th May, 2017

Source: 	Intel Corporation

Title:	On PT-RS for CP-OFDM

Agenda item:	   7.1.2.4.3

Document for:	Discussion and Decision

# Introduction

In the RAN1 NR Ad-Hoc meeting some agreements on RS for phase tracking for CP-OFDM waveform have been achieved as follows [1]:

- Regarding PT-RS for CP-OFDM, the following is supported

  - For a given UE, the designated PT-RS is confined in scheduled resource as a baseline

    - Whether/how to share DL PT-RS among UEs is FFS

  - Presence of PT-RS in scheduled resource is UE-specifically configured/indicated

  - Multiple PT-RS densities defined in time/frequency domain are supported

  - UE can assume same precoding for a DM-RS port and a PT-RS port

    - Among which ports and mapping rules (fixed and/or configurable, etc) are FFS

  - Number of PT-RS ports can be fewer than number of DM-RS ports in scheduled resource

- Study the following for PT-RS, taking overhead and forward compatibility into account

  - Details on frequency domain patterns/densities

  - How to indicate presence/patterns of PT-RS

    - E.g., implicitly indicated based on association with numerology/MCS/number of allocated PRBs/UE category

    - E.g., explicit indication by L1/L2/L3 signaling

  - Port multiplexing methods

    - E.g., non-orthogonal multiplexing within PT-RS ports and with data

  - Using PT-RS for CFO/Doppler estimation

  - QCL relationship between PT-RS and DM-RS

  - Joint transmission of CSI-RS and PT-RS for improving CSI acquisition accuracy

  - Others are not precluded

In the RAN1 #88 meeting some agreements on RS for phase tracking have been achieved as follows [2]:

- Presence/patterns of PT-RS are configured by a combination of RRC signaling and association with parameter(s) used for other purposes (e.g., MCS) which are (dynamically) indicated by DCI.

  - Whether PT-RS can be present or not depends on RRC configuration.

  - When configured, the dynamic presence is associated with DCI parameter(s) including at least MCS

  - FFS: Time domain density is associated with dynamic configuration by MCS.

  - When present, frequency domain density is associated with at least dynamic configuration of the scheduled BW.

    - FFS: Frequency domain density is associated with dynamic configuration by MCS.

    - FFS: Frequency-domain pattern design supports both frequency-localized and frequency-distributed allocation of PT-RS subcarriers.

  - Other association factors/rules are not precluded.

  - Usage of PT-RS, e.g. CFO/Doppler correction, is not precluded, pattern/signaling for this use case can be different

In RAN1 #88bis meeting, the following agreements are achieved. [3]

- For CP-OFDM, the same PTRS to RE mapping and PTRS densities in time and frequency are available for DL and UL

- Distributed PTRS (non-consecutive subcarriers) in the frequency domain is used as default configuration

  - FFS: Support optional frequency-localized pattern with UE-specific explicit signaling.  (e.g. higher MCS case)

- For single-user case, support orthogonal multiplexing among PTRS ports, if multiple PTRS antenna ports are supported.

  - FFS: how to multiplex multiple PTRS ports, e.g. FDM, TDM, CDM

  - FFS: Whether to support multiple PTRS ports or not (FFS: Max number of PTRS APs).

- Support orthogonal multiplexing between PTRS and data transmitted or received by a single UE.

- For MU-MIMO, non-orthogonal multiplexing of e.g. PTRS/PTRS and PTRS/data is possible but also orthogonal multiplexing to be considered

- FFS: Support multiplexing through multiple scrambling sequences for PTRS port(s)

- Support association between PTRS port and DMRS port group

In this contribution, we provide some discussion on the PT-RS for CP-OFDM waveform including the dynamic presence, time and frequency pattern, precoding of PT-RS as well as some considerations for diversity support for PT-RS.

# Discussion

## 2.1 Dynamic presence of PT-RS

It has been agreed that the dynamic presence of PT-RS relies on at least the MCS. For some modulation orders, such as QPSK, the phase noise impact may not be significant. Hence comparing to the performance degression from phase error, the overhead of PT-RS can be more critical. Hence for low modulation order, it seems the PT-RS should not be present.

However in some cases, the MCS may not be the only factor to be considered. For example, for the MUST operation as shown in Figure 1, the Far-UE will be configured to use QPSK. However the real modulation order is like 16QAM, which may need the PT-RS. Therefore some other indicators as the complementary to MCS to determine the presence of PT-RS should be taken into account.

Figure 1: one example for constellation in MUST

Proposal 1: some other indicators as the complementary to MCS to determine the presence of PT-RS should be taken into account.

## 2.2 Time domain density

In time domain, PT-RS can be mapped continuously (every symbol) or discontinuously (every other symbol). According to the simulation results in [4] – [7], the performance with different time domain densities does not have significant association with SNR or MCS. In most cases, to map the PT-RS in every symbol could achieve good performance. Hence to save the signalling overhead and reduce the implementation complexity, it is better to support to map the PT-RS in every symbol.

Proposal 2: in time domain, the PT-RS should be mapped to every symbol.

## 2.3 Frequency domain density and inter-cell interference suppression

With regard to the frequency domain density of PT-RS, as the CPE is the same in every subcarriers within one OFDM symbol, one PT-RS REs per OFDM symbol theoretically can be sufficient for high SINRs. However, due to noise and ICI impact, more REs allocated to the PT-RS should be considered to improve the CPE estimation accuracy. It has been agreed that the frequency density of PT-RS should be determined at least by scheduling bandwidth.

However the frequency density of PT-RS should also be determined by the SINR. The MCS could be used to reflect the SINR. Hence the MCS should be used to determine the frequency domain density as well as the scheduling bandwidth.

Further, to save the overhead of PT-RS, one possible way is to enable some inter-cell interference suppression scheme. For example, the Zero Power (ZP) PT-RS can be considered to reduce the inter-cell interference. As shown in Figure 2, when one UE in cell 2 has larger bandwidth, in frequency domain, its PT-RS may be enough, then it may use some ZP PT-RS so that the PT-RS of UE in cell 1 could have high SINR so that its frequency density of PT-RS can be reduced. Hence when determining the frequency density, some inter-cell interference suppression scheme, such as ZP PT-RS, should be taken into account.

Figure 2: one example for ZP PT-RS

Further, it has been agreed that the frequency densities of PT-RS should rely on the scheduling bandwidth. For smaller resource allocation case, the density of PT-RS can be relatively high to achieve more accurate channel estimation; for larger resource allocation case, its density can be low to save the overhead. However for transmit diversity case, the beam/precoder cycling can be used. Different beams/precoders can be used in different PRGs. Then the PT-RS within one PRG can use the same beam/precoder and different beams/precoders can be used for PT-RS in different PRGs. Hence for beam/precoder cycling based transmission scheme, the frequency domain density of PT-RS should be also determined by PRG size.

Proposal 3: the frequency domain density should also be determined by the MCS, and the inter-cell interference suppression scheme, such as ZP PT-RS, should also be taken into account.

Proposal 4: for beam/precoder cycling based transmission scheme, the frequency domain density of PT-RS should be determined by PRG size.

## 2.4 Precoder of PT-RS

The phase shift caused by phase noise and CFO could be the same from different antenna elements in one antenna panel. To save PT-RS overhead, 1 antenna port per beam for PT-RS could be enough. In this case the following options for the PT-RS precoding can be considered:

Option 1: The PT-RS shares one AP of DMRS. The AP index for PT-RS could be one of the APs for DMRS.

Option 2: the PT-RS precoder is determined by the DMRS precoder. The precoder of PT-RS can be obtained by averaging the precoder of all layers of DMRS precoder.

Option 3: Independent rank-1 precoding for PT-RS. The PT-RS should be mapped to every symbols including the DMRS symbol.

For option 1, the TRP could select the strongest AP to transmit the PT-RS to get better phase tracking performance. For uplink, it is easy for the TRP to determine which AP is better. For downlink, if only one codeword is used for one beam, some additional signalling is needed to indicate the best AP index for PT-RS. Hence at least for uplink, the precoder of PT-RS should be the same as one AP of DMRS or the PT-RS and DMRS can share one AP, and the AP index of PT-RS can be configured via DCI. For option 2, no additional signalling or operation is needed, but the performance cannot be the best comparing to other options. For option 3, as best rank 1 precoder may not be any column in rank>1 precoder, independent precoder of PT-RS could help to achieve better performance. However the PT-RS should be mapped to DMRS symbol, which could result in additional overhead and may have some impact on the PAPR of DMRS.

Figure 3 illustrates the link level simulation results for the PT-RS in rank=2 transmission. Three different kinds of precoder schemes have been evaluated. For the AP selection scheme, the PT-RS could use the same precoder as the one of the AP of DMRS with strongest SINR. For the fixed AP scheme, the PT-RS AP is fixed to be the same as one of the AP of DMRS. For the random AP scheme, the PT-RS AP index is determined by the slot index. Given the DMRS is using AP 0 and 1, the AP index of PT-RS can be determined by ns mod 2, where ns indicates the slot index. Detail simulation assumption is illustrated in Table A-1 in Appendix. It can be observed that with AP selection, about 2.5 dB performance gain can be achieved.

Figure 3: link level simulation results for rank=2 transmission 2/3 16QAM and CDL-A

Observation 1: compared to fixed AP and random AP selection, about 2.5dB performance gain can be achieved with PT-RS AP selection.

Proposal 5: to save the overhead of PT-RS, one PT-RS antenna port per beam should be supported, and at least one of the following option should be supported for rank>1 transmission:

- Option 1: the precoder of PT-RS and that of one AP of DMRS is the same

- Option 2: the precoder of PT-RS can be derived by averaging the precoder of all DMRS layers

Proposal 6: for DL, if the same precoder can be applied to PT-RS and one AP of DMRS, the feedback of best AP index associated with rank>1 CSI feedback should be considered.

## 2.5 PT-RS for multi-TRP and multi-panel operation

For the multi-TRP and multi-panel operation, a UE could transmit or receive the signal by different panels and the panels could have some physical separations with the help of directional antenna. Due to different propagation delay between the TRP and UE, the receiving timing for different panels can be different. Then it is not easy to create orthogonal PT-RS among different panels. Further to create orthogonal PT-RS could increase the overhead. Hence generally for multi-TRP and multi-panel operation, consider to use non-orthogonal PT-RS.

Proposal 7: For multi-TRP and multi-panel operation, consider to use non-orthogonal PT-RS as a result of physical separation between panels, different receiving timings in different panels and the overhead of PT-RS.

# Conclusion

In this contribution, we provide some discussion on the reference signal for phase tracking, and the following observations and proposals have been achieved.

Observation 1: compared to fixed AP and random AP selection, about 2.5dB performance gain can be achieved with PT-RS AP selection.

Proposal 1: some other indicators as the complementary to MCS to determine the presence of PT-RS should be taken into account.

Proposal 2: in time domain, the PT-RS should be mapped to every symbol.

Proposal 3: the frequency domain density should also be determined by the MCS, and the inter-cell interference suppression scheme, such as ZP PT-RS, should also be taken into account.

Proposal 4: for beam/precoder cycling based transmission scheme, the frequency domain density of PT-RS should be determined by PRG size.

Proposal 5: to save the overhead of PT-RS, one PT-RS antenna port per beam should be supported, and at least one of the following option should be supported for rank>1 transmission:

- Option 1: the precoder of PT-RS and that of one AP of DMRS is the same

- Option 2: the precoder of PT-RS can be derived by averaging the precoder of all DMRS layers

Proposal 6: for DL, if the same precoder can be applied to PT-RS and one AP of DMRS, the feedback of best AP index associated with rank>1 CSI feedback should be considered.

Proposal 7: For multi-TRP and multi-panel operation, consider to use non-orthogonal PT-RS as a result of physical separation between panels, different receiving timings in different panels and the overhead of PT-RS.

# Appendix

Table A-1: Simulation Assumption

| Parameter | Assumption |
| --- | --- |
| Channel Model | CDL-A |
| Carrier frequency | 30GHz |
| Simulation bandwidth | 80MHz |
| Subcarrier spacing | 60kHz |
| Modulation and coding scheme | 2/3 16QAM |
| MIMO configuration | 2x2, rank 2 transmission |
| PT-RS density | 0.5 RE/Symbol/RB |
| DMRS channel estimation | Perfect |
| PT-RS channel estimation | Matched filter |
| MIMO equalizer | MMSE-IRC |
| PT-RS AP selection period | 5 slots |
| Number of symbols for data | 11 |

# References

- 3GPP Chairman's Notes RAN1 NR Ad-Hoc

- 3GPP Chairman’s Notes RAN1 #88

- 3GPP Chairman’s Notes RAN1 #88bis

- R1-1612186, “Phase noise reference signal design for high frequency systems”, CMCC

- R1-1612499, “Frequency domain pattern for RS for phase tracking”, Samsung

- R1-1612624, “Study of Time and Frequency Density of Phase Noise RS”, National Instruments Corp

- R1-1611981,“On Phase Tracking for NR”, Intel

- R1-166562, “Study of Phase Noise Tracking”, Intel

- R1-1700355, “Discussion on Reference Signal for Phase Tracking”, Intel