3GPP TSG RAN WG1 Meeting #89		R1-1707448

Hangzhou, P.R.China 15th - 19th May 2017

Source:	CATT

Title:	Discussion on 64QAM modulation scheme in V2X Phase 2

Agenda Item:	6.2.3.1.2

Document for:	Discussion and Decision

Introduction

In WID of V2X Phase 2 on the RAN#75 meeting [1], the following detailed objective for 64QAM has been specified:

The detailed objectives of this work item are as follows: - Specify solutions for the following PC5 functionalities, which can co-exist in the same resource pools as Rel-14 functionality and use the same scheduling assignment format (which can be decoded by Rel-14 UEs), without causing significant degradation to Rel-14 PC5 operation compared to that of Rel-14 UEs: [RAN1, RAN2, RAN4] - 64QAM;

In this contribution, introduction of 64QAM scheme in V2X Phase2 is discussed and relative simulation results are provided.

Discussion

Higher modulation scheme for V2X Phase 2

In Rel-14 LTE-based V2X, by assuming number of PRB nPRB=96 and ITBS=15, the allowed maximum modulation order is 16QAM, and the maximum payload size is 31706 bits (approx. 3963 bytes).

Based on the new SA requirements on enhanced V2X [2], higher data rate should be supported as one of the key objectives. For example, with communication between eV2X UEs or between UE and RSU, to support limited automated driving, it requires up to 6000 bytes with high reliability. It is expected that solutions for higher data rate can be investigated. 64QAM modulation would be one of them in some potential scenarios considering quite short distance and good channel environment could happen between cars, e.g. for platooning and advanced driving. 64QAM modulation could support approximate 1.5 times of the payload size by comparing with 16QAM under the similar working assumption.

Observation 1: 64QAM is one of solutions to improve high data rate in some scenarios, e.g. for platooning and advanced driving.

For the Rel-14 LTE-based V2X, a 5 bits field in SCI format 1 for modulation and coding scheme has been defined. With this 5-bit field in SA, 3 types of modulation schemes (QPSK, 16QAM AND 64QAM) can be indicated by the 32 MCS levels in the Table 8.6.1-1 in [3]. However, currently a limitation condition has been set as the modulation order is set to Q'=min⁡(4,Qm'), which means 64QAM cannot be applied to all UEs. When channel condition and corresponding scenario is suitable to Rel-15 UEs for communication with 64QAM to further improve high data rate, the limitation should be released. Rel-14 UEs and Rel-15 UEs can co-exist in the same pool without any impact on the legacy UEs. Since Rel-14 UEs cannot recognize this released limitation, the data transmitted from Rel-15 UEs with 64QAM cannot be decoded. Therefore message carried by 64QAM shall not be the road safety message which must be received by Rel-14 UEs. However, by decoding the SA sent by Rel-15 UEs, the Rel-14 UEs can obtain the resource occupation information.

Observation 2: Rel-14 UEs cannot demodulate the data with 64QAM modulation transmitted by Rel-15 UEs.

Proposal 1: Release the current modulation order limitation can be one of the solutions to support 64QAM for new release UEs to transmit new type of messages in eV2X.

Since the Rel-15 UEs can exist without impact on legacy UEs in the same pool, the legacy SCI format 1 can be used. In order to compatible with Rel-14 UEs, the reserved bits in SCI format 1 can be applied to differentiate Rel-15 UEs and Rel-14 UEs, and it can be also used to indicate the Rel-15 UEs new features information, such as 64QAM modulation. Another option to realize the compatibility is to introduce new SCI format (e.g. SCI format 2) including additional controlling information field.

Proposal 2: Two alternatives on the PSCCH format should be discussed to support 64QAM:

- Alt 1: The reserve bits in SCI format 1 can be used to indicate the Rel-15 UEs’ additional information for compatible with Rel-14 UEs.

- Alt 2: A new SCI format including information for Rel-15 UEs can be introduced (e.g. SCI format 2).

Link level simulation and result

In order to verify the 64QAM performance in V2X phase 2, some relative link level simulation based on the assumption given in Table 1 has been done, which is mostly the same as that in [5]. The channel model of LOS/NLOS is simulated, and four relative speeds (30km/h, 60kn/h, 120km/h and 240km/h) are chosen for simulation, while the absolute speed is 15km/h, 30km/h, 60km/h, and 120km/h, respectively. The number of transmission is also set as 1 and 2.

Table 1. Simulation assumption

| Bandwidth | 10M |
| --- | --- |
| Carrier frequency | 6.0GHz |
| Number of antenna | 1 TX and 2 RX |
| channel | LOS/NLOS |
| Modulation | 64QAM |
| Coding | Turbo,1/2 |
| TB size | 8504bits without GP |
| Number of Occupied PRB | 20 |
| Relative speed | 30km/h, 60km/h, 120km/h 140km/h, 240km/h |
| Channel estimation criteria | LS |
| Number of transmission | {1, 2} |
| Frequency offset compensation | ON |

Figure 1 and figure 2 shows the link level simulation result for 64QAM based on the assumption in the Table 1 above.

In figure 1, for the relative speed 140 km/h under LOS/NLOS channel model, 64QAM can support the BLER less than  10-1 when the SNR is larger than 16. Also for NLOS under this speed, the BLER is 0.25 with the SNR is 22. For the higher relative speed of 240 km/h, error floor seems to appear in the result. Since the speed increases, carrier frequency interference caused by Doppler Effect becomes larger, which lead to high BLER in this results. 64QAM would be affected by high relative speed (e.g. higher than 140 km/h) and NLOS channel model.

Observation 3: Channel condition of NLOS can significantly degrade the 64QAM performance compared with the case of LOS.

Observation 4: High mobility speed can significantly degrade the 64QAM performance for both LOS and NLOS in V2X Phase 2.

In figure 2, four relative speeds (30km/h, 60km/h, 120km/h, 240km/h) under LOS channel model with number of transmission {1, 2} are simulated. For single transmission, since the carrier frequency offset compensation is applied to eliminate the offset, the BLER performance of 30km/h, 60km/h and 120km/h are close to each other. However, higher speed of 240km/h has an error floor due to the inter carrier interference. For transmission number of 2, it is obvious that the BLER performances of different relative speeds are getting close, even for the higher speed of 240km/h.

Observation 5: Retransmission can significantly improve the 64QAM modulation in link level performance.

Proposal 3: Retransmission can be applied while higher order modulation scheme is used for transmission in V2X phase 2.

Figure 1. 64QAM simulation result under LOS and NLOS with two different speeds

Figure 2. LOS-64QAM simulation result with four different speeds

Conclusion

In this contribution, 64QAM for V2X Phase 2 is investigated in order to achieve higher data rate and reliability. By analyzing some requirements and use cases of eV2X, this contribution provides the following observations and proposal:

Observation 1: 64QAM is one of solutions to improve high data rate in some scenarios, e.g. for platooning and advanced driving.

Observation 2: Rel-14 UEs cannot demodulate the data with 64QAM modulation transmitted by Rel-15 UEs.

Proposal 1: Release the current modulation order limitation can be one of the solutions to support 64QAM for new release UEs to transmit new type of messages in eV2X.

Proposal 2: Two alternatives on the PSCCH format should be discussed to support 64QAM:

- Alt 1: The reserve bits in SCI format 1 can be used to indicate the Rel-15 UEs’ additional information for compatible with Rel-14 UEs.

- Alt 2: A new SCI format including information for Rel-15 UEs can be introduced (e.g. SCI format 2).

Observation 3: Channel condition of NLOS can significantly degrade the 64QAM performance compared with the case of LOS.

Observation 4: High mobility speed can significantly degrade the 64QAM performance for both LOS and NLOS in V2X Phase 2.

Observation 5: Retransmission can significantly improve the 64QAM modulation in link level performance.

Proposal 3: Retransmission can be applied while higher order modulation scheme is used for transmission in V2X phase 2.

References

  - RP-170798, "New WID on 3GPP V2X Phase 2", Huawei, CATT, LG Electronics, HiSilicon, China Unicom, Dubrovnik, Croatia, 3GPP RAN #75.

  - RP-162234," Discussion on requirements and future work plan to eV2X ", CATT, Vienna, Austria, 3GPP RAN #74.

  - 3GPP TS 36.213, “Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedure (Release 14)”.

  - 3GPP TR 22.886, “Study on enhancement of 3GPP Support for 5G V2X Services (Release 15)”.

  - 3GPP TR 36.885, “Study on LTE-based V2X Service (Release 14)”.