3GPP TSG RAN WG1 Meeting #89		                                                      R1-1707452

Hangzhou, P.R. China 15th – 19th May 2017

Source:	CATT

Title:	Initial evaluation for Tx diversity schemes in PC5

Agenda Item:	6.2.3.2.2

Document for:	Discussion and Decision

Introduction

In RAN1 #88bis meeting, the potential Tx diversity schemes for PC5 are discussed, particularly with some agreements on evaluation assumptions and candidate Tx diversity schemes to be evaluated [1].

Agreement:

- At least the following candidate TxD schemes for PSSCH transmission to be evaluated:

  - Small delay CDD

  - STBC (including half symbol STBC proposal in R1-1705002)

  - SFBC

  - PVS in time domain

Note: other schemes are not precluded

Agreement:

- At least the following candidate TxD schemes for PSCCH transmission to be evaluated:

  - Small delay CDD

- Note: other schemes are not precluded provided that they fulfil objective 2 of the WID

In this contribution, we will provide some initial evaluation results on the above candidate Tx diversity schemers for PSSCH transmission. The details about the evaluated Tx diversity schemes are illustrated in company’s contribution [2].

Evaluation assumptions

The following Tx diversity schemes for PSSCH are evaluated based on the evaluation assumptions in Table 1:

- Baseline: Rel-14 single antenna transmission scheme.

- SFBC: The REs are paired in adjacent subcarrier, which will introduce higher PAPR.

- STBC: The first and the last symbols are employed as “orphan” symbols, other data symbols are paired subsequently.

- PVS in time domain: Each subframe is divided into 4 precoding sub-blocks, and each sub-block randomly selects a precoding vector from a precoding vector set.

Table 1: Evaluation assumptions

| Parameters | Value |
| --- | --- |
| Carrier frequency | 6GHz |
| Antenna number | 2 x 2 |
| Channel model | LOS in TR36.885, NLOS in TR36.885 |
| Antenna pattern | linear polarization, half-lambda spacing |
| Vehicle speed (absolute) | 15 km/h, 60 km/h |
| MCS | 16QAM, 1/2 |
| Payload size for PSSCH | 300 bytes |
| PRB numbers | 12 |
| Channel estimation | MMSE: Rel-14 single port, SFBC, STBC LS: PVS in time domain |
| Receiver | MMSE |
| Frequency offset correction | No frequency offset correction algorithm |

Initial evaluation results

The link level simulation results are provided in Figure 1 to Figure 4. It can observe that the PVS in time domain performs the worst link performance, especially for the cases that channel is varied rapidly. It is mainly due to the fact that only single DMRS symbol can be employed for channel estimation for each sub-block, which will deteriorate the channel estimation performance.  Furthermore, it can be also observed that both STBC an SFBC schemes perform better performance than Rel-14 transmission scheme. STBC scheme has similar link performance as SFBC schemes in medium UE speed. Since STBC schemes will not increase the PAPR compare to Rel-14 transmission scheme, STBC schemes should be further studied in eV2X.

Figure 1: Evaluation results for LOS channel with 15 km/h

Figure 2: Evaluation results for LOS channel with 60 km/h

Figure 3: Evaluation results for NLOS channel with 15 km/h

Figure 4: Evaluation results for NLOS channel with 60 km/h

Observation 1: Both STBC and SFBC schemes provide obvious link performance gain comparing to Rel-14.

Observation 2: STBC scheme has similar link performance as SFBC schemes in medium UE speed.

Observation 3: PVS in time domain provide the worst link performance due to the worse channel estimation performance.

Proposal: Either STBC or SFBC could be applied to Rel-15 eV2X for PSSCH transmission.

Conclusion

In this contribution, the different Tx diversity schemes are evaluated. Particularly, we have following observations and proposal:

Observation 1: Both STBC and SFBC schemes provide obvious link performance gain comparing to Rel-14.

Observation 2: STBC scheme has similar link performance as SFBC schemes in medium UE speed.

Observation 3: PVS in time domain provide the worst link performance due to the worse channel estimation performance.

Proposal: Either STBC or SFBC could be applied to Rel-15 eV2X for PSSCH transmission.

References

  - RAN1 #88bis chairman notes

  - R1-1707451, “Discussion on Tx diversity scheme in PC5”, CATT