3GPP TSG RAN WG1 meeting #89	R1-1707792

Hangzhou, China 15th - 19th May 2017

Agenda Item:	7.1.4.2.1.1

Source:	Spreadtrum Communications

Title:	Discussion on Polar code construction

Document for:	Discussion and decision

Introduction

In RAN1#88 bis, some agreements on Polar code design were reached and some are FFS.

Agreement:

- The J’ (and J” if any) bits may be CRC and/or PC and/or hash bits (downscope if possible)

- Placement of the J, J’ (and J” if any) assistance bits is FFS after the study of early termination techniques

  - Appended?

  - Distributed?

    - evenly?

    - unevenly?

In this contribution, we take J’ assistance bits as PC bits, and the placement and the improved generation schemes of J’ PC bits are discussed.

Polar code design

Placement of J’ PC bits

In Parity Check Polar code (PC-Polar)[1], PC bits are selected by bit channel reliability and weight together, and this leads to an uneven distributed PC bit position. On one hand, this distributed placement of PC bits show a good block error rate(BLER)and false alarm rate(FAR) performance, and can play a role on early termination. On the other hand, with the distributed placement of PC bit, improved PC bit generation schemes can be designed to further enhance Polar code BLER and FAR performance.

Proposal 1: The J’ PC bits can be distributed and unevenly placed.

Improved PC bit generation schemes

As PC bit can be used for decoding and path selection respectively, the generation schemes of the two PC bit groups can be different also. Based on the distributed placement of PC bit stated in section 2.1, this section gives two PC bit generation schemes to further improve the BLER and FAR performance.

For description convenience, the main parameters of PC-Polar are listed below. Suppose that the information bits length is K, Polar code length is M and the mother Polar code length is N. On the basis of PC-Polar in [1], a sequence Q is used to contain the bits position arranged in the ascending order of bit channel reliability, including information bits positions, frozen bits positions, PC bits positions and puncture bits positions. The length of PC bits is denoted as q, part of which are used for SCL decoding and others for path selection.

PC bit generation for decoding

Suppose PC bits for SCL decoding form a set Pdecode, the propose new generation scheme for the ith PC bit pi in Pdecode is depicted below.

Step1: Find the PC bit(s) before the current PC bit pi that is involved in the calculation of pi

Divide the PC bit sequence whose length is q into s (s>=1) segments. In each segment, the former PC bit always participates in the current PC bit. For example, pi-1will participate in the calculation of pi.The relationship between PC bits can be found in Fig.1.

Fig.1.The relationship between PC bits

Step2: Find the information bits before the current PC bit pi that are involved in the calculation of pi

- For the current PC bit pi, use a set Ii to include all the information bits before pi, the size of which can be denoted as Li.

- Set an initial value a=i, where a is the position of a candidate information bit in Ii that can be involved in the calculation of pi.

- When a≤Li

If the ath information bit in Ii is not marked to have been involved in the calculation of any previously processed PC bit, i.e., p1,…,pi-1,this information bit is added to the calculation of pi and also marked to prevent it from being used in later PC bits. Then update a=a+(q-i+1) and go to c) to find other information bits that are to be used for the calculation of pi.

If the ath information bit in  Ii is marked to have been involved in the calculation of a previously processed PC bit, this information bit is skipped. Then update a=a+(q-i+1) and go to c), to find other information bits that are to be used for the calculation of pi.

When a> Li set i=i+1 and go to a) to process the next PC bit until all PC bits are processed.

Note that, if no other PC bit(s) or information bits are involved in the current PC bit pithis PC bit pi is treated as frozen bit.

After the above two steps, the relationship between PC bits and information bits can be depicted in Fig.2. The blue blocks represent information bits, the white blocks represent frozen bits and the orange blocks represent PC bits.

Fig.2. The relationship between PC bits and information bits.

This scheme provides multi-check for information bits in step 1, and adds the randomness of information bit positions involved in each PC bit with an unequal interval in step 2, to increase the probability of correct decoding, and also improve the BLER error floor performance.

PC bit generation for path selection

For PC bits used for SCL decoding path selection, we propose a new generation scheme depicted below.

Step1: Choose PC bits for SCL decoding to form a set Pselect

Arrange all q PC bits in the order of bit channel reliability, and use x PC bits with the lowest reliability to form a set Pselect.

Step2: Find the information bits involved in the calculation of each pi in Pselect

For each PC bit pi in Pselect, the information bits involved can be randomly selected from all K information bits with a probability P (0<P<1).

This scheme mainly expands the range of protected information bits, to increase the probability of correct path selection, and thus improve the error floor performance.

Simulation

Proposed PC bit generation scheme for SCL decoding

With the proposed generation scheme for Pdecode in section 2.1. Fig.3 gives the simulation results with the simulation assumptions listed in Appendix1. In this scheme, all q PC bits are used for SCL decoding and no PC bit for path selection. The results of PC-Polar in [1] and CRC-Polar in [2] are given for comparison.

Fig.3.BLER performance of PC-Polar for proposed generation scheme for Pdecode

We can see from Fig.3 that, compared to PC-Polar and CRC-Polar, the proposed PC bit generation scheme for Pdecode can improve the error floor performance of PC-Polar.

Observation 1: With the proposed PC bit generation scheme forPdecode, the error floor performance of PC-Polar code can be improved.

Proposed PC bit generation scheme for path selection

In this scheme, suppose x PC bits are used for SCL decoding path selection and the other (q-x) PC bits are for SCL decoding. The (q-x) PC bits for SCL decoding are generated with the scheme mentioned in [1], and x PC bits for SCL decoding path selection are generated with the proposed method in section 2.2. Here, Pselect is formed by a selection of x PC bits with the lowest reliability in Q starting from e.g. the N-K-q2th bits. Fig.4 gives the simulation results for this proposed generation scheme with simulation assumption listed in Appendix2. For comparisons, PC-Polar in [1] where all PC bits are used for SCL decoding is given also.

Fig.4.BLER performance of PC-Polar for proposed generation scheme for Pselect

We can see from Fig.4 that, compared to PC-Polar without Pselect, with the proposed PC bit generation scheme for Pselect, the error floor performance of PC-Polar code can be improved, the SNR gain can reach 0.6dB at the BLER of 10-6.

Observation 2: With the proposed PC bit generation scheme for Pselect, the error floor performance of PC-Polar code can be improved.

Combination of two proposed generation schemes for PC bits

Apply the proposed generation scheme for Pdecode and  Pselect in section 2.1 and section 2.2 together for PC-Polar code. Here, Pselect is formed by a selection of x PC bits with the lowest reliability in Q starting from e.g. the N-K-q2th bits. The FAR performance are shown in Fig.5 with simulation assumption listed in Appendix3. For comparisons, the FAR performance of PC-Polar in [1] and CRC-Polar in [2] are listed also. Note that, all three code schemes contain extra CRC16 which is used for FAR calculation. And in CRC-Polar, CRC4 is used for path selection.

Fig.5 FAR performance of PC-Polar with the combined two generation schemes

We can see from Fig.5 that, with the proposed two PC bit generation schemes, the false alarm rate of PC-Polar code can be improved largely.

Observation 3: With a combination of the proposed two PC bit generation schemes, the false alarm rate of PC-Polar code can be improved.

Conclusions

In this contribution, we present two PC bit generation schemes for PC bits with distributed placement. Based on the above discussion and simulations, we have the following observations:

Observation 1: With the proposed PC bit generation scheme forPdecode, the error floor performance of PC-Polar code can be improved.

Observation 2: With the proposed PC bit generation scheme for Pselect, the error floor performance of PC-Polar code can be improved.

Observation 3: With a combination of the proposed two PC bit generation schemes, the false alarm rate of PC-Polar code can be improved.

Proposal 1: The J’ PC bits can be distributed and unevenly placed.

Proposal 2: The J’ PC bits can be designed with combined generation schemes.

References

R1-1608862 “Polar Code Construction for NR”, Huawei, HiSilicon.

Kai Niu and Kai Chen, CRC-Aided Decoding of Polar Codes[J]. IEEE COMMUNICATIONS LETTERS, VOL. 16, NO. 10, OCTOBER 2012.

Appendix

Appendix 1: Simulation assumption for proposed PC bit generation scheme for Pdecode

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Code rate | 1/2 |
| Info. block length (K) | 1000 |
| Mother code length (N) | 2048 |
| Polar code length (M) | 2000 |
| Decoding algorithm | SCL(list-32) |
| CRC | 16 (0x18005) |
| Puncture method | QUP |
| Construction method | Gaussian construction with construct Eb/N0=1.75dB |

Appendix 2: Simulation assumption for proposed PC bit generation scheme for Pselect

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Code rate | 1/2 |
| Info. block length (K) | 64 |
| Polar code length (M) | 128 |
| Decoding algorithm | SCL(list-32) |
| PC select bits number | 5 |
| Probability P | 0.5 |
| Puncture method | QUP |
| Construction method | Gaussian construction with construct Eb/N0= - 1.59dB |

Appendix 3: Simulation assumption for proposed two PC bit generation schemes

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Code rate | 1/2 |
| Info. block length (K) | 32 |
| Polar code length (M) | 64 |
| Probability (P) | 0.5 |
| Decoding algorithm | SCL(list-8) |
| CRC for FAR calculation | 16 (0x18005) |
| Extra CRC for path selection in CA-Polar | 4(0x13) |
| Puncture method | QUP |
| Construction method | Gaussian construction with construct Eb/N0=1.25dB |