3GPP TSG RAN WG1 Meeting #89	R1-1707946

# Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:	7.1.2.1.5

Source: 	Samsung

Title: 	Diversity transmission for UL

Document for:	Discussion and Decision

Introduction

In 3GPP RAN1#88bis [3], following agreements on UL transmission are made:

Agreements: - Codebook based transmission for UL is supported at least by following signaling in UL grant: - SRI+TPMI+TRI, where - The TPMI is used to indicate preferred precoder over the SRS ports in the selected SRS resource by the SRI. - No SRI when a single SRS resource is configured - The TPMI is used to indicate preferred precoder over the SRS ports in the configured single SRS resource. - Support indication on selection of multiple SRS resources - FFS details Agreements: - For DFTsOFDM in data channel, the following schemes are candidates for transmit diversity: - Low PAPR Alamouti-based transmit diversity applied in frequency or time domain, transparent transmit diversity (e.g. short delay CDD, panel selection), time domain beam/precoder cycling

In this contribution, we discuss about the design of diversity based transmission scheme for UL.

Discussion

Design of UL diversity based transmission scheme

In order to design diversity based transmission scheme, both DMRS and transmission scheme should be considered. Considering DMRS, identical precoding between DMRS and NR PDSCH should be supported in NR. When UE estimates interference for interference cancellation such as IRC and SIC, identical precoding allows simple estimation. However, UE should re-calculate interference when different precoding is used and such calculation increases UE complexity.

For diversity based transmission scheme, precoder cycling should be supported as well as transmission scheme 2 in downlink [4]. While precoder cycling requires simple UE structure due to the usage of identical mapping for codebook/non-codebook based transmission and diversity based transmission, SFBC requires high UE complexity since UE should apply additional operation due to its double dimension of transmission scheme. If NR supports precoder cycling for UL diversity based transmission, then UE requires only one transmitter for codebook/non-codebook based transmission schemes and diversity based transmission scheme. However, UE requires additional hardware to encode predefined precoding due to different precoding between codebook/non-codebook based transmission and diversity based transmission. At the same time, the performance benefit of SFBC is unclear. The extra spatial diversity gain offered by SFBC (over precoder cycling) is present only for very high channel coding rate (i.e. corner cases).

Common framework between DL and UL diversity based transmission scheme

In LTE specification, diversity transmission schemes such as SFBC/FSTD, large delay CDD and precoder cycling are supported only in downlink transmission due to the robustness of DFT-S OFDM waveform in uplink. However, the situation is different since support of CP-OFDM is agreed in NR. While DFT-S OFDM can provide more coverage with low PAPR, CP-OFDM is beneficial since CP-OFDM can provide more system performance benefits. Considering such aspects, diversity based transmission scheme should be provided at least for CP-OFDM.

For CP-OFDM, common framework for diversity based transmission scheme should be supported. One objective of NR system is to enable a network to instantaneously adapt to traffic changes and enable time domain multiplexing (TDM) of traffic types with different transmission characteristics. For such flexible duplex operation, a slot type (DL, UL, or hybrid) may not be fixed and can be dynamically indicated by gNB.

Dynamic indication on slot type in an uncoordinated manner can create cross-link interference when different slot types are used in neighbouring cells. In that case, support of cross-link interference cancellation is required especially for gNB since DL interference is typically much stronger than UL interference. In order to support interference cancellation between different slot types, common framework between DL and UL transmission scheme is beneficial since it allows simple interference channel estimation regardless of slot types.

Support of multi-beam based diversity transmission

In NR, UE can be configured to monitor NR-PDCCH on multiple beam pair links simultaneously [2]. Such downlink control data transmission based on multiple beam pair can provide robustness and reliability when UE experiences blockage on one of configured beams. Such reliable transmission via multi-beam pair links can be applied to UL diversity based transmission to provide robustness and reliability. Figure 1 shows exemplary operation of multi-beam based UL diversity transmission. In this operation, UE can realize multiple Tx beam via multiple SRS resources which is indicated via SRI by gNB. Based on the information, UE can apply different beam to its data transmission with pre-defined or configured granularity (e.g. slot, mini-slot or REG).

Figure1. UL diversity transmission via multiple SRS resource indication

Observations:

- Same precoding in data and DMRS allows simple estimation for interference cancellation.

  - However, different precoding requires more UE complexity due to different precoding and blind detection.

- Precoder cycling requires only one transmitter for codebook/non-codebook based transmission and diversity transmission.

  - For SFBC, additional hardware to decode predefined precoding due to different precoding and double dimension in adjacent REs.

- At least for CP-OFDM, common framework of transmission scheme has benefits on interference cancellation from different slot types.

- Diversity transmission based on multi-beam can provide robust transmission to UE.

Proposals:

- For UL diversity based transmission scheme, precoder cycling should be supported for both CP-OFDM and DFT-S OFDM.

- At least for CP-OFDM, common framework between transmission scheme 2 in DL and diversity based transmission scheme in UL should be supported.

- UL diversity transmission based on multiple SRS resource indication should be supported in NR.

Conclusions

In this contribution, diversity based transmission for UL is discussed. Based on the discussions, the following observations and proposals are provided:

Observations:

- Same precoding in data and DMRS allows simple estimation for interference cancellation.

  - However, different precoding requires more UE complexity due to different precoding and blind detection.

- Precoder cycling requires only one transmitter for codebook/non-codebook based transmission and diversity transmission.

  - For SFBC, additional hardware to decode predefined precoding due to different precoding and double dimension in adjacent REs.

- At least for CP-OFDM, common framework of transmission scheme has benefits on interference cancellation from different slot types.

- Diversity transmission based on multi-beam can provide robust transmission to UE.

Proposals:

- For UL diversity based transmission scheme, precoder cycling should be supported for both CP-OFDM and DFT-S OFDM.

- At least for CP-OFDM, common framework between transmission scheme 2 in DL and diversity based transmission scheme in UL should be supported.

- UL diversity transmission based on multiple SRS resource indication should be supported in NR.

- References

- RP-170847, “New WID on New Radio Access Technology”, NTT DOCOMO

- 3GPP TSG RAN1 NR Ad-Hoc Spokane, Chairman’s Note

- 3GPP TSG RAN1#88bis, Chairman’s Note

- R1-1707943, “Discussions on transmission scheme 2”, Samsung.