3GPP TSG RAN WG1 Meeting #89	R1-1707981

# Hangzhou, China, 15th – 19th May 2017

Agenda item:	7.1.2.4.7

Source: 	Samsung, Huawei, HiSilicon

Title: 	Remaining issues on NR CSI-RS

Document for:	Discussion and Decision

Introduction

During NR SI, CSI-RS have been discussed for various purposes such as CSI acquisition, beam management, mobility support, as captured by [1]. This contribution provides remaining details on CSI-RS for NR MIMO based on the previous agreements.

Remaining issues on NR CSI-RS

According to the agreements from RAN1#85 to RAN1#88bis in Appendix [2]-[8], potential functionalities of CSI-RS can be summarized as follows:

- CSI acquisition

- Beam management

- L3 CONNECTED mode mobility support

- Others, e.g., fine time/frequency tracking, beam failure monitoring, power control, etc.

For each functionality, the following remaining details should be finalized taking into account possible impacts on other working group:

- For CSI acquisition,

  - CSI-RS RE patterns within a PRB and slot for 1-port and >=8-port up to 32-port CSI-RS including RE pattern aggregation details.

  - Details on CDM-2/-4/-8 including CDM pattern, configuration, etc.

  - RS RE densities

    - Values of D other than {1, 1/2} RE/port/PRB.

    - Detailed configurations for each value of D, including comb offset values for D<1, etc.

    - Possible combinations of value(s) of D and the number of CSI-RS ports X.

  - Applicability for interference measurement.

- For beam management,

  - Number of ports

  - CSI-RS RE patterns and configuration details of CSI-RS.

    - Tx and Rx beam sweeping mechanism(s)

    - Sub-time unit details including down-selection among options when sub-time units smaller than an OFDM symbol in a reference numerology, e.g. IFDMA, larger subcarrier spacing, DFT-based methods

  - How to support multiplexing of CSI-RS ports, e.g. FDM/TDM/CDM.

  - RS RE densities.

  - Whether CSI-RS for beam management is UE-specifically configurable only or cell-specific options are also possible.

- For L3 CONNECTED mode mobility,

  - Whether CSI-RS configured for L3 CONNECTED mode mobility is the same as CSI-RS configured for beam management

    - Note: There may be differences if additional requirements are identified on the CSI-RS to support L3 CONNECTED mode mobility.

  - Number of ports.

  - CSI-RS RE patterns.

  - How to support multiplexing of CSI-RS ports, e.g. FDM/TDM/CDM.

  - RS RE densities.

  - Whether CSI-RS for L3 CONNECTED mode mobility is UE-specifically configurable only or cell-specific options are also possible.

- For other functionalities, e.g., fine time/frequency tracking (if supported by CSI-RS), beam failure monitoring, power control (if supported by CSI-RS), etc.

  - Number of ports.

  - CSI-RS RE patterns.

  - How to support multiplexing of CSI-RS ports, e.g. FDM/TDM/CDM.

  - RS RE densities.

- Common issues for all functionalities:

  - Mapping of port numbers to RE pattern.

  - Location of OFDM symbols for CSI-RS transmission.

  - Reference signal sequence, including sequence initialization and generation.

  - QCL assumption and/or association (if supported) with other signal(s) and relevant signalling (if any).

    - Note: CSI-RS can be a candidate for the other signal(s) above as well.

  - Numerology of CSI-RS

  - Configuration of CSI-RS parameters including at least

    - CSI-RS bandwidth parameters, including bandwidth granularity.

    - Timing related configuration (e.g. periodicity, slot offset, etc.) for semi-persistent/periodic CSI-RS.

    - Numbers of configured Resource settings, Resource sets, Resources, and CSI-RS ports.

  - Transmission power related configuration (e.g. EPRE ratio between CSI-RS and PDSCH)

  - Triggering mechanism(s) for aperiodic CSI-RS including activation/deactivation.

  - Activation/deactivation mechanism(s) for semi-persistent CSI-RS.

- Appendix

- Agreements in RAN1#85

RAN1#85 Agreements: - In NR multi-antenna schemes, studies on RS design and CSI acquisition considering following use cases - Non-UE-specific RS use case for CSI measurement - UE-specific RS use case for CSI measurement - Note: functionally may be analogous e.g.: to R13/14 UE-specific beamformed CSI-RS (with dynamic beamforming) - Note: maybe relevant to UE receiving beam sweeping - Use of a joint operation among multiple RS from the same or different use cases - RS for interference measurement use cases - RS for channel reciprocity use cases - Other use cases is not precluded - Note: including measurement over aperiodic/periodic/semi-persistent RS - Note: Subband RS is not precluded

- Agreements in RAN1#86

Agreements: - At least one of following RS configurations for CSI measurement are supported in NR - Aperiodic RS - Semi-persistent RS - Periodic RS - Study RS pattern for CSI measurement in NR both for DL and UL - Study the density of RS - Study the position of RS in the [subframe] - Study the mapping of RS in one or several symbols

- Agreements in RAN1#86bis

Working assumptions: - Beam management procedures can utilize at least the following RS type(s): - RS defined for mobility purpose at least in connected mode - FFS: RS can be NR-SS or CSI-RS or newly designed RS - Others are not precluded - CSI-RS: - CSI-RS is UE-specifically configured - Multiple UE may be configured with the same CSI-RS - The signal structure for CSI-RS can be specifically optimized for the particular procedure - Note: CSI-RS can also be used for CSI acquisition - Other RS could also be considered for beam management such as DMRS and synchronization signal Agreements: - At least for CSI acquisition, NR supports CSI-RS, SRS - FFS: Use of DL DMRS for CSI measurement - NR supports aperiodic transmission of CSI-RS - For CSI-RS transmission, NR supports at least one of following: - Semi-persistent transmission - Preconfigured CSI-RS resources can be activated or de-activated - FFS: Activation/De-activation mechanism - Periodic transmission - Periodic CSI-RS can be configured by higher layer - FFS: mechanisms (e.g. protocol layer) for dynamic control of activation/deactivation for semi-persistent CSI-RS - FFS: mechanisms to provide reliable activation/deactivation for semi-persistent CSI-RS Agreements: - A CSI-RS resource set should be studied whereby CSI-RS resources within the set may be dynamically shared amongst users - Study CSI-RS resources configuration in 2 steps - Step 1: Pre-configure a set of K CSI-RS resources for one or more users through UE-specific RRC configuration - Step 2: For a given user, dynamically indicate N out of K (where N >= 1) of the CSI-RS resources from the set to enable CSI measurement in a given time window - Study the following alternatives - Alt 1: Dynamic signaling through DCI - Alt 2: Dynamic signaling through MAC CE - The CSI-RS transmissions can be dynamically indicated as one shot or multi-shot within the window - The configured CSI-RS resource pool can be used for channel/interference measurement - Configure multi sets of CSI-RS resources for one UE Agreements: - The feasibility of CSI-RS transmission and CSI reporting in a self-contained structure (e.g., subframe) should be studied, considering UE implementation complexity - FFS: DM-RS based CSI reporting Agreements: - At least the following RSs are supported for NR downlink - CSI-RS: Reference signal with main functionalities of CSI acquisition, beam management - FFS: RRM measurement Agreements: - Support NR CSI-RS pattern with at least the following properties: - CSI-RS mapped in one or multiple [consecutive] symbols - FFS: At least CSI-RS located at the earlier part of a slot in some cases - FFS: before or after the DM-RS agreed to study in RAN1#85 - FFS: CSI-RS located at other part of a slot - Working assumption: Configurable density of CSI-RS in frequency and/or time domain by UE-specific manner - Working assumption: CSI-RS for NR should support up to 32 ports - FFS: whether or not to have 32 ports codebook - Support at least following configurations of NR CSI-RS - UE-specific configuration to support - Wideband CSI-RS, i.e. from UE perspective, the full bandwidth the UE is configured to operate with - Partial-band CSI-RS, i.e. from UE perspective, part of the bandwidth the UE is configured to operate with - FFS: Different patterns may be used for wideband and subband CSI-RSs Agreements: - CSI-RS configuration for NR includes at least ‘number of antenna ports’ - Configuration can be explicit or impliit - The number of CSI-RS antenna ports can be independently configured for periodic/semi-persistent CSI reporting and aperiodic CSI reporting Working assumption: - The number of antenna ports supported for CSI-RS configuration includes at least the following values - 1,2,4,8,[12],16,[20],[24],[28],32 - …

- Agreements in RAN1#87

Agreements: - NZP CSI-RS resource is defined in NR, as a set of NZP CSI-RS port(s) mapped to a set of REs within a frequency span/a time duration (details FFS) which can be measured at least to derive a CSI - Multiple NZP CSI-RS resources can be configured to UE at least for supporting CoMP and multiple beamformed CSI-RS based operations, where - Each NZP CSI-RS resource at least for CoMP can have different number of CSI-RS ports. - Further study at least the following aspects: - FFS QCL aspects - E.g., set of QCL parameters - E.g., QCL assumptions within a NZP CSI-RS resource, among two or more resources, etc. - FFS whether or not a single NZP CSI-RS resource can be used to derive two or more CSIs - FFS whether or not many NZP CSI-RS resources can be used to derive a single CSI Agreements: - NR supports semi-persistent CSI-RS transmissions - Activation(s)/de-activation(s) of CSI-RS resource is triggered dynamically - Note: “dynamically” here can be DCI and/or MAC CE based. FFS details. - NR supports semi-statically configured/re-configured periodic CSI-RS transmissions - FFS: Details on signaling mechanisms Agreements: - CSI-RS supports the DL Tx beam sweeping and UE Rx beam sweeping - NOTE: CSI-RS can be used in P1, P2, P3 - NR CSI-RS supports the following mapping structure - NP CSI-RS port(s) can be mapped per (sub)time unit - Across (sub)time units, same CSI-RS antenna ports can be mapped - Values of NP is FFS - Here, “time unit” refers to n>=1 OFDM symbols in a configured/reference numerology, where the value of n is FFS - FFS whether OFDM symbols comprising a time unit is consecutive or not - FFS Port multiplexing method, e.g., FDM, TDM, CDM, any combinations - Each time unit can be partitioned into sub-time units - FFS Partitioning method, e.g., TDM, IFDMA, OFDM symbol-level partition with same/shorter OFDM symbol length(i.e. larger subcarrier spacing) as/than the reference OFDM symbol length (subcarrier spacing), and other methods are not precluded - This mapping structure can be used for supporting multiple panels/Tx chains - Options to map CSI-RS for Tx and Rx beam sweeping for further study - Option 1: - Tx beam(s) are same across sub-time units within each time unit - Tx beam(s) are different across time units - Option 2: - Tx beam(s) are different across sub-time units within each time unit - Tx beam(s) are same across time units - Option 3: combination of Alt1 and Alt2: - Within one time unit, Tx beam(s) are same across sub-time units. - Within another time unit, Tx beam(s) are different across sub-time units. - FFS combination of the different time units in terms of e.g., number and periodicity - Note that only Tx sweeping or Rx sweeping is also a possibility - Other options are not precluded. - FFS: how to capture “same beam” and “different beam” in spec - FFS: Whether the above mapping structure is configured with one or multiple CSI-RS resource configurations Agreements: - In NR, a UE can be configured with a CSI-RS resource configuration with X ports - Supported values of X are up to at least 32 - NR supports up to at least 32 port codebook - FFS: Codebook design - Study the potential benefits of 64 ports - A UE can be configured for CSI acquisition with the following features: - N≥1 CSI reporting settings, M≥1 RS settings, J≥1 IM settings, and a CSI measurement setting which links the N CSI reporting settings with the M RS settings and J IM settings - … - An RS setting includes at least the following: - Time-domain behavior: aperiodic or periodic/semi-persistent - RS type which encompasses at least CSI-RS - RS resource set(s) of K resources Working assumptions: - For Type I, CSI feedback using a PMI codebook for X CSI-RS ports is supported - Supported values of X are at least 1,2,4,8,12,16,[24],32 - Note: For X=1, Type I does not have PMI feedback - Support for other values of X is not precluded - Note the number of ports in CSI-RS resource configuration may not be the same with the number of ports in the PMI codebook Agreements: - Support UE specific RRC configuration of one or more CSI-RS resource sets - As one operation example, CSI-RS resources within a set can be dynamically shared amongst users - Support dynamic allocation of one or more CSI-RS resources from the one or more sets to one or more users - Allocation can be aperiodic (single-shot) - Allocation can be on a semi-persistent basis - Note: semi-persistent CSI-RS transmission is periodic while allocated - Study the following aspects - Signaling mechanism for dynamic allocation/de-allocation - e.g. via MAC CE or L1 control signaling, via hierarchical control signaling with MAC-CE and DCI - De-allocation may or may not be dynamically signaled at a later point in time - Signaling mechanism for slot configuration (i.e., periodicity, subframe offset like in LTE) - Signaling format for dynamic indication of CSI-RS resource(s) - Signaling overhead reduction approaches - e.g., structured groupings of CSI-RS resources and/or allocation scheme utilizing a combination of RRC configuration and dynamic signaling - Methods to support resources of variable number of ports - e.g. Aggregation of resources of a smaller number of ports - Note: CSI-RS can be NZP - FFS: ZP - Note: mapping of CSI-RS resources with TRP(s) is an implementation issue RAN1#87 Agreements: - NR strives for efficient support of dynamic resource allocation of different numerologies in FDM/TDM fashion. - Potential specification impact includes but is not limited to: - FFS:CSI-RS measurement - FFS: the time and frequency granularity of dynamic resource allocation - FFS: If spectrum confinement (filtering, windowing, …) can be dynamically varied or not RAN1#87 Agreements: - Consider the following signal combinations for inter-cell RRM measurements for CONNECTED and IDLE until RAN1-NR: - Option 1: Same RS - … - Option 2: Not same RS - Option 2-1: NR-SSS in IDLE; MRS-{1,2} in CONNECTED - Option 2-2: NR-SSS in IDLE; NR-SSS and MRS-{1,2} in CONNECTED - Option 2-3: NR-PSS and/or NR-SSS in IDLE; NR-PSS and/or NR-SSS and CSI-RS in CONNECTED - Option 2-4: For CONNECTED, RS for IDLE and MRS-{1,2,3} - …

- Agreements in RAN1 NR-AH

Agreements: - UE measurement based on RS for beam management (at least CSI-RS) composed of K (= total number of configured beams) beams and reporting measurement results of N selected beams: - N is not necessarily fixed number - FFS: whether/how to configure and/or indicate the values of N - Note: The above procedure based on RS for mobility purpose is not precluded. - Reporting information at least include - Measurement quantities for N beam (s) - FFS: Detailed reporting contents, e.g., CSI, RSRP or both - FFS: How to select N beam(s) - FFS: how to identify the subset - Information indicating N DL Tx beam(s), if N < K - FFS: the details on this information, e.g., CSI-RS resource IDs, antenna port index, a combination of antenna port index and a time index, sequence index, etc. Agreements: - … - At least the following configuration parameters are signaled via RRC at least for CSI acquisition: - … - In each Resource setting: - A configuration of S≥1 CSI-RS resource set(s) - Note: each set corresponds to different selections from a “pool” of all configured CSI-RS resources to the UE - A configuration of Ks ≥1 CSI-RS resources for each set s, including at least: mapping to REs, the number of ports, time-domain behavior, etc. - … - At least following are dynamically selected by L1 or L2 signaling, if appilicable - One or multiple CSI reporting settings within the CSI measurement setting - One or multiple CSI-RS resource sets selected from at least one Resource setting - One or multiple CSI-RS resources selected from at least one CSI-RS resource set - FFS until the next meeting about details of dynamic triggering Agreements: - For periodic CSI-RS, - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI - Aperiodic CSI reporting is triggered by DCI - FFS: Necessity of additional signaling with MAC CE - For semi-persistent CSI-RS, - Periodic CSI reporting is not supported. - Semi-persistent CSI reporting is activated/deactivated by MAC CE and/or DCI - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission - Aperiodic CSI reporting is triggered by DCI - Semi-persistent CSI-RS is activated/deactivated by MAC CE and/or DCI - FFS: Necessity of additional signaling with MAC CE - For aperiodic CSI-RS, - Periodic [and semi-persistent] CSI reporting is not supported - Aperiodic CSI reporting is triggered by DCI - Aperiodic CSI-RS is triggered by DCI and/or MAC CE - FFS: Relationship of signaling between CSI reporting and CSI-RS transmission, e.g., common DCI signaling between CSI reporting and CSI-RS transmission - FFS: Necessity of additional signaling with MAC CE - Note that further down selection can be done later between MAC CE and DCI in above bullets - Note that it is possible to dynamically trigger RS and reports through links in the measurement setting Agreements: - NR supports mechanism(s) to trigger aperiodic CSI-RS and aperiodic CSI reporting simultaneously. - FFS: signaling details (e.g. single DCI or separate DCI) - FFS: method(s) for interference measurement - FFS: reference resource of aperiodic CSI reporting - For aperiodic CSI-RS timing offset X, support X=0 at least, if aperiodic CSI-RS triggering is done by DCI - FFS: supporting other values of X - Note: ‘aperiodic CSI-RS timing offset X’ refers to the time gap between aperiodic CSI-RS triggering and aperiodic CSI-RS transmission w.r.t. number of slots. - For CSI reporting timing offset Y that is fixed or configurable by the network but with certain restriction on lower limit of Y to provide sufficient CSI computation time. - Candidate values of Y are fixed or pre-determined by certain rule(s). - Rule is FFS (e.g. number of CSI measurement/RS/reporting settings, CSI feedback type, number of ports, nearest CSI-RS transmission timing, UE capability, etc.) - Note: ‘aperiodic CSI reporting timing offset Y’ refers to the time gap between aperiodic CSI reporting triggering and aperiodic CSI reporting w.r.t. number of slots. - FFS Y is fixed or configurable - FFS configuration is done by DCI and/or MAC CE and/or higher layer signaling Agreements: - For NR, support at least two types of resources used for interference measurement in CSI configuration based on the following candidates: - ZP CSI-RS, NZP CSI-RS, DMRS - Including independent or joint usage of any combination of the above three candidates - Selection is to be done in RAN1#88 Agreements: - When a UE is configured with K>1 NZP CSI-RS resources, a UE can report a set of N UE-selected CSI-RS-resource-related indices - FFS: The name of the corresponding CSI reporting parameter - FFS: Maximum value of N (support of N=1 only is not precluded) - FFS: What other CSI parameter(s) are reported together with the CSI-RS-resource-related indices, if any, and joint reporting mechanism (in case of multiple reporting parameters) - FFS: Extension of this feature to other types of Resources Agreements: - The CSI acquisition framework (including CSI measurement, Resource, and CSI reporting settings) supports configurations that provide CSI similar to Rel.14 eFD-MIMO hybrid CSI mechanisms 1 and 2, e.g. - Long-term CSI for a number of antenna ports or multiple NZP CSI-RS resources - Short-term CSI for a number of antenna ports with one or more NZP CSI-RS resources - The number of ports for long-term and short-term CSIs can be the same or different - Contents of the CSI report are FFS - This is achieved without introducing new features into the agreed upon CSI acquisition framework Agreements: - Study the following DL CSI feedback for different degree of channel reciprocity, - For full channel reciprocity - Explicit interference feedback: e.g., Interference covariance matrix, diagonal elements of interference covariance matrix - Implicit interference feedback: e.g., Interference PMI feedback - Explicit channel feedback: e.g., CSI of multiple TRPs - For partial channel reciprocity (e.g., more Rx ports than Tx ports at UE) - Partial CSI feedback for eNB to acquire full CSI - Study whether or not to support CSI-RS and SRS transmission in the same slot (e.g., for fast CSI acquisition) - Study if a limitation on the number of SRS and/or CSI-RS ports due to UE complexity, slot duration of different numerology are needed. Agreements: - Indication of QCL between the antenna ports of two CSI-RS resources is supported. - By default, no QCL should be assumed between antenna ports of two CSI-RS resources. - Partial QCL parameters (e.g., only spatial QCL parameter at UE side) should be considered. - For downlink, NR supports CSI-RS reception with and without beam-related indication, - When beam-related indication is provided, information pertaining to UE-side beamforming/receiving procedure used for CSI-RS-based measurement can be indicated through QCL to UE - QCL information includes spatial parameter(s) for UE side reception of CSI-RS ports - FFS: information other than QCL Agreements: - The CSI-RS RE mapping pattern of one N-port CSI-RS resource is composed of one or multiple CSI-RS RE mapping patterns of CSI-RS resources of equal or smaller number of ports, [e.g., 2, 4, or 8] - A CSI-RS RE mapping pattern is defined within a slot - FFS: A CSI-RS RE mapping pattern can span multiple configurable consecutive/non-consecutive OFDM symbols - FFS on mapping of ports to the CSI-RS RE mapping pattern - Density per port in terms of RE per port per PRB is configurable supports for density greater than 1 is not precluded Agreements: - Beam management overhead and latency are to be considered during the CSI-RS design for NR beam management, considering the following possible candidate solutions: - Opt1. IFDMA - Opt2. Larger subcarrier spacing - Other solutions are not precluded - FFS: whether the above structure should be utilized for P-1 and/or P-2 and/or P-3. - Other aspects considered during the CSI-RS design for NR beam management include, e.g. CSI-RS multiplexing, UE beam switch latency and UE implementation complexity (e.g. AGC training time), coverage of CSI-RS, etc. - Note that it does not imply prioritizing different aspects in CSI-RS design Agreements: - For fine time/frequency tracking, consider - The required RE resource/patterns - E.g. CSI-RS, RS for mobility (if adopted, may be CSI-RS or other RSs), DM-RS for broadcast/unicast/control (if introduced), PT-RS, a dedicated RS (if required), etc. - Other RSs are not precluded - The required periodicity and the corresponding configurations - Tracking performance Agreements: - Pathloss measurement for UL power control to be based on at least one type of DL RS for beam measurement is supported. - Notes: beam measurement RS includes CSI-RS, RS defined for mobility purpose, FFS: SS & DMRS - FFS: on multiple type of RS - FFS: other DL RS - … Agreements: - ... - For CONNECTED mode RRM measurement for L3 mobility, the following RS can be used if needed, in addition to IDLE mode RS: - FFS: CSI-RS, - FFS: RS separately designed from CSI-RS - Note that possibility of multiplexing of wideband RS in SS block is not precluded

- Agreements in RAN1#88

Agreements: - A UE can be configured with the following high layer parameters for beam management: - N≥1 reporting settings, M≥1 resource settings - The links between reporting settings and resource settings are configured in the agreed CSI measurement setting - CSI-RS based P-1 & P-2 are supported with resource and reporting settings - P-3 can be supported with or without reporting setting - … - A resource setting at least including - Time-domain behavior: e.g. aperiodic, periodic, semi-persistent - RS type: NZP CSI-RS at least - At least one CSI-RS resource set, with each CSI-RS resource set having K≥1 CSI-RS resources - FFS whether or not support >1 CSI-RS resource set per resource setting - Some parameters of K CSI-RS resources can be the same, e.g. port number, time-domain behavior, density and periodicity if any - Further discussion whether or not the mechanism for CSI acquisition framework can be applicable Agreements: - NR supports the following beam reporting considering L groups where L>=1 and each group refers to a Rx beam set (Alt1) or a UE antenna group (Alt2) depending on which alternative is adopted. - For each group l, UE reports at least the following information: - Information indicating group at least for some cases - FFS: condition(s) to omit this parameter e.g. when L=1 or Nl=1 - Measurement quantities for Nl beam (s) - Support L1 RSRP and CSI report (when CSI-RS is for CSI acquisition) - … - Information indicating Nl DL Tx beam(s) when applicable - FFS: the details on this information, e.g., CSI-RS resource IDs, antenna port index, a combination of antenna port index and a time index, sequence index, beam selection rules for assisting rank selection for MIMO tx, etc. - … Agreements: - FFS whether or not to support at least the following association of channel properties between UL and DL antenna ports at the same node for the case with and without beam correspondence - UL SRS/DM-RS/RACH and DL CSI-RS/DM-RS/SS - Details of beam related indication signaling (if any) Agreements: - For NR, the CSI parameter CRI (CSI-RS Resource Indicator) is supported - FFS the applicability to CSI acquisition/beam management - CRI functionality includes selection and reporting of indices for N out K NZP CSI-RS resources - FFS: Maximum value of N (Nmax), including the possibility of having Nmax=1 - FFS using CRI to additionally select NZP CSI-RS resources for interference measurement (if supported) - If Nmax > 1 is supported, the value of N is included in the associated CSI reporting setting - The maximum value of N∈{1,2,…,K} may be a UE capability - FFS whether N is higher-layer configured or UE selected Agreements: - FFS for aperiodic NZP CSI-RS, semi-persistent CSI reporting is not supported. - FFS in each semi-persistent or periodic CSI reporting setting, reporting periodicity is additionally included in the configuration information - FFS other information that can be included (e.g. timing offset, etc.) - In each semi-persistent or periodic resource setting, periodicity is additionally included in the configuration information Agreements: - Study further activation mechanism (selecting N out of K NZP/ZP CSI-RS resources or resource sets, including joint trigger vs. separate triggers for resources, IMR, and/or reporting setting, reliability/latency associated with the activation, etc.) and the associated details (e.g., impact on DCI signaling, etc.) for aperiodic (AP) and semi-persistent (SP) CSI-RS Agreements: - NR supports ZP CSI-RS based interference measurement for CSI feedback - Note: this support is not transparent to specification - FFS the case of DM-RS & NZP CSI-RS based interference measurement for CSI feedback - Whether to support one of them or both - Whether the support is transparent to specification or not Agreements: - NR supports aperiodic, semi-persistent (as a working assumption), and periodic IMR based on ZP CSI-RS for interference measurement for CSI feedback - For IMR based on ZP CSI-RS, above three different time-domain behaviors are configured in the resource setting(s). - FFS the potential impact on PDSCH rate matching Agreements: - The RE pattern for an X-port CSI-RS resource spans N ≥ 1 OFDM symbols in the same slot and is comprised of one or multiple component CSI-RS RE patterns where - A component CSI-RS RE pattern is defined within a single PRB as Y adjacent REs in the frequency domain and Z adjacent REs in the time domain - FFS: Support for more than one component CSI-RS RE pattern definition, i.e., value of Y and Z - FFS: Supported value(s) of Y and Z, e.g. (Y, Z) = {(1,2), (2,1), (4,1), (8,1), (2,2), (2,4)} - FFS: How to apply CDM within a component CSI-RS RE pattern and across multiple component CSI-RS RE patterns - Note: Depending on the density reduction approach agreement(s), the Y REs of a component CSI-RS RE pattern may be non-adjacent in the frequency domain - The multiple component CSI-RS RE patterns can be extended across the frequency domain within the configured CSI-RS bandwidth - At least the following numbers of OFDM symbols for a CSI-RS resource are supported - N = {1, 2, 4} - FFS, other value(s) of N - The N OFDM symbols can be adjacent/non-adjacent - FFS, down-selection on adjacent/non-adjacent OFDM symbols - OFDM symbol(s) can be configured to contain CSI-RS only - FFS: the applicability of above proposals for beam management (e.g., for beam sweeping, for generation of sub-time units) - FFS on the following aspects: - Location of N OFDM symbols within a slot - Replication of RE pattern across the N OFDM symbols - Supported CDM values - Exact port number assignment to CSI-RS RE pattern, considering at least CDM of ports and relationship of port numbers to dual polarized antenna elements - Support for densities D > 1 RE/port/PRB - Mapping of other physical channels and/or reference signals within the same OFDM symbol(s) as CSI-RS Agreements: - For beam management overhead and latency reduction, NR considers following options for a CSI-RS supporting beam sweeping within an OFDM symbol - Opt-1: IFDMA [e.g., R1-1700350, R1-1703179] - Opt-2: Larger subcarrier spacing [e.g., R1-1700350, R1-1701813] - Opt-3: DFT-based [e.g., R1-1702329, R1-1703179] - Other options are not precluded - If supported, down-selection among the options during WI phase - Note: the symbol duration is based on a reference numerology Agreements: - FFS indication of QCL assumption for CSI-RS to be associated with an SS block (e.g., SSS, PBCH DMRS (if defined)) and/or RS for fine time-frequency tracking (if it’s not CSI-RS) - FFS: w.r.t. which QCL parameter(s) Agreements: - The candidate reference signal for time and frequency tracking for study - Demod RS for common control resource set or common control search space together with extension in time domain (if introduced) - CSI-RS - Consider the enhanced type in both time and frequency domain (if needed) - Dedicated RS (if introduced) - DMRS for PBCH (if introduced) - MRS (if introduced) - PT-RS - PSS/SSS - Note that it doesn’t mean that only one RS can serve tracking for time and frequency - …

- Agreements in RAN1#88bis

Agreements: - For UE RRC connected mode, periodic signal is supported at least for P1 procedure (Tx/Rx beam alignment) using following options in addition to UE-specifically configured CSI-RS. Down selection from following options will be conducted in the next meeting. - Opt. 1: SS blocks - Opt. 2: Cell-specifically configured CSI-RS - Configuration of CSI-RS is obtained from the broadcast message (e.g., MIB, SIB) - Opt. 3: No additional option Agreements: - Aim for low-overhead indication for spatial QCL assumption to assist UE-side beamforming/receiving - FFS details (e.g., tag-based where the tag refers to previous CSI-RS resources, BPL-based, referring to previous measurement reports, indication one resource (set) out of multiple resource (set)s configured by RRC, CSI-RS resource/port index based, etc.) Agreements: - … - Beam failure detection - UE monitors beam failure detection RS to assess if a beam failure trigger condition has been met - Beam failure detection RS at least includes periodic CSI-RS for beam management - SS-block within the serving cell can be considered, if SS-block is also used in beam management as well - FFS: Trigger condition for declaring beam failure - New candidate beam identification - UE monitors beam identification RS to find a new candidate beam - Beam identification RS includes - Periodic CSI-RS for beam management, if it is configured by NW - Periodic CSI-RS and SS-blocks within the serving cell, if SS-block is also used in beam management as well - … Agreements: - Semi-persistent CSI reporting is not supported for aperiodic CSI-RS. - Note: This doesn’t preclude one CSI report carried by multiple UL reporting instances Agreements: Based on at least the supported number S of CSI-RS resource sets and number of CSI-RS resources Ks per resource set, agree to specify one or both of these options for aperiodic CSI-RS - Option 1: use RRC + MAC CE + DCI - Option 2: use RRC + DCI Agreements: - For interference measurement, down selection from options will be conducted. - NZP CSI-RS based - Opt. A1: Estimation on NZP CSI-RS for channel estimation (by subtracting CSI-RS from Rx signal) - Opt. A2: Emulation on NZP CSI-RS which is represented by multiplied value of channel and precoding matrix - DM-RS based - Opt. B1: Estimation on DM-RS for own data demodulation (by subtracting DM-RS from Rx signal) - Opt. B2: Estimation on DM-RS for other UEs - … Agreements: - Aperiodic IMR is triggered by DCI. - Herein the IMR refers to ZP CSI-RS based IMR. Other types of IMR are FFS - FFS: use of MAC CE to reduce the set of candidate IMRs among the resources configured in RRC. - FFS whether or not to jointly trigger aperiodic NZP CSI-RS and/or aperiodic ZP CSI-RS - FFS: semi-persistent IMR is activated/deactivated by MAC CE or DCI. Working assumption: - For CSI-RS for Beam Management, NR supports sub-time units equal to and smaller than an OFDM symbol in a reference numerology - FFS details including configurability, e.g., taking into account UE implementation complexity/capability and impact on CSI-RS design - FFS the case of time unit larger than an OFDM symbol in a reference numerology - E.g., - Opt-1: IFDMA - Opt-2: Larger subcarrier spacing - Opt-3: DFT-based Agreements: - At least for CSI-acquisition, for density 1 RE/port/PRB, X<8, and N=1 or 2 OFDM symbol, support X-port CSI-RS resource composed of M adjacent RE(s) in the frequency domain and N adjacent RE(s) in the time domain - FFS X=1 - X=2: (M, N)= (2, 1) , FFS (1, 2) - X=4: (M, N)= (4, 1) , (2, 2) - FFS N > 2 - FFS: RE patterns for beam management - FFS: the RE pattern for an X-port CSI-RS resource when X>=8 - FFS: the number of component CSI-RS RE patterns for X>=8-port CSI-RS resources - Strive to minimize the possible pairs of (Y,Z) for the component CSI-RS RE patterns while considering configuration complexity and overhead Agreements: - Support at least CDM-2, CDM-4 and CDM-8 - FFS: Support of no CDM - FFS: Time-domain and/or frequency-domain CDM can be configured on or off - Consider CDM pattern design with at least the following aspects - Measurement target, e.g. CSI acquisition, beam management, fine time/frequency tracking - A CDM pattern cannot span over more than U OFDM symbols and V subcarriers, FFS the values of U and V - Full power utilization Agreements: - At least CSI-RS for CSI acquisition, NR supports CSI-RS density d RE/RB/port for x-port CSI-RS - Value(s) of d are at least d=1,1/2. - For d<1, PRB-level comb-type transmission is supported. - FFS whether offset value(s) can be the same or different across antenna ports - FFS on supporting d>1 in the consideration of use case, e.g., NZP CSI-RS for IMR. - FFS on the supported combinations of value(s) of x and d. Conclusions: - For Mobility - Following remaining issues need to be finalized in the next meeting - Properties of CSI-RS for RRM measurement for L3 mobility, e.g., - Timing of CSI-RS for RRM measurement for L3 mobility - Bandwidth of CSI-RS for RRM measurement for L3 mobility - Numerology of CSI-RS for RRM measurement for L3 mobility - The number of REs in the freq. domain of CSI-RS for RRM measurement for L3 mobility - Resource mapping and parameters of CSI-RS for RRM measurement for L3 mobility - Configuration of CSI-RS parameters for RRM measurement for L3 mobility - … Agreements: - Association between one or multiple occasions for SS block and a subset of RACH resources and/or subset of preamble indices is informed to UE by broadcast system information or known to UE or FFS dedicated signaling - FFS gNB can configure an association between CSI-RS for L3 mobility and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam Conclusions: - NR studies following beam management options for RACH procedure: - … - Reporting of DL TX beam indices in Msg3 - Note: reported beams could be SS block indices and/or CSI-RS beam indices of P-2 procedure performed during Msg2 - RACH procedure using an association between CSI-RS for L3 mobility and subset of RACH resources and/or subset of RACH preamble indices - Beam refinement procedure before Msg2 by utilizing CSI-RS for L3 mobility - … Agreements: - For CONNECTED mode RRM measurement for L3 mobility based on CSI-RS, NR supports following targets for CSI-RS design and configuration: - Design: - Reuse CSI-RS design for beam management as baseline - Identify additional requirements on the CSI-RS to support L3 mobility - Configuration: - Support measurement of a large number of beams - Minimize configuration overhead, reporting overhead, and UE complexity - FFS: Detailed signaling or format Agreements: - For Connected mode, CSI-RS are supported to be configured using at least dedicated RRC signaling for DL based RRM measurement for L3 mobility. - Note that signalling other than dedicated RRC signalling is not precluded Agreements: - Clarify previous RAN1 agreements on the RSRP definition for DL RRM measurements for L3 mobility as follows - Define SS block RSRP and CSI-RS RSRP as - SS block RSRP : measured RSRP from SSS - FFS additional use of PBCH-DMRS for measurement - CSI-RS RSRP : measured RSRP from CSI-RS in connected mode Agreements: - The time synchronization reference for a CSI-RS for L3 mobility is the frame/slot/symbol timing of a cell. - Note: The frame/slot/symbol timing of the cell can be obtained from an SS block - FFS: Note: timing synchronization between CSI-RS and SS block of the cell is assured. Timing synchronization refers to frame/slot/symbol timing. - NR cell ID for time reference of CSI-RS(s) is informed to the UE

- References

- 3GPP, Study on new radio access technology physical layer aspects, TR 38.802 V14.0.0

- 3GPP, RAN #85, Chairman’s Notes

- 3GPP, RAN1 #86, Chairman’s Notes

- 3GPP, RAN1 #86bis, Chairman’s Notes

- 3GPP, RAN1 #87, Chairman’s Notes

- 3GPP, RAN1 NR-AH, Chairman’s Notes

- 3GPP, RAN1 #88, Chairman’s Notes

- 3GPP, RAN1 #88bis, Chairman’s Notes