3GPP TSG RAN WG1#89 																	R1-1708236

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:		7.1.1.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Paging in multi-beam system

Document for:		Discussion and Decision

# 1	Introduction

In RAN1#88, the following agreements were made pertaining to paging channel design [1]:

Agreements:

- Support the paging channel design at least for RRC idle mode as follows:

  - Paging message is scheduled by DCI carried by NR-PDCCH and is transmitted in the associated NR-PDSCH

  - FFS:

    - Paging indication triggers UE beam reporting (if supported)

      - Opt-1: paging indication is in DCI

      - Opt-2: paging indication is in non-scheduled physical channel

    - How to indicate SI update if it is supported in paging

RAN2 sent also a reply LS in [2]:

RAN2 would like to thank RAN1 for the LS on paging requirements. RAN2 has discussed the questions asked by RAN1 and would like to provide the following RAN2 inputs:

Q1: The minimum content size for a single paging message?

A1: In LTE, the minimum content size for a single paging message is about 48bits with octet alignment when single UE is paged with S-TMSI. Note in LTE, multiple UEs (up to 16) can be paged within the same message. If paging is used for other purpose only (e.g. notification of system information modification without paging any specific UE), it can be about 8bits with octet alignment.

Q2: The requirement on the number of UEs that need to be able to be paged within a certain time period and the scenario?

A2: In LTE, the number of UEs that can be paged per paging DRX cycle can be configured by the network. With the most aggressive network configuration, the number of UEs that can be paged per second is up to 6400.

RAN2 assumes that the above information based on LTE could be used for initial NR design but the design is not finalized for NR and some new aspects (need to support paging for the UE in inactive state, UE ID size to be decided by SA/CT WGs, or size of e.g. modification notifications, etc.) might impact the numbers above.

In this contribution we discuss the paging concept for NR.

# 2	Discussion

## 2.1	Content size of paging message

Considering some example calculations for the number of symbols required to convey certain paging message size, Table 1 provides examples for above 6GHz case. The Table 1 depicts, the required number of symbols in time domain for certain sized paging messages with QPSK modulation with coding rates of 1/6 and 1/12 when system BW of 80MHz and SCS 120kHz is used.

Table 1: Number of OFDM symbols required per paging message size

| Paging message bits/CR | “1/6” | “1/12” |
| --- | --- | --- |
| 8 bits (SI change/PWS indication) | 1 symbol | 1 symbol |
| 160 bits / ~4 UEs | 2 symbols | 3 symbols |
| 400 bits / ~10 UEs | 3 symbols | 5 symbols |

As can be seen from Table 1, the required number of time domain symbols for paging message (assuming the full BW is used) varies in the function of paging message size and used coding rate. Hence, it would be beneficial to be able to adapt the time domain length of the paging message flexibly. Indeed, this could be beneficial with multi-beam system as the same paging content needs to be swept to each direction.

Observation 1: Multi-beam system would benefit of being able to adapt flexibly the time domain length of the paging message.

## 2.2	Paging design in multi-beam cells

For multi-beam cells where not all the beam directions can be served simultaneously, beam sweeping needs to be applied for the PCH (Paging Channel). Considering the requirements on the paging channel bitrate and possible payload sizes of single paging message defined above, it is beneficial to be able to adapt the paging message size. Using normal slot lengths, however, to sweep through all the beams in a cell would take unnecessary amount of time and resources and is hence not feasible in practice.

One possibility is to use the concept of mini-slots for paging transmissions and hence shorten the sweep considerably if the paging message size was not too large as presented above. This could be applied in conjunction of the solution for single beam cells when the number of beams in a cell is fairly low and the mini-slot “sweep” length would hence fit within the defined PO limits (e.g., within a slot). This is illustrated in 1. However, when there is bigger number of beams in the cell, such option might considerably increase the power consumption of the UEs listening to PCH (Paging Channel) as the sweep length would not be deterministic – when the paging message size is large, the paging sweep will last longer spanning over multiple slots than when the paging message size is small, or even when it is non-existing.

Figure 1. Mini-slot based DCI/PCH sweep

Thus, generally for the multi-beam system, the scheduling information provisioning for the PCH should at least be made deterministic based on which the UE could determine the corresponding PCH transmission resource on the NR-PDSCH. For this purpose, a separate scheduling block sweep with NR-PDCCH could be introduced, like illustrated in Figure 2. NR-PDCCH is swept for the purpose of paging where a DCI is carried scheduling the PCH on NR-PDSCH after the scheduling sweep. Each scheduling block in the sweep could have association to an SS-block in the cell and hence the UE could determine the scheduling block and its transmission timing based on the SS-block timing in the SS sweep and the given paging configuration in the system information.

Figure 2. NR-PDCCH sweep for paging

On the other hand, in principle there does not need to be dependency between SS-block and scheduling block and should be discussed whether any advantages can be achieved. For the PCH sweep carried over NR-PDSCH, mini-slot concept could be exploited to accommodate also the fairly small paging message sizes not wasting system resources in time domain with overly long sweep (e.g., by applying always a full slot for each direction). Together this sweep control channel with NR-PDCCH and PCH sweep over NR-PDSCH would form a single PO.

Generally, the sweep control channel as well as the PCH sweep should be able to be combined from block to block to ensure maximal reception reliability by the UEs.

This is illustrated in the Figure 3.

Figure 3. Paging in multi-beam cell with multiple PO sweep bursts within one PO.

As can be seen from Figure 3, the PCH sweep for the PO #2 is longer than for PO #0 as there could be more UE IDs to be paged matching the PO #2. However, the control channel sweep is of the same size and thus deterministic for the UE to receive with maximal power saving. It should be noted there may not be anything to be carried in the paging message, in that case the NW should be allowed to schedule normal UL/DL slot allocation to replace both, the control channel sweep as well as PCH sweep.

Proposal 1: Separate sweep control channel carrying DCI is defined to schedule variable sized PCH sweep.

Proposal 2: Sweep control channel as well as PCH sweep shall be able to be combined from block to block in the receiver.

As in LTE, system information – rather minimum system information – can be used to configure the UEs to listen to the paging through sweep control channel/PCH sweep as well as how frequently such sweep POs exist. Such information is not required before the full minimum system information is determined by the UE, hence, the PBCH/MIB does not need to convey any information pertaining to paging reception.

Proposal 3: PBCH does not carry information pertaining to paging reception by the UEs.

As proposed in, e.g., [2], the sweep control channel could be considered to be multiplexed with the SS-block in FDM manner to save additional overhead. However, the overhead reduction would primarily be needed for the systems operating with narrow carrier bandwidths but most likely the FD multiplexing with SS-block is not possible there. Also, the minimum UE bandwidth aspects need also to be taken into account when such optimization is discussed. Furthermore, with such scheme the number of POs would be limited by the SS burst set periodicity which might not always suffice for the amount of paging load in the cell. Thus, for simplicity, RAN1 should primarily consider the separate sweep to be used for the sweep control channel before considering possible optimization by multiplexing with SS-blocks for systems operating with enough wide carrier bandwidths. In any case, NW should have means to configure the sweep control channel to a separate sweep or to the SS-sweep (or even both).

Proposal 4: Primarily, separate sweep in time domain is considered for the sweep control channel design in RAN1 and the configurable multiplexing with SS-block can be considered to be used if notable performance gain can be achieved.

On the other hand, one option to consider is also whether to align the POs with the system information transmissions as they need also to be swept over all the beams in the cell. This approach might better exploit the multiplexing gain in frequency and time domain (as muxing with SS-block) as both the paging message and system information message scheduling can be adapted flexibly.

Proposal 5: Discuss whether to align POs with system information transmissions when such is possible.

# 3	Response-driven paging mechanism

Like discussed in previous section there is a need to support different mechanism to allow feasible support of DL based paging in multi-beam deployments. It has been discussed in RAN1 [3] and also considered in RAN2 [4] to use response driven paging mechanism to reduce the DL radio resource overhead in multi-beam deployments at higher frequency bands. In this section, we consider the RAN1 related options to support paging indicator mechanism.

In response-driven paging indicator mechanism, instead of sending a paging message containing the UE IDs (e.g. S-TMSI), UEs are assigned to groups, Paging Groups (PGs), for which UE monitors in PO. To page an UE, network sends the paging indicator and as a response all the UEs monitoring the same paging indicator will perform UL access..

## 3.1 Paging group indicator

For transmitting the paging indicator in DL there are basically two options. As network needs to be able to page multiple PGs in a single PO, it would seem most straight forward that the said indication is carried in a form of a bit map, where each bit corresponds to a single PG. The required number of PG’s per PO determine the number of bits required.

In [4] we have presented evaluation of the impact of different assumptions to the DL and UL resource overhead. From the results, it can be concluded that increasing the number of the PGs that can be supported per PO, reduces the number of UEs that can be expected to perform a UL access based on the DL indication. Of course in order to increase the number of PGs that can be supported, larger space is required for the PG indication, resulting more DL overhead. In [4] up to 32 bit long bitmap was considered for PG indication. Bitmap based indication (is less than payload considered for NR-PBCH) of this size could be carried feasibly over NR-PDSCH. Other option is to deliver the bitmap with NR-PDCCH (on a related format). The NR-PDCCH design has not yet been concluded, but depending on the assumed aggregation level number of bits that can be carried would vary e.g. between 40 to 60 bits (+16 bit CRC). Hence it would seem possible to carry the PG identification on NR-PDCCH.

Observation 2. It would seem possible to carry the bitmap based paging group indication on NR-PDSCH or NR-PDCCH.

Other alternative would be that each PG is given an identity, not very different of what is considered for normal paging. Also in order to page multiple groups in this case, use of NR-PDSCH to carry multiple (individual) PG identifiers would be required to be used, it would not seem feasible to fit the required payload to NR-PDCCH. As noted above, if the number of PGs would be in order of 25 = 32 (i.e. 5 bits for individual PG), the payload size required would start to approach the same level as in normal paging, reducing the obtainable DL resource overhead saving.

Observation 3. Using individual identity to deliver the PG index would reduce the DL resource savings.

## 3.2 UE response

As noted above, in response-driven paging mechanism it is assumed that UE will provide a response to the network to indicate its presence in case a positive match is found between the received PG indication and the group UE belongs to. To provide the response two principle approaches can be identified:

Firstly, UEs can initiate the UL access procedure as normally and convey their user identity (e.g. S-TMSI) as a part of it (e.g. Msg#3). Then based on the obtained identities, network can respond by a normal RACH response with an UL grant to those UEs whom are paged. For those UEs that belonged to the same group but whose identity did not correspond to the intended paging message, could get a false paging indication, or no response at all.

Other alternative to provide the response, would be to assign dedicated PRACH preamble sequence(s) on RACH occasion for each PG identity and based on detecting a transmission on a said preamble sequence, network could transmit the paging message(s) over PCH as normally on the beams where a preamble was received. Only UEs belonging to the intended PGs would need to monitor the PCH.

These different approaches differ from the distribution of resource utilization in DL and UL. Using normal RACH procedure, as a response to the PG indication, to identify exactly presence of the UEs would require less resource utilization in DL, while the UL access resource utilization would be increased. The UL resource utilization of the normal RACH procedure based response mechanism could be alleviated by increasing the number of PGs. As discussed in Section 3.1, rather large number of PG indexes could be provided in each PO for example by NR-PDCCH.

Using dedicated preamble sequences to detect a presence of the UE(s) belonging to certain group(s) would reduce the overall UL resource utilization. From DL resource utilization perspective, this could result transmitting the paging message needlessly to several beams. To reduce the DL load (in terms of sending PCH needlessly to beams) the number of groups could be increased, but this would increase the amount of resources needed in the UL.

Observation 4: Further analysis is required to study the trade-offs between dedicated preamble and normal RACH procedure.

# 4	Conclusions

In this contribution we discussed the paging design for NR system operating with single/sector beam or multi-beam modes. Based on the discussion, we made the following observations and proposals:

Observation 1: Multi-beam system would benefit of being able to adapt flexibly the time domain length of the paging message.

Proposal 1: Separate sweep control channel carrying DCI is defined to schedule variable sized PCH sweep.

Proposal 2: Sweep control channel as well as PCH sweep shall be able to be combined from block to block in the receiver.

Proposal 3: PBCH does not carry information pertaining to paging reception by the UEs.

Proposal 4: Primarily, separate sweep in time domain is considered for the sweep control channel design in RAN1 and the configurable multiplexing with SS-block can be considered to be used if notable performance gain can be achieved.

Proposal 5: Discuss whether to align POs with system information transmissions when such is possible.

In addition in Section we give some initial considerations regarding the response-driven paging mechanism as follows:-

Observation 2. It would seem possible to carry the bitmap based paging group indication on NR-PDSCH or NR-PDCCH.

Observation 3. Using individual identity to deliver the PG index would reduce the DL resource savings.

Observation 4: Further analysis is required to study the trade-offs between dedicated preamble and normal RACH procedure.

# References

- RAN1#88 Chairman Notes

- R1-1700102, Paging in NR, ZTE, ZTE Microelectronics

- R1-1705570, ‘Paging design consideration’, Qualcomm Incorporated

- R2-17xxxxx, ‘Evaluation of Uplink Access using Paging Indicators’, Nokia, Alcatel-Lucent Shanghai Bell