3GPP TSG-RAN WG1 #89 	R1-1708282

May 15th – 19th, 2017

Hangzhou, P. R. China

Agenda item:	7.1.3.2.2

Source: 	Apple Inc.

Title: 	On Channel Design for Long PUCCH

Document for:	Discussion/Decision

# Introduction

During the study item phase of Rel 15 NR, several important agreements have been made on the physical uplink control channel (PUCCH) for NR. It was agreed that NR will support two categories of PUCCH: the long format and the short format. The general consensus is that the long format is most suitable to support the link budget limited scenarios for better cell coverage as well as larger UCI payload, while the short format is mainly targeted for time critical UCI feedback with limited UCI payload.

Based on such design philosophy, it was agreed that the long PUCCH format will support low PAPR DFT-S-OFDM waveforms, frequency diversity (e.g. intra-slot frequency hopping), as well as transmit diversity, as shown in the following agreements:

Agreements: [1]

- For PUCCH in long-duration,

  - Long UL-part of a slot can be used for transmission of PUCCH in long-duration.

    - i.e., PUCCH in long-duration is supported for both UL-only slot and a slot with the number of uplink symbols greater than X (X >= 2).

      - FFS exact value of X

  - In addition to simultaneous PUCCH-PUSCH transmission, UCI on PUSCH is supported.

  - Intra-slot frequency-hopping is supported

Agreements: [1]

- For further discussion of PUCCH in short-duration, UCI payload of 1 – at least a few tens of bits (or SR) is assumed.

- For further discussion of PUCCH in long-duration, UCI payload of 1 – at least a few hundreds of bits (or SR) is assumed.

- For PUCCH in long-duration, DFT-s-OFDM waveform is supported.

- For PUCCH in long-duration, transmit antenna diversity is supported.

Specific to the long PUCCH format, it was also agreed that the design should take into account the fact that NR supports variable slot structure

Agreements [2]:

- For long duration NR-PUCCH in a given slot, FFS the detailed NR PUCCH formats. Companies are encouraged to provide the corresponding details.

  - Some examples as a starting point:

    - For small UCI payload with 1 or 2 bit(s), LTE PUCCH 1a/1b especially in light of # of symbols available for NR-PUCCH

      - FFS: Time domain OCC is applied over allocated multiple symbols.

    - For large UCI payload with X bits, LTE PUCCH format 4, or PUSCH

      - FFS on applicability of (virtual) frequency domain OCC

    - FFS for the value of X

    - FFS for medium UCI payload with less than X bits

    - Scalability of NR-PUCCH for different number of symbols available for NR-PUCCH

- The set of the number of symbols for long duration NR-PUCCH in a slot includes {4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14}

  - FFS whether or not it depends on the slot type, # of symbols per slot, etc

In general, NR has more flexibility than LTE regarding the frame/slot structure, i.e. the network can configure PUCCH in different UL slot format including UL-only, UL-centric slots, or even mini-slots with variable lengths. However, this also makes it very challenging to provide a simple yet scalable channel design for NR, as well as take into account other design guidelines such as low PAPR, frequency and transmit diversity, etc.

In this contribution, we provide some thoughts on how to minimize the design/specification efforts in the long PUCCH channel design, while still supporting the very flexible NR slot formats.

# Long PUCCH formats in 14 symbol slots

The PUCCH formats are closely related to the various UL slots/mini-slots format to be supported. On one hand, supporting all possible values from {4, 5, 6, …, 14} gives full flexibility. On the other hand, it also makes specifications, as well as testing, unnecessarily complicated with diminishing return. Therefore, it is desirable to down select a few most typical long PUCCH format lengths from the set {4, 5, 6, …, 14}, at least for the first NR release.

The most common slot format in NR contains 14 symbols, although 7-symbol slot can also supported for SCS below 60kHz [3]. It is expected that the most common use case would be the 14-symbol UL-only slot and 14-symbol UL-centric slot, as shown in Figure 1 (a) and Figure 1 (b), respectively.

Figure 1 UL slots with 14 symbols

## UL-Only Slots

The simplest case is the UL-only slot with 14 symbols. In order to maximize link budget, while provide intra-slot hopping, a simple design would be leveraging existing LTE PUCCH format with two “7-symbol modules”. Each module has identical structure in terms of reference signal and data, such that the two parts can have independent channel estimation and support frequency-hopping diversity. Generally, for a few UCI bits requires roughly 50% pilot overhead ratio, while much larger UCI payload size require much lower pilot overhead ratio. Those can be supported, as an example, by the following structure

Figure 2 Example format of the 7-symbol PUCCH module

Specifically, both structures in Figure 2 contain even number of data symbols, which leads to easy support of STBC for transmit diversity while still maintaining the single carrier property of DFT-S-OFDM.

The detailed RS and data distribution within each 7-symbol module can be further studied and agreed within RAN1 to cover different payload sizes.

## UL-Centric Slots

For the UL-centric slot, a likely structure would be 1~3 DL symbols for DCI, a DL-UL switching gap with length of roughly 1 symbol, which leaves roughly 10~12 UL symbols within each slot, as shown in Figure 1 (a). Therefore, a potential PUCCH format could consist of two 5-symbol modules or two 6-symbol modules. Similarly, the two modules within the same slot could have identical structure in terms of reference signal and data, such that the two parts can have independent channel estimation and support frequency-hopping diversity. Examples of such structures are shown in Figure 3 and Figure 4.

Figure 3 Example format of the 5-symbol PUCCH module

Figure 4 Example format of a 6-symbol PUCCH module

Similar as the 7-symbol module, the structures in Figure 3 and Figure 4 contain even number of data symbols, which allow easy support of STBC for transmit diversity while still maintaining the single carrier property of DFT-S-OFDM waveform.

The detailed RS pattern and data multiplexing within each module should be further studied and agreed within RAN1.

Proposal 1: To support long PUCCH format within 14-symbol slots, NR should consider supporting intra-slot hopping with a modularized design using 5-symbols, 6-symbols, and 7-symbols PUCCH modules.

# Long PUCCH formats in other slot structure

It was agreed that NR can also support 7-symbol slot structure for SCS below 60kHz, as well as mini-slots with various lengths. The 7-symbol slot can further includes UL-centric and UL-only slot structures as shown in Figure 5.

Figure 5 UL slots with 7 symbols

For the 7-symbol UL-only slot, the corresponding PUCCH can directly reuse the 7-symbol module as defined for the 14-symbol case.

For the 7-symbol UL-centric slot, due to the shorter slot length, it is reasonable to expect that DCI only span 1 symbol. Considering the extra UL-DL switching gap, this leaves about 5 symbols for the long PUCCH format. In this case, the 5-symbol module as defined for the 14 symbol case in section 2 can be leveraged.

To extend coverage to uplink mini-slots, an additional 4-symbol format might be introduced, which can also be used for intra-slot hopping in case a 8 symbol PUCCH format is needed.

In case uplink mini slots with length 9, 11, 13 symbols are to be used, the simplest approach is to use the long PUCCH formats that are 1-symbol shorter and still composed of the 4-symbol, 5-symbol and 6-symbol module.

Proposal 2: To support long PUCCH in other uplink slot structures, including 7-symbol slot and mini-slots, the same 5-symbol, 6-symbol, 7-symbol PUCCH modules can be reused, with the possible extension of 4-symbol module.

# Conclusions

In this contribution, we provide some thoughts on how to minimize the NR design/specification complexity on the long PUCCH format, so as to support the flexible NR slots structure, as well as supporting the agreed features such as frequency diversity and transmit diversity. Specifically, we have the following proposals:

Proposal 1: To support long PUCCH format within 14-symbol slots, NR should consider supporting intra-slot hopping with a modularized design using 5-symbols, 6-symbols, and 7-symbols PUCCH modules.

Proposal 2: To support long PUCCH in other uplink slot structures, including 7-symbol slot and mini-slots, the same 5-symbol, 6-symbol, 7-symbol PUCCH modules can be reused, with the possible extension of 4-symbol module.

# References

- 3GPP RAN1 NR AdHoc Chairman Notes, “Chairman’s Notes RAN1_NRadhoc_final”, January 16th-20th, 2017,  Spokane, USA.

- 3GPP RAN1 #88-bis Chairman Notes, “Chairman’s Notes RAN1_88bis_final”, April 3rd-7th, 2017,  Spokane, USA.

- 3GPP TR 38.802 V2.0.0

- 3GPP TR 38.913 “Study on scenarios and Requirements for next generation access technologies”.