3GPP TSG RAN WG1 Meeting #89                                  	    R1-1708351

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:	7.1.3.2.2

Source:	InterDigital Inc.

Title:	On transmit diversity for Long PUCCH

Document for:	Discussion and Decision

# Introduction

In RAN1#88bis meeting [1], transmit diversity for PUCCH was discussed and the following agreements were made for long-PUCCH with DFT-s-OFDM:

Agreements:

- For DFTsOFDM in long-PUCCH, the following schemes are candidates for transmit diversity:

  - Low PAPR Alamouti-based transmit diversity applied in frequency or time domain, transparent transmit diversity (e.g. short delay CDD), time domain beam/precoder cycling or SORTD

  - FFS: for which PUCCH format and/or payload size

  - Other schemes with low PAPR are not precluded.

- Companies proposing a certain transmit diversity scheme are encouraged to jointly propose PUCCH structure and the transmit diversity scheme.

It is a well-established fact that among the diversity scheme candidates, non-transparnet transmit diversity schemes such as space time block coding (STBC) and space time frequnecy coding (SFBC) are the ones with the best link-level performance. Both of these schemes, however, present challenges: STBC needs an even number of DFT-s-OFDM symbols and SFBC breaks the single carrier property due to the re-ordering of the frequency domain symbols. The schemes proposed in [2] and [3] aim to address these challenges.

In this contribution, we discuss two candidate diversity schemes for DFT-s-OFDM. One is an STBC scheme where diversity coding is applied over the sub-symbols of a single DFT-s-OFDM symbol wherein each sub-symbol is equipped with its own internal cyclic prefixes (CP). The other is an SFBC scheme using a multiple-DFT precoding. Both schemes are qualitatively compared to those in [2] and [3] and conclusions are drawn.

# STBC with one DFT-s-OFDM Symbol

Space time block coding can be applied over an even number of DFT-s-OFDM symbols. When the number of symbols is odd, STBC can be applied on sub-symbols created within a single DFT-s-OFDM symbol [2]. Due to the single carrier nature of DFT-s-OFDM, the sub-symbols that enter the DFT block and come out at the output of the IDFT are subject to an oversampling operation with the oversampling ratio of the IDFT size to the DFT size. Since each sub-symbol is subject to DFT separately at the receiver, the following types of inter-symbol interference (ISI) and inter-carrier interference (ICI) may occur:

- ISI due to interpolation: The samples at the output of the IDFT would have contributions from the other samples at the output of the IDFT. The contribution of the samples to each other decreases when the distance between them increases.

- ISI due to the channel: The sub-symbols would leak into each other as a result of the channel delay spread.

- ICI due to the loss of cyclic convolution: Using CP attachment to the whole symbol does not guarantee the cyclic property for each sub-symbol. Therefore, the loss of cyclic convolution per sub-symbol results in inter-carrier interference (ICI).

To prevent non-negligible performance loss due to ISI and ICI, each sub-symbol may have its own internal CP as illustrated in Figure 1 for the case of two DFT-s-OFDM sub-symbols transmitted from two antenna ports. Although the internal CPs may result in slightly higher overhead, since they are created within a data block, it would be relatively easy to adapt the CP length without any changes in the transmitter.

Figure 1 STBC for a single DFT-s-OFDM symbol

For the case of two transmit antenna ports, the receiver may work as follows: Assume that the two halves of the received DFT-s-OFDM symbol corresponding to the two half sub-symbols are denoted

r1n= h1n⊗s1tn+h2n⊗s2t*-n

r2n= h1n⊗s2tn-h2n⊗s1t*-n

where ⊗ denotes the circular convolution operator that is made possible due to the internal CPs. Note that, in general, due to the interpolation, the cyclic prefixes in s1t and s2t may not be exact; however, the ISI is expected to be very small and we can assume cyclic convolution with the channel.

After ignoring the internal CPs and passing each one sub-symbol through a DFT, we get

R1k= H1kS1k+H2kS2*k

R2k= H1kS2k-H2kS1*k

where k is the subcarrier index. Multiplying the sub-symbols R1k and R2k with H*1k and H*2k,

R1kH1*(k)=H1kS1kH1*(k)+H2kS2*kH1*(k)

R1kH2*=H1kS1kH2*(k)+H2kS2*kH2*(k)

R2kH1*(k)=H1kS2kH1*(k)-H2kS1*kH1*(k)

R2kH2*(k)=H1kS2kH2*k-H2kS1*kH2*(k)

Diversity combining can now be achieved by

R1kH1*k-R2kH2*(k)*=  H1k2+H2k2S1(k)

R1kH2*k+R2kH1*(k)*=H1k2+H2k2S2(k)

# SFBC with one DFT-s-OFDM Symbol

Another technique to achieve transmit diversity is space frequency block coding which was extensively used in LTE. To apply SFBC on DFT-s-OFDM, the output of the DFT in the second antenna port needs to be re-ordered before being mapped to the subcarriers. The method in [3] performs the re-ordering by employing circular rotation, conjugation and negation of every other symbol. Since these operations preserve the single carrier property, PAPR is not increased.

In this method, SFBC is applied on a pair of symbols transmitted on non-adjacent subcarriers. In general, with DFT size of M, the largest separation between subcarrier pairs would be M/2-1 i.e., as the data block size grows, the maximum distance in a given subcarrier pair used for SFBC grows. As a result, the channel response experienced on the paired subcarriers may be significantly different, resulting in potential loss of performance when M is large or the channel delay spread is high. One method to prevent or reduce the performance loss is to decompose the input data into multiple blocks and applying a separate DFT to each data block as shown in Figure 2. If SFBC is now performed over blocks of M/2 subcarriers, the maximum subcarrier pair separation would be reduced to M/4-1 from M/2-1 I.

Figure 2 SFBC with multiple DFT blocks

One of the key issues associated with multiple DFT blocks in a single DFT-s-OFDM symbol described above is a relative loss of single carrier property which will result in the slight increase in PAPR. An alternative solution to maintain single carrier property of the signal while achieving transmit diversity is using a larger number of antennas at the UE side and then apply a group antenna-based DFT spreading and SFBC. For example with four transmit antennas at the UE, each sub-sequence can be transmitted over a group of two antennas after performing DFT spreading and SFBC on each sub-sequence.

# Summary

Table 1 compares the two diversity schemes discussed above with the schemes proposed in [2] & [3].

Table 1 Comparison of diversity schemes

|  | STBC [2] | STBC with internal CP | SFBC [3] | SFBC with multiple DFT blocks |
| --- | --- | --- | --- | --- |
| ISI | High | Negligible | No | No |
| PAPR | Low | Low | Low | 4Tx: Low 2Tx: Slightly higher |
| Cyclic convolution | Not preserved | Preserved | Preserved | Preserved |
| Channel variation over subcarrier pairs | Low | Low | Higher | Lower than [3] |
| Overhead | Low | Slightly higher | Low | Low |

Based on the above discussion we propose:

Proposal: Consider sub-symbol STBC and SFBC with multiple DFT blocks for further study for PUCCH transmit diversity with DFT-s-OFDM waveform.

# References

RAN1#88bis Chairman’s Notes, Spokane, USA, 3rd – 7th April 2017

R1-1705578, “On UL diversity transmission scheme,” Qualcomm, RAN1#88bis, Spokane, USA, April 2017.

R1-1704813, “Transmit diversity for PUCCH in long duration,” Mitsubishi Electric, RAN1#88bis, Spokane, USA, April 2017.