3GPP TSG-RAN WG1 #89 	R1-1708634

May 15th – 19th, 2017

Hangzhou, P. R. China

Agenda item:	7.1.3.3.6

Source: 	Qualcomm Incorporated

Title: 	URLLC DL control and indication channel

Document for:	Discussion/Decision

# Introduction

In RAN #71, the technology study item for 5G new RAT (NR) has been approved [1]. URLLC (ultra-reliable low latency communication) requirements has been discussed in RAN plenary in June 2016. In RAN1#86bis and RAN1#87 and #88 the following agreements were made regarding URLLC and eMBB multiplexing and mini-slot designs:

Agreements:

- From network perspective, multiplexing of transmissions with different latency and/or reliability requirements for eMBB/URLLC in DL is supported by

  - Using the same sub-carrier spacing with the same CP overhead

    - FFS: different CP overhead

  - Using different sub-carrier spacing

    - FFS: CP overhead

  - NR supports both approaches by specification

NR should support dynamic resource sharing between different latency and/or reliability requirements for eMBB/URLLC in DL

Agreements:

- For DL, dynamic resource sharing between URLLC and eMBB is supported by transmitting URLLC scheduled traffic

URLLC transmission may occur in resources scheduled for ongoing eMBB traffic

Agreements:

- For DL, support indication of time and/or frequency region of impacted eMBB resources to respective eMBB UE(s)

  - FFS: Details of  the granularity for impacted region used in the indication

    - e.g., PRB (group)/symbol (group)/mini-slot (group)/CB (group)/TB/Slot

  - The indication is transmitted at one of the following (will be down selected later)

    - during current eMBB TTI

    - after current eMBB TTI

    - during  and after current eMBB TTI

  - The indication is one of the following (will be down selected later)

    - explicit

    - implicit

    - explicit and implicit

Agreements:

- No new physical channel specific for indication of DL resources being preempted by another DL transmission is introduced

  - FFS whether the indication is based on NR-PDCCH or a group common PDCCH

  - FFS location of the indication

  - FFS timing of the indication

In this contribution, we discuss the concrete indication channel design for URLLC/eMBB downlink dynamic multiplexing.

# Indication channelization design

Dynamic resource sharing between URLLC and eMBB (such as pre-emption, puncturing or superposition) is an essential feature that NR has adopted to serve different service types of very different latency, reliability and efficiency requirements in order to meet 5G URLLC and eMBB requirements respectively. Static/semi-static resource partitioning between URLLC and eMBB, e.g., through FDM-ing between URLLC and eMBB, would lead to low system efficiency as we showed in previous contributions [4]. Alternatively, dynamic eMBB and URLLC multiplexing can be performed at a mini-slot granularity to get the most efficient multiplexing scheme. For example, it is shown that 2/4-symbol mini-slot of 60kHz SCS provides significant capacity gain in URLLC compared with slot based URLLC scheduling in previous contributions [3], [4]. To better serve URLLC design in both the first and future releases, additional control signaling is desired to indicate where URLLC pre-emption happens.

As shown by many companies, the indication-based multiplexing approach is beneficial for both URLLC and eMBB UEs at the cost of indicator overhead. As discussed in our previous contributions [6], the current and/or post indication schemes should be considered. See Figure 1&2 for the examples of the current and post indication, respectively.

Figure 1. Scheme of current-indication of preemption: the mini-slot PDCCH carries an indicator.

(a)

(b)

Figure 2. Schemes of post-indication of preemption: (a) post-indication occurs at the last symbol of the current slot; (b) post-indication occurs at the control channel of the next slot.

In particular, the current indication draws much attention due to its low processing latency. The current indication could be per mini-slot or per a few mini-slots. The indication periodicity could be configured semi-statically (via RRC) or dynamically (via PDCCH). In what follows, we discuss two major formats of indication channel based on its allocation w.r.t. the eMBB data: separated from or embedded in eMBB data.

Proposal 1: Indication channel for URLLC/eMBB DL multiplexing could be separated from or embedded in eMBB data.

## Separate indication from eMBB data

We consider an independent indication channel separated from eMBB data. The information carried in this indication channel could be wide-band based, sub-band based, RB-based or UE specific. If wide band or sub-band based, this indication applies to all UEs in the wideband or a set of sub-bands. The payload can be small due to its broadcast or multicast nature. However, a positive indication will influence all RBs even though some of them are not used/punctured by URLLC UEs, leading to resource waste and performance degradation of eMBB UEs. If RB-based, the indication indicates puncturing on per RB or per RB group basis, e.g, every 4 RBs. For a wideband NR, however, the payload of this indicator can be large. If UE-specific based, the indication indicates puncturing on per UE basis. Clearly, this requires very large payload if a large number of eMBB UEs are scheduled.

Observation 1: Indication channel could indicate wide-band based, sub-band based, RB-based or UE specific puncturing information. FFS the tradeoff between performance and payload of each scheme.

One indication channel structure can be the group-common-control type of structures at slot level and applying it to the mini-slot level. In particular, we propose PCFICH-type structure.  In this scheme, group common PDCCH can be used in every mini-slot or every a few mini-slots, according to the configurations. The reference signal can be shared among group common PDCCH within a slot.

Observation 2: Indication channel structure could follow the PCFICH-type structure at the mini-slot level.

## Embedded indication in eMBB data

In this option, the indication channel is embedded in the eMBB data region. Typically, the indication channel could have comb-based structure, e.g, every 4 tones used for indication channel. When URLLC traffic is not present, indication is not sent to reduce indicating overhead. When URLLC traffic is present, the comb-based indication channel will also turn into DMRS for the corresponding URLLC UE. By doing so, URLLC data needs rate match around these indication channel (a.k.a. DMRS for URLLC data).  In every indication monitoring duration (one or more mini-slot(s)), eMBB UEs will try to decode the pre-assigned indication channel locations to see if the URLLC traffic is present.

Meanwhile, this indication channel also serves as DMRS for URLLC, hence it serves as dual purposes and the resource will be useful for both eMBB and URLLC UEs.

Observation 3: Embedded indication channel results in much less overhead. FFS approaches to reduce false alarm/misdetection rate in blind decoding.

Figure 5. Indication channel embedded in eMBB data

# Application to the Indication Channel Design for UL eMBB/URLLC multiplexing

The aforementioned indication schemes can directly apply to the indication channel for the UL URLLC/eMBB multiplexing. In particular, when UL URLLC traffic presents, a scheduling request (SR) is sent on a dedicated uplink resource. Upon receiving a SR, gNB will send UL URLLC grants on the current indication channel (e.g. PCFICH-type structure burst) in the next DL mini-slot. Both eMBB UEs (if transmitting on UL) and URLLC UEs could monitor this indication channel for the scheduling information. The on-going eMBB UL transmission will yield on the resources indicated by the UL URLLC. One can notice that, by doing so, we achieve a unified indication channel design for both UL and DL transmissions URLLC. One example is presented in Figure 5, where we assume an indication channel separated from the eMBB data and carried by the mini-slot PDCCH.

Figure 3. A unified indication channel design for UL URLLC/eMBB multiplexing

Proposal 2: A unified indication channel design for both DL and UL should be considered.

# Conclusion

Observation 1: Indication channel could indicate wide-band based, sub-band based, RB-based or UE specific puncturing information. FFS the tradeoff between performance and payload of each scheme.

Observation 2: Indication channel structure could follow the PCFICH-type structure at the mini-slot level.

Observation 3: Embedded indication channel results in much less overhead. FFS approaches to reduce false alarm/misdetection rate in blind decoding.

Proposal 1: Indication channel for URLLC/eMBB DL multiplexing could be separated from or embedded in eMBB data.

Proposal 2: A unified indication channel design for both DL and UL should be considered.

# References

- R1-160671, New SID Proposal: Study on New Radio Access Technology

- RAN1-87, chairman’s notes

- R1-166360, “URLLC numerology and frame structure design”, Qualcomm, 3GPP TSG-RAN WG1 #86

- R1-1612014, “Mini-slot design for URLLC”, Qualcomm , 3GPP TSG-RAN WG1 #87

- R1-1700828, “Simultaneous scheduling request and data transmission for uplink URLLC”, Qualcomm, 3GPP TSG-RAN WG1 NR Ad-hoc, Spokane, Washington, USA

- R1-700827, “URLLC/eMBB downlink dynamic multiplexing schemes”, Qualcomm , 3GPP TSG-RAN WG1 #87