3GPP TSG RAN WG1 Meeting #89                                     R1-1708835

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:		7.1.4.2.1.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Rate matching for Polar codes

Document for:		Discussion and Decision

# 1	Introduction

In RAN1 #88 meeting [1], the following agreement was made for eMBB control channels:

Working Assumption for UCI:

- Nmax,UCI =1024

  - Optimise code design for K up to 200

    - Also aim for code design that supports values of K up to 500 with good performance, typically using higher code rates

      - Without prejudice to the final design, companies are encouraged to investigate advanced code rate matching schemes until RAN1#88bis

- Working assumption can be revisited at RAN1#88bis if it does not prove to be possible to generate a good code design with Nmax,UCI =1024

Some details on rate matching were discussed in Ran1 #88bis meeting. In this contribution, we discuss possible schemes for rate matching focusing on the studies of shortening, puncturing, and repetition.

# 2	Puncturing/Shortening techniques

As polar codes are mainly constructed for code lengths that are the power of two, special attention is required to support in-between codeword lengths. In [2], different puncturing schemes are identified as follows (Type II Puncturing also known as Shortening).

Natural – Type I: Puncture code bits with indices {1,2,⋯,N-M}, i.e. the first (N-M) bit positions.

Natural – Type II: Puncture code bits with indices {M+1,M,⋯,N}, i.e. the last (N-M) bit positions.

Bit-reversed – Type I: Puncture code bits with indices given by bit reversing {1,2,⋯,N-M}.

Bit-reversed – Type II: Puncture code bits with indices given by bit reversing {M+1,M,⋯,N}.

Reliability- Type I: Puncture the indices corresponding to the least reliable bit-channel positions first.

Reliability- Type II: Puncture the indices corresponding to the most reliable bit-channel positions first.

Split-natural – Type I: Puncture the first N/4 bits naturally from bit index 1, and additional bits if needed are punctured alternately from N/4+1 and N/2+1.

Split-natural – Type II: Puncture the last N/4 bits in reverse order from bit index N and additional bits if needed are punctured alternately in reverse order from 3N/4 and N/2.

Based on their study, Split-Natural or Reliability-based puncturing method showed good performance. But, both Type I and Type II are needed depending on the code rate.

Moving forward, we should focus on the exact use cases of these puncturing schemes. At least for downlink, we expect fixed coded block sizes which are associated with the aggregation levels. Considering the latest agreement on CCE size which now contains 6 REGs, we could expect 96, 192, 384, and 768 bits for 1st, 2nd, 4th, and 8th aggregation levels, respectively. In such cases, puncturing patterns are fixed where we can define a given scheme which is performing better for that operating points. Otherwise, the puncturing scheme could be a sub-optimal scheme as it justified based on averaged observation across different coded block sizes.

For UL control, we can increase the use of mother codeword size, where natural rate becomes lower even for the larger payload sizes. As Type-1 is doing good for lower code rates, we could check the possibility of defining a single puncturing scheme. Selecting the puncturing scheme based on the code rate may require different treatments at the decoder and may not be a good solution. In general, we see that further investigations are required by closely following discussions in HARQ/scheduling to finalize the best scheme for shortening/puncturing.

# 3	Repetition techniques

From the working assumption, the maximum mother codeword size of uplink control channel is limited to1024. Polar code repetition must be done for rate matching when the target block size is greater than the underlying polar codeword.

It has been observed in 3GPP that an inappropriate design of polar code repetition would lead to inferior decoding performance compared to the polar coding without repetition. Therefore, a good design of polar code repetition for control channels is important.

A trivial scheme for polar code repetition is very similar to the circular buffer design of WCDMA and LTE turbo codes. The scheme repeats the bits from the very beginning of the encoded block depicted in Fig. 1. However, this trivial scheme does not have good decoding performance as we will see later in the simulation results.

Figure 1. Classical Arikan encoding graph example

In this section, we assume:

- u0, …,uK-1 are the information bits to be encoded;

- x0,…, xN-1 are the polar encoded bits;

- K is the total number of information bits, which includes CRC bits if CRC-aided polar coding is used;

- N=2n is the total number of coded bits, and the coded bits are obtained by encoding a (N, K) polar mother code;

- M is the total number of bits to be transmitted.

In this case, we need to select M-N bits from the N polar coded bits and concatenate them with the original length-N codeword to form the code block for transmission.

Here, we describe a repetition scheme based on a double-circular buffer structure, which is depicted in Fig. 2.

Figure 2. Double circular buffer

In Fig. 2, the N coded bits, x0,…, xN-1, are divided and put into two buffers. In the outer buffer, the coded bits are denoted by x0, … , xφ-1, and x0, … , xτ-1 refer to the bits in the inner buffer. φ and τ are the length of the outer buffer and the inner buffer, respectively. φ is not required to be equal to τ, and clearly, we have N=φ+τ. The operator ab means mod(a,b).

For a given cell in the outer buffer, it could contain any bit in x0, … , xφ-1. The selection of the exact xi, which is put into the given cell, is controlled by an offset (or displacement) value, and it is called disp1 in Fig. 2. Similar argument is applicable to the inner buffer, and the offset value is called disp2.

There are two steps in the process with the double circular buffer:

- Distribution: The first step is to distribute the coded bits into the two buffers.

- Spooling: the second step is to send out the coded bits from the double circular buffer.

At step-1, we need to distribute the N coded bits into two buffers, and in this contribution we sequentially put all coded bits which are not directly connected to the information bits in Fig. 3 into the outer buffer, and put the coded bits which are directly connected to the information bits into the inner buffer.

Figure 3. Encoding graph agreed in 3GPP

In Fig. 3, the red lines refer to the direct connection relationship, and in this case (x0, … , xφ-1)=(x0,x1,x2,x3,x6) and x0, … , xτ-1=(x4,x5,x7).

At step-2, when the distribution of coded bits is completed, spooling of the double circular buffer is applied to yield code repetition. The steps are:

- Determine the values of disp1 and disp2;

- Determine the starting point for buffer cell visiting, which can be anywhere in the outer buffer or the inner buffer;

- Determine the direction and way for visiting the buffer cells (clockwise, counter-clockwise, or others);

- Start from the starting point and keep reading out coded bits from the double buffer alongside the red line in Fig. 2, until M bits are collected. (The arrowhead in Fig. 2 is just for illustration purposes. In Fig. 2 it means clockwise direction, but it could also point to the opposite direction.)

The proposed scheme provides much flexibility for various applications. Here is the list of the degrees of freedom:

- Ways to distribute coded bits

- Offset values for the inner and outer buffers

- Starting point

- Direction

- Ways to visiting the cells in the buffers (sequential, zigzag, etc.)

And these factors can be finely tuned to further optimize the performance.

Simulation results are provided to show the performance gain of the proposed scheme against the most straightforward and turbo-like repetition scheme in Fig. 4.

For the double buffer based scheme, we choose disp1=0, disp2=0, counter-clockwise buffer visiting, and starting from xτ-1. For the turbo-like repetition scheme, we repeat the bits from the very beginning of the encoded block depicted in Fig. 1.

Figure 4. BLER performance comparison

Fig. 4 compares the BLER performance of the proposed scheme and the turbo-like scheme for the (N = 1024, K = 500) case, and the length-1024 codeword is repeated to fulfil M = 1500. The decoding method is CRC aided list decoding and 19 CRC bits are attached. The list size is 8. The proposed scheme has about 1 dB gain when the BLER reaches 10-3.

Proposal 1: Consider the double circular buffer structure for polar code repetition.

# 4	Conclusion

In this contribution, we discussed the rate matching schemes for polar codes.

Proposal 1: Consider the double circular buffer structure for polar code repetition.

# References

- Ran1 Chairman’s notes, Ran1 #88, Athens, Greece

- R1-1704317	“Rate Matching Scheme for Polar Codes”, Ericsson