3GPP TSG RAN WG1#89 					          	                 	   R1-1708908

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:		7.1.2.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Multi beam transmission of UL channels

Document for:		Discussion and Decision

# 1	Introduction

The new radio work item based on [1] was accepted in RAN#75 plenary.

In this contribution we discuss about methods against beam pair blocking for NR-PUCCH transmissions. As a starting point we consider the following agreement made in RAN1#88 [2]:

Agreement: - Study further multi-beam based NR-PUCCH transmission for robustness against beam pair link blocking - E.g., UE transmits NR-PUCCH on different UL Tx beams in different NR-PUCCH OFDM symbols - FFS: multi-beam triggering condition/mechanism (e.g. event-triggered, network configured, etc.) - FFS: number of OFDM symbols for each beam

In RAN1#88bis there was a way forward with the following proposals [3]:

- Support robust transmission on PUCCH via multiple UL beams. At least one of following methods is supported - Method 1: Event-triggered multi-beam based PUCCH transmission - Method 2: NW initiated multi-beam based PUCCH transmission - FFS: Each beam for PUCCH can configured in different symbols within a slot or symbols in different slots.

# 2	Discussion

To provide link robustness between UE and gNB for both downlink and uplink, there may be multiple beam pair links between UE and gNB to be kept tracked and aligned, where a beam pair link would comprise a UE TX beam and a gNB RX beam. There may be different situations when a certain beam pair link gets non-aligned:

Case 1: UE rotates but no blockage between UE and gNB RX beam

In this case, the gNB RX beam would remain the same but the UE may need to switch its TX beam or widen the TX beam in order to compensate the rotation effect and get the beam pair link aligned again. There is a downlink RS associated to gNB RX beam and the UE should keep its RX and TX aligned to the downlink RS. If UE cannot re-align, it shall consider the beam pair link in question out-of-synch. If the UE detects beam pair link out-of-synch it’s considered not feasible that the UE would try different UL TX beams for PUCCH transmissions but rather trigger beam recovery procedure. Reasoning are, as illustrated also in Figure 1:

- UE would introduce interference to the system

- gNB needs to pre-configure and pre-reserve multiple time domain resources introducing potentially significant system overhead, especially when gNB / TRPs operate using analog/hybrid beamforming architecture.

Figure 1 UE rotating and tries different TX beams.

Case 2: There is a blockage between UE and gNB RX beam

In this case both the UE TX and gNB RX beams would need to be changed in order to re-align the beam pair link or alternative beam pair link is to be used. In other words, the beam pair link of interest becomes blocked.

As said, apparent mechanism to provide robustness against blockages is beam pair diversity where UE may be configured with multiple NR-PUCCH resources each corresponding to a certain UE TX beam and gNB RX beam pair. Given the fact that both UE and gNB may have limited capabilities and be able to form one beam at a time, the resources for different beam pair links need to be time domain multiplexed.

Observation: If UE is configured with multiple NR-PUCCH resources for beam pair diversity, TDM multiplexing is needed for UE’s NR-PUCCH resources.

Then, do above Case 1 and Case 2 need to be separated when UE is configured multiple NR-PUCCH resources? It is assumed that UE derives UL TX power from DL RS which has an association to the gNB RX beam pair of the certain beam pair link. Thus, it’s considered that in configuration the UE needs to be indicated what will be gNB RX beam for the NR-PUCCH reception.

Observation: Configuration of NR-PUCCH resources for beam diversity needs to indicate the gNB RX beam and associated DL RS for UL TX power control.

The UL transmission diversity has some commonalities and differences from the DL case. In DL the idea is that different beams from same or different TRPs are used for multiple PDCCH transmission. This covers also the case when beam widths used for these transmission occasions are different, for example using a narrower beam for the primary PDCCH transmission and a wider beam for the secondary PDCCH transmission. We can see such transmission as a form of beam selection from a CoMP-type of transmission.

In UL there are a few options for the transmission, and in fact more importantly for the reception of the UL signals:

- the primary and secondary ULs can be received by the same TRP, on different gNB RX beams.

- the primary and secondary ULs can be received by different TRP, on different gNB RX beams.

- There can be a single UL transmission received by different TRP beams or different TRPs. In such a case the UE would utilize a wider beam for UL transmission. Such a scenario is not possible in downlink and resembles a form of UL CoMP.

The first 2 cases above can mirror the DL diversity operation where the two transmissions can be either TDMed or transmitted in the same time. The beam widths used for the of these transmission of such UL signals are UE implementation aspects and hence a narrower beam can be used for the primary UL transmission and a wider beam can be used for the secondary UL transmission. From the network operation perspective, only the reception of the UL diversity transmission in the same TRP does not involve cooperation between TRPs.

Proposal: Compare the reception of different UL transmissions in same TRP, different TRPs as well as the reception of a single UL transmission by multiple TRPs.

Then, how to trigger and/or configure transmission when UE has been configured multiple receive gNB beams (corresponding to multiple UE TX beams) NR-PUCCH transmissions. Two options can be considered:

- Semi-static time domain pattern similar to NR-PDCCH monitoring configuration

- gNB indicates in DCI, when it allocates dynamically NR-PUCCH resources to UE, that what is receive gNB beam (and QCL associated DL RS)

Since NR supports aperiodic, semi-persistent, and periodic CSI reporting, it’s considered that both options 1. and 2. are needed. In other words, option 1. could be used for all aperiodic, semi-persistent and periodic CSI-reporting, and option 2. could be for aperiodic CSI reporting. Rationale to have also option 2 is to have possibility for the gNB to balance dynamically load on beam resources.

Proposal: Support both semi-statically configured multi-beam NR-PUCCH transmissions as well as dynamic receive gNB beam allocation for NR-PUCCH.

# 3	Conclusions

In this contribution we discussed about providing robustness for NR-PUCCH in multi-beam scenario. Based on the discussion, the following observations and agreements were made:

Observation: If UE is configured with multiple NR-PUCCH resources for beam pair diversity, TDM multiplexing is needed for UE’s NR-PUCCH resources.

Observation: Configuration of NR-PUCCH resources for beam diversity needs to indicate the gNB RX beam and associated DL RS for UL TX power control.

Proposal: Compare the reception of different UL transmissions in same TRP, different TRPs as well as the reception of a single UL transmission by multiple TRPs.

Proposal: Support both semi-statically configured multi-beam NR-PUCCH transmissions as well as dynamic receive gNB beam allocation for NR-PUCCH.

# References

- 3GPP Work Item Description, “New WID on New Radio Access Technology”, March 2017.

- RAN1#88 Chairman notes

- R1-1706454, “WF on multi-beam based PUCCH transmission”, LG Electronics, WILUS Inc., KT Corp.R1-1703167, “Multi-beam control channel transmission”, Nokia, Alcatel-Lucent Shanghai Bell