3GPP TSG RAN WG1 Meeting #89		                          R1- 1709173

Hangzhou, P.R. China 15th – 19th May 2017

Source: 	ETRI

Title: 	Structure of PUCCH in short-duration

Agenda Item:	7.1.3.2.1	Structure of PUCCH in short-duration

Document for:	Discussion

- Introduction

In this contribution, we address our view about short PUCCH. The remaining issues can include how to multiplex other signals and channels.

- Discussion

  - Combed transmission for short PUCCH

The multiplexing capability should take into account interference orthogonalization and randomization. For orthogonalization, LTE ZC sequences are a good baseline. Namely, UE-specific cyclic shift effectively orthogonalize intra-cell interference. Encoded UCI are modulated by QPSK and collides with other modulated symbols. UE-specific scrambling can randomize both intra-cell and inter-cell interferences. Those two multiplexing capability should be similar because either multiplexing capability can be bottle neck to the multiplexing capability of the short PUCCH.

We should consider multiplexing other signals and channels. SRS is not specified in NR, but as a baseline, we could assume that a SRS has the transmission comb to increase the multiplexing order. When the SRS collides with PUCCH DM-RS, then the interference can be orthogonalized. When the SRS collides with the PUCCH UCI, then the interference may be randomized. In this case, the PUCCH demodulation will be degraded. The serving gNB can configure SRS transmission comb with the same as PUCCH DM-RS transmission comb. This imply that SRS and PUCCH DM-RS can share the same transmission comb. This can reduce the multiplexing capability of short PUCCH. Instead, the PUCCH UCI can be mapped using a transmission comb; not filling all REs with modulated UCIs. The multiplexed SRS can use the transmission comb that is the same as PUCCH DM-RS or the other transmission combs that are not used by PUCCH. By introducing combed transmission for short PUCCH, we can increase the multiplexing capability of PUCCH and SRS.

In this section, we assume that SRS symbol is transmitted using consecutive PRBs and should visit all (UE-specific) subbands within the UL system bandwidth. This is because the UL channel response should be available for UL link adaptation. Depending on SRS configurations, the collection of SRS subbands from all connected UEs of a serving gNB can span a majority of system bandwidth. Since the PUCCH subband for a given UE should guarantee the performance requirement, in such case it would be desirable to orthogonally multiplex the SRS subbands. Additionally, we can consider further interference mitigation between SRS subbands and PUCCH subbands.

Figure 1 Example of overlapped PUCCH subband with SRS subband

One of solutions to allow full overlap of each subband. In the case of full overlap, the interference mitigation technique should also be considered. If the SRS symbol is specified similar to the LTE at least for below 6 GHz, then we can assume that the SRS sequence is derived from a CAZAC sequence with UE-specific cyclic shifts and the RE mapping follows the configured RPF. Similar power control design to LTE can be considered in NR. However, the full overlap implies the high intra-cell interference due to the SRS subcarrier mapping. To this end, it is necessary to design the PUCCH and SRS together to avoid collisions in the subcarrier level within the same subband.

Figure 2 Example of SRS and PUCCH in a symbol and a PRB

It is proposed to allow the similar design principle for SRS for at least PUCCH of short duration. Using this approach, the PUCCH and SRS symbol will show the similar interference mitigation performance. If the serving gNB configures PUCCH subband as similar to one of SRS subbands, the PUCCH subband and SRS subband can be multiplexed in the same subband. Even with PUCCH in the same subband, the different zero power subcarriers can orthogonalize the interferences as Figure 2 illustrates. In this case, the RE mapping should avoid the zero power resources and the UCI can be encoded with the corresponding code rate. it is beneficial to allow zero power on those PUCCH subcarriers and to adapt the RE mapping for PUCCH.

In order to specify the RE mapping for the PUCCH of short duration, the explicit modelling of interferences should be considered in the evaluation. In the LTE Rel-13 eCA (LTE_CA_enh_b5C), RAN1 discussed the UL SINR distribution in the email loop [80-02]. In the NR, RAN1 needs to perform similar study to capture UL interference in PUCCH design.

Proposal 1: ZP resources in PUCCH is introduced to reduce intra-cell interferences.

ZP resources increase the encoding rate of UCI bits. Some companies observed in the previous meeting that there is the optimal DM-RS density in terms of the BLER of PUCCH with the UCI payload. Referring to the observation, ZP resources can be additionally considered to determine the effective encoding rate for UCI bits. Depending on the multiplexing capability of a PUCCH subband and SRS, the serving gNB should carefully configure the ZP resources and the DM-RS density to protect PUCCH REs and to assist UCI decoding. Thus, it is worth to mention that ZP resource can affect the encoding rate of UCI bits and also affect the PUCCH bandwidth. By introducing ZP resources, the PUCCH subband with combed transmission should be increased to meet similar encoding rate with respect to the PUCCH without ZP resources because similar number of REs should be available. This leads to the higher frequency diversity.

Observation 1: PUCCH with ZP resource requires more bandwidth and achieves more frequency diversity.

The optimal DM-RS overhead (e.g., between 1/2 or 1/3) can also adjust the number of encoded UCI bits. In turn, the effective encoding rate for UCI bits can also depend on the amount of ZP RS resources. Furthermore, if NR CA or semi-static TDD are configured, then the number of UCI bits can be dynamically changed. In this case, the channel coding (e.g., polar coding) for PUCCH should support such dynamically varying encoding rate.

  - Clustered transmission for short PUCCH

Agreements: (88bis) - For 1-symbol short PUCCH with > 2 UCI bits, the following is supported for the agreed Option 1: - … - X1 to X2 PRBs can be configured to support various UCI payload sizes - Both localized (contiguous) and distributed (non-contiguous) allocations are supported - FFS: detailed PRB allocations and signaling of the configuration - … - …

RAN1 agreed to support both localized and distributed allocations to achieve the reliability. In the case of localized allocation, the serving gNB can assign the PUCCH in the good channel condition. The gain comes from the selection of frequency resource. Since PUCCH transmission involves in the RRC_CONNECTED UEs, it is natural to assume that the serving gNB tracks the UL timing advance, UL CSI, etc. Those information is estimated by any UL transmission or UL-DL channel reciprocity if possible, and can be used for UL dynamic scheduling. Therefore, the serving gNB knows which frequency resources (or subband) has a better quality than others. If PUCCH is transmitted in the good subband, then the detection performance will be increased.

In the case of distributed allocation, the serving gNB can expect the frequency diversity. In LTE, the frequency diversity at UL is implemented by inter-slot frequency hopping. In NR, PUCCH of long duration supports intra-slot frequency hopping. However, the PUCCH of one symbol duration needs study to achieve frequency diversity. One aspects to consider is the PAPR/CM. The increased PAPR/CM may not be a significant problem when it is only configured to cell center UE, but it should not intentionally limit the coverage of short PUCCH.

In addition, we should study the impact to the channel estimation accuracy. In the multi-cluster transmission, the effective channel should be estimated and interpolated with own DM-RS resource in each cluster. If the channel is less selective, then the wider cluster can achieve better channel estimation due to more applicable RS REs. If the channel is highly selective, then the wider cluster can achieve similar channel estimation performance to the narrow cluster because the wider cluster does not obtain any gain from wideband RS.

Moreover, each cluster has decreased transmission power. It is possible to equally divide a whole transmission power into each cluster. This means that each cluster should have low code rate to avoid power outage events. With given encoded UCI payload, the transmission power per cluster should meet the minimum performance requirement.

This reasoning leads us how to allocate single cluster and multi-cluster to achieve reliability. It is desirable to allocate a single cluster provided that UE knows (roughly) the channel gain, and otherwise multiple clusters can be allocated. In other words, when UE receives DCI, a single cluster is chosen, and when UE does not receive DCI, multiple clusters are chosen.

Proposal 2: A few bits in PDCCH can indicates a subband of PUCCH of short duration.

Proposal 3: If DCI is available, then the short PUCCH is transmitted using a single cluster.

The subband selection depends on the UL CSI, and the UL CSI may not be always available at the serving gNB. For instance, during the initial access, the M4 should be acknowledged via PUCCH but with no UL CSI. Other example includes UEs not configured to the periodic SRS or UEs in RRC connection reconfiguration. Those cases prevent the aggressive use of UL CSI and the subband selection may not work well. However, those cases may not require the use of PUCCH of short duration. The serving gNB can indicate the resource of PUCCH of long duration.

- Conclusion

In this contribution, we made the following observation:

Observation 1: PUCCH with ZP resource requires more bandwidth and achieves more frequency diversity.

Based on above observations, we draw the following proposals.

Proposal 1: ZP resources in PUCCH is introduced to reduce intra-cell interferences.

Proposal 2: A few bits in PDCCH can indicates a subband of PUCCH of short duration.

Proposal 3: If DCI is available, then the short PUCCH is transmitted using a single cluster.

- Appendix

In this section, we share our initial result from link simulation. The assumptions are listed in the Table 1.

Table 1 Evaluation assumptions for one symbol PUCCH

| Parameter | Value |
| --- | --- |
| Channel model | TDL-A (100 ns, 700 ns), 4 GHz |
| UCI bits | (10, 20, 30, 40) bits, QPSK, LTE TBCC |
| Numerology | 15kHz, normal CP |
| System bandwidth | 20 MHz (100 PRBs) |
| Antenna configuration | 1Tx, 2Rx (uncorrelated) |
| Receiver | MRC, MMSE |
| Sequence for RS | LTE based CAZAC |
| PRB index selection | Fixed at edge |
| Mobility | 5 km/h |
| RS power boost | 0 dB |
| RS density | 1/2 (every 4th tone for RS, other 4th tone for UCI) |
| Interference | Not modelled |

The two transmission schemes are compared: the single cluster PUCCH and the two cluster PUCCH. Both schemes have the same number of subcarriers to apply the same EPRE, and have the transmission comb (Tc). The single cluster PUCCH occupies contiguous n PRBs at one edge of UL spectrum, whereas the two cluster PUCCH occupies two subbands which has contiguous n/2 PRBs each at each edge of UL spectrum. The DM-RS overhead is a half. Two zero power tones are allocated at every four (Tc) tones. The resource indication by DCI is not considered.

(a) TDL-A (RMS 100 ns) 			(b) TDL-A (RMS 700 ns)

Figure 3 BLER with respect to SNR for PUCCH of 4 PRBs

(a) TDL-A (RMS 100 ns) 			(b) TDL-A (RMS 700 ns)

Figure 3 BLER with respect to SNR for PUCCH of 8 PRBs

For moderate code rate (e.g., 10 bits payload in 4 PRBs, and 30 bits payload in 8 PRBs), the frequency diversity gain from two cluster transmission could be observed. On the other hand, a single cluster of wide bandwidth may achieve more accurate channel estimation, and it can lead to less BLER. In this simulation setting, it is observed that two cluster PUCCH has the similar or better diversity gain over the one cluster PUCCH. In addition, more diversity gain and SNR gain are observed if the channel has more delay spread from 100 ns to 700 ns.

The two cluster PUCCH also has SNR gain (~ 2dB) to the one cluster PUCCH because the frequency selectivity is further exploited, whereas one cluster PUCCH can sometimes experience deep fading which cannot be overcome by the channel coding alone. This 2 dB gain can be offset by applying the frequency selective scheduling of one cluster PUCCH. The serving gNB can choose one of good subband for one cluster PUCCH, and this selection gain would give many dBs depending on the channel delay spread. In this case, a single wide cluster can be a better option if the selection gain can be utilized.

For high code rate (e.g., 20 bits payload in 4 PRBs, and 40 bits payload in 8 PRBs), the diversity order is observed similarly to the moderate code rate but the operating SNR range should be quite large. In this case, relying on the selection gain from the gNB scheduler may not be sufficient. Instead, in order to avoid such high code rate, more PUCCH resource should be allocated at least when large UCI bits are generated. Thus, the serving gNB can configure a few PUCCH resources, and the UE should choose one appropriate PUCCH resource according to its payload. One example scheme is to allow PUCCH resource adaptation according to the payload, which is helpful for UL resource efficiency especially when UL duration is small in the considering slot. The detailed signalling scheme can be further studied.