3GPP TSG RAN WG1 Meeting#89                 		R1-1709183

Hangzhou, China, 15th - 19th May 2017

Agenda Item:	7.1.2.3.5

Source: 	LG Electronics

Title: 	Discussion on CSI feedback Type II

Document for:	Discussion/Decision

- Introduction

In RAN1 #88bis meeting, agreements on CSI feedback Type II were captured in Chairman’s note as follows:

Agreements:

- For Type I and II Cat1 (if Cat1 is supported) single panel codebooks (W=W1W2 structure):

  - The exact design of W1 is to be decided in RAN1#89 for both Type I and Type II Cat1 (if Cat1 supported)

    - For W1 codebook, companies are encouraged to perform more evaluations comparing the different alternatives

- For Type I: Study further the values of L among L=1 and L= 4, at least for rank 1

Agreements:

- For Type II CSI feedback (Cat 1, if supported), at least rank 1 and rank 2 are supported

  - FFS other ranks

- For beam selection:

  - Support at least unconstrained beam selection from orthogonal basis

Agreements:

- FFS to support a common design of W2 for Cat. 1, Cat. 2 and Cat. 3

  - FFS for Cat. 3, W2 only feedback is allowed

    - FFS amplitude feedback for W2 (e.g., wideband, subband, etc.)

  - Note: this does not mean NR supports all three categories

- FFS whether or not to merge Category 1 and Category 3 using a unified codebook formulation

Agreements:

- Study mechanisms targeting efficient use of peak and/or average CSI overhead for CSI feedback Type II.

  - For Category I, e.g.

    - Mechanism 1: Frequency selective precoding feedback with delay-related parameter(s) (e.g. R1-1704884, R1-1705927)

    - Mechanism 2: Differential CSI reporting in time domain h(e.g. R1-1705349, R1-1705588)

    - Mechanism 3: Uneven quantization bit allocation for the beam amplitudes or/and phases (e.g. R1-1705076)

    - Mechanism 4: Matrix quantization considering inter-layer orthogonality for W2(e.g., R1-1704408)

    - Note: performance should be also considered for overhead reduction

    - Other examples are not precluded.

In this contribution, we discuss CSI feedback Type II for NR MIMO and provide our simulation results on Type II CSI feedback.

Discussion on CSI feedback Type II for NR

In NR MIMO, the availability of accurate CSI to guarantee enhanced MU performance becomes a main driver for discussion of Type II feedback. Compared to Type I feedback, main merit of Type II feedback is the point that inter-user interference can be reflected in case of MU-MIMO. In LTE, a set of implicit CSI is basically determined based on one interference hypothesis, and thus UE calculates CSI with no MU interference hypothesis, i.e., SU-MIMO assumption. When TRP wants to conduct MU-MIMO, a transformation of CSI from the CSI computed by SU-MIMO assumption to MU-MIMO case is needed. It may lead to inaccuracy of CSI, especially when the number of paired UEs increases. On the other hand, TRP can estimate channel quality according to different MU interference hypothesis by utilizing pure MIMO channel information in case of Type II CSI feedback.

In RAN1 adhoc meeting in January, it was listed up three categories for Type II CSI feedback. Category 1 is precoder feedback based on linear combination codebook. In Rel-14 eFD-MIMO, it has been verified the potential benefit of Type II CSI feedback in the context of MU-MIMO enhancement. Especially, the linear combining (LC) codebook with the orthogonal basis in W1 is considered to provide high resolution channel information. In RAN1#88bis, it was agreed to support at least rank 2 for Cat 1 of Type II CSI feedback. The remaining details for this category are 1) W1 structure, 2) number of L beams in W1, 3) combining coefficients (power and phase).

Regarding 1), among the candidates for W1 structure, we prefer to have the same W1 structures of Type I CSI feedback for the unified frame work of codebook design as

Regarding 2), larger number of L can provides more degrees of freedom for beam combining at the expense of increased feedback bits for both W1 and W2. Note that it is more critical for W2, since the number of combining coefficient phase and/or power is linearly increasing. Figure 1 exhibits the performance comparison of LC codebook with various values of L for 3D-UMi. In the simulation, we assume (N1,N2 )=(2,4) and medium traffic load. As shown in the plot, increasing L provides better performance and L=6 shows almost identical performance with SVD precoding when ideal SB feedback for both amplitude and phase are applied. In the case of non-ideal feedback, there is no performance difference between L=4 and L=6 where we assume WB 3-bit amplitude, SB 3-bit phase quantization. Thus, in our view, L should not be larger than 4.

Figure 1. Performance comparison with various value of L

Regrading 3), we evaluate the performance of LC codebook with various values of quantization bits for both amplitude and phase in Figure 2. As shown in the plot, 3-bit SB wise quantization provides the best performance at the expense of the increased feedback bits. Also, 3-bit WB amplitude quantization exhibits the comparable performance compared to the SB 2-bit amplitude quantization. Thus, in our view, WB amplitude quantization is preferable considering the trade-off between performance and feedback overhead.

Figure 2. Performance comparison with various value of quantization bits for amplitude and phase

Proposal 1. If Category 1 is supported, following design constraints need to be considered.

- L should not be larger than 4.

- Wideband power combining and subband phase combining.

In the Category 2, covariance matrix feedback is considered for Type II CSI feedback. In this category, dimension reduction methods for lowering the payload size of channel covariance feedback are required especially for the larger number of antenna ports. However, such dimension reduction technique inevitably brings the notable performance loss, and this trade-off between performance and payload size becomes severe when the number of Tx antenna ports become large. For this reason, limiting the number of port supported by Category 2 based Type II feedback can be considered.

Proposal 2. If Category 2 is supported, consider limiting supported maximum number of CSI-RS antenna ports in a single instance reporting.

In eFD-MIMO, hybrid CSI feedback is an important feature, since it provides performance benefit by efficiently reducing the system overhead. In this context, Category 3 can also be a good candidate for CSI feedback Type II. In Category 3, more accurate CSI can be acquired based on either Category 1 or 2 in long-term manner, and short-term port-selection or full-CSI feedback can be carried out similar to LTE Class B. To support Category 3, we need to first decide which category (category 1 or 2) for Type II CSI feedback is supported in NR. Thus, RAN1 should carefully decide Type II CSI feedback between Category 1 and 2 by comparing the performance benefits as well as feedback overhead.

Codebook design for category I of CSI feedback type II

In this section, we present a new codebook design (frequency selective precoding feedback: FSPF) for category I in order to reduce the payload size of SB reporting. The key idea of codebook design is to apply linear combining of 2L beams which can have different level of cyclic phase shift in frequency domain, and thus SB phase combining in the legacy LC codebook can be skipped. Then, the linear combination codebook structure can be constructed as

where  is comprised with orthogonal basis,is a 2D-DFT beam,  is the relative power coefficient for i-th beam, k represents the frequency domain index (e.g., subcarrier index, RB index), is the phase offset for i-th beam, and  controls the degree of the phase shift with respect to k. Here,  is defined as

where the quantity  is the smallest number from the set  such that , is the number of the subcarriers in the configured bandwidth, and is an integer value, e.g., .

Without loss of generality, W with rank 1 can be expressed as

.

Here, we assume that the 1st column is the strongest beam. In order to determine (2L-1) parameter set, we calculate the frequency domain samples by projecting basis beam(s) to channel matrix or dominant eigenvector of k-th subcarrier. Then, we take IFFT of obtained frequency domain samples to derive the maximum delay () for l-th basis beam. The amplitude and phase offset,, can be computed using the time domain sample corresponding to maximum delay. For rank 2, layer independent codebook construction can be applied.

Table I lists the required feedback bits for both proposed FSPF and conventional LC codebook. In the case of 16-ports, L=4, K=9,FFTsize=64 and rank 1, the resulting total payload can be 99 bits and 213bits for proposed scheme and conventional LC codebook, respectively. For rank 2, 183 bits and 402 bits are required for proposed scheme and conventional LC codebook, respectively, and this means that the proposed FSPF can reduce the total payload by 46%.

Observation1. Proposed FSPF significantly reduce the total payload size by 46% compared to the conventional LC codebook.

Table I exhibits the feedback bits comparison between proposed FSPF and conventional LC codebook for Rank 1

|  |  | Proposed FSPF codebook | LC codebook |
| --- | --- | --- | --- |
| WB | Leading beam selection | log2N1N2O1O2 |  |
|  | (L-1) beam selection | log2(N1N2-1*…*N1N2-L-1) |  |
|  | Delay parameter quantization | 2L-1*log2*FFTsize) | N/A |
|  | Amplitude quantization (3bit) | (2L-1)*3 | (L-1)*3 (pol & layer common) |
|  | Phase offset quantization (3bit) | (2L-1)*3 | N/A |
| SB | Phase quantization (3bit) (K = #of SB) | N/A | K*(2L-1)*3 |

Figure 3 presents the performance comparison between proposed FSPF and conventional LC codebook. It is assumed that (N1,N2 )=(2,4) and high traffic load. Also, each UE is equipped with 2 Rx antenna ports so that maximum rank 2 transmission can be considered. Note that for rank 2 transmission of our proposed scheme, additional layer orthogonality process such as Gram-Schmidt is applied after determining the codebook parameters. In addition, we consider RB-level frequency domain samples, and assume FFT size is 64. As shown in the plot, our proposed scheme provides 22% and 48% performance gain over Class A codebook Config 1 in terms of mean UE UPT and 5% UE UPT, respectively. Also, it is shown that proposed scheme outperforms the conventional LC codebook with much reduced payload size.

Figure 3. Performance comparison with various codebook scheme

Observation2. Proposed scheme provides 22% and 48% performance gain over Class A codebook Config 1 in terms of mean UE UPT and 5% UE UPT, respectively.

Observation3. Proposed scheme outperforms the conventional LC codebook with much reduced payload size.

Proposal 3. Support frequency selective precoding feedback (FSPF) for CSI feedback Type II.

Conclusion

This contribution discussed CSI feedback for NR MIMO. Following observations and proposals are given, based on the discussion:

Observation1. Proposed FSPF significantly reduce the total payload size by 46% compared to the conventional LC codebook.

Observation2. Proposed scheme provides 22% and 48% performance gain over Class A codebook Config 1 in terms of mean UE UPT and 5% UE UPT, respectively.

Observation3. Proposed scheme outperforms the conventional LC codebook with much reduced payload size.

Proposal 1. If Category 1 is supported, following design constraints need to be considered.

- L should not be larger than 4.

- Wideband power combining and subband phase combining.

Proposal 2. If Category 2 is supported, consider limiting supported maximum number of CSI-RS antenna ports in a single instance reporting.

Proposal 3. Support frequency selective precoding feedback (FSPF) for CSI feedback Type II.

Annex A: Simulation Parameters and Assumptions

Table A-1. Simulation assumptions

| Scenarios | 3D-UMi with ISD = 200m in 4GHz |
| --- | --- |
| BS antenna configurations | Antenna elements config: (+/-45), 0.5λ horizontal / 0.8 λ vertical antenna spacing |
| MS antenna configurations | 2 Rx X-pol (0/+90) |
| System bandwidth | 10MHz (50RBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Duplex | FDD |
| Network synchronization | Synchronized |
| UE distribution | Follows TR36.873 |
| UE speed | 3km/h |
| Polarized antenna modeling | Model-2 from TR36.873 |
| UE array orientation | ΩUT,α uniformly distributed on [0,360] degree, ΩUT,β = 90 degree, ΩUT,γ = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (low ~20% RU, medium ~50% RU) |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |
| Receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel-12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions |
| CSI-RS, CRS | CSI-RS one-to-one mapping to TXRU, only CRS port 0 is modeled for UE attachment, CRS port 0 is associated with the first TXRU |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | CQI, PMI and RI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 REs per PRB |
| Transmission scheme | TM10, single CSI process, SU -MIMO with rank adaptation (no CoMP) |
| Wrapping method | Geographical distance based |
| Handover margin | 3 dB |
| Metrics | Average UE throughput, 5% UE throughput. |
| Max MU layer | 4 |