3GPP TSG-RAN WG1 RAN1#88b	                                  	                  R1-1709800

Hangzhou, China, 15-19th May 2017

Source:	Mitsubishi Electric

Title:	Summary of AI 7.1.2.1.5 UL transmit diversity

Agenda Item:	7.1.2.1.5

Document for: 	Discussion/Decision

# Introduction

In RAN1#88b [1]  it was decided that:

- For DFTsOFDM in data channel, the following schemes are candidates for transmit diversity:

- Low PAPR Alamouti-based transmit diversity applied in frequency or time domain, transparent transmit diversity (e.g. short delay CDD, panel selection), time domain beam/precoder cycling.

No agreement has been made for CP-OFDM UL transmit diversity, but several companies were targeting a unified framework with DL transmission scheme 2. At this meeting, concerning DL transmission schemes, it has been decided that:

- For NR in Rel-15, DL transmission scheme 2 is not explicitly supported for unicast PDSCH in specification

  - Note: CSI feedback assuming open-loop/semi-open-loop and/or closed-loop transmissions is to be discussed separately

Based on the current situation and on the submitted contributions [2] - [13] in RAN1#89 for this agenda item, the following open issues and observations are identified.

# UL transmit diversity for CP-OFDM PUSCH

Many simulations have been performed in the context of DL transmission scheme 2, but only one company provided results in an UL context [9].

The following alternatives remain for UL transmit diversity for CP-OFDM:

- Alt. 1: transmit diversity is not explicitly supported for PUSCH in Rel. 15

  - Supporting companies: Qualcomm, Huawei, Samsung, Intel, Ericsson, LGE

- Alt. 2 non-transparent UL transmit diversity for CP-OFDM

  - SFBC: Nokia

  - Non-transparent precoder cycling: Nokia, ZTE

# UL transmit diversity for DFTsOFDM PUSCH

Only two companies provided simulation results for PUSCH DFTsOFDM UL transmit diversity [7], [11], recommending low PAPR Alamouti-based methods.

The following candidates remain for UL transmit diversity with DFTsOFDM

- Low PAPR Alamouti-based transmit diversity

  - Supporting companies: Mitsubishi Electric, Qualcomm, Interdigital

- transparent transmit diversity (e.g. short delay CDD, panel selection, transparent time domain beam/precoder cycling)

  - Supporting companies: IITH, Cewit, Reliance Jo, IITM, Tejas Networks, Nokia, Samsung, Intel, Ericsson, LGE

- Non-transparent time domain beam/precoder cycling:

  - Supporting companies: IITH, Cewit, Reliance Jo, IITM, Tejas Networks, Samsung, [ZTE]

# Proposed offline outcome

Possible offline conclusion

For DFTsOFDM and CP-OFDM, companies are encouraged to provide evaluation results for the next RAN1 meeting.

# Current list of related WFs

-

# References

### Chairman’s notes, RAN1#89, Spokane, Washington, April 2017.

### R1-1707115 Diversity based uplink transmission schemes	ZTE

### R1-1707351 On diversity based UL transmission	Intel Corporation

### R1-1707472 Further discussion on transmission diversity scheme in UL	CATT

### R1-1707602 Discussion on diversity transmission for UL	LG Electronics

### R1-1707739 TxD for UL	AT&T

### R1-1707798 UL diversity transmission for DFTsOFDM	Mitsubishi Electric RCE

### R1-1707946 Diversity transmission for UL	Samsung

### R1-1708131 Diversity-based transmission for UL	Huawei, HiSilicon

### R1-1708223 Uplink diversity scheme for DFT-S-OFDM	IITH

### R1-1708583 On UL diversity transmission scheme	Qualcomm Incorporated

### R1-1708671 Robust Transmission Scheme for PUSCH	Ericsson

### R1-1708896 UL diversity schemes	Nokia, Alcatel-Lucent Shanghai Bell