3GPP TSG RAN WG1 Meeting #89                                                                 R1-1709815

Hangzhou, P.R. China 15th – 19th May 2017

Source:	NTT DOCOMO

Title:	Summary of CSI Acquisition Details

Agenda Item:	7.1.2.3.2

Document for: 	Discussion

# List of agreements in RAN1#88bis

Agreements: - For interference measurement, down selection from options will be conducted. - NZP CSI-RS based - Opt. A1: Estimation on NZP CSI-RS for channel estimation (by subtracting CSI-RS from Rx signal) - Opt. A2: Emulation on NZP CSI-RS which is represented by multiplied value of channel and precoding matrix - DM-RS based - Opt. B1: Estimation on DM-RS for own data demodulation (by subtracting DM-RS from Rx signal) - Opt. B2: Estimation on DM-RS for other UEs - Criteria for design and down selection are as follows. - Required RS densities - UE processing latency - Support of self-contained CSI reporting (if supported) at least depends on the location of IMR. - FFS: Whether the emulation is performed at TRP side or UE side - FFS: RAN1 specification impact, if any, on the options above Agreements: - Aperiodic IMR is triggered by DCI. - Herein the IMR refers to ZP CSI-RS based IMR. Other types of IMR are FFS - FFS: use of MAC CE to reduce the set of candidate IMRs among the resources configured in RRC. - FFS whether or not to jointly trigger aperiodic NZP CSI-RS and/or aperiodic ZP CSI-RS - FFS: semi-persistent IMR is activated/deactivated by MAC CE or DCI.

# Open issues

Note: This may not list all aspects. See official information in chairman’s notes.

## 2.1 Interference measurement

- Down selection of interference measurement schemes from the options A1, A2, B1 and B2

  - Current status

    - Agreed to support NZP CSI-RS based interference measurement (Opt. A1 and A2) as a working assumption with some FFS parts

  - Remaining issues in this meeting

    - Nothing

  - Remaining issues in the next meeting

    - Possible down selection from options A1 and A2

    - Whether to support options B1 and B2

- Detailes of interference measurement

  - Current status

    - For ZP CSI-RS based IMR, aperiodic IMR is triggered by DCI.

  - Remaining issues in this meeting

    - Nothing

  - Remaining issues in the next meeting

    - Triggering mechanims of semi-persistent IMR with ZP CSI-RS

    - Time domain behavior and triggering mechanisms for other interference measurement resources, e.g., NZP CSI-RS [and DM-RS].

    - Resource configuration design of IMR, e.g., RE mapping and periodicity

## 2.2 Measurement restriction

- Channel measurement restriction (MR)

  - Current status

    - No agreement for now

  - Remaining issues in this meeting

    - Samsung’s WF (listed below)

  - Remaining issues in the next meeting

    - Whether to support channel MR

    - Detailed design of channel MR

- Interference measurement restriction (interference MR) and measurement subset

  - Current status

    - Agree to support at least one of the following schemes

      - Measurement subsets in both time and frequency domain

      - Interference measurement restriction in both time and frequency domain

  - Remaining issues in this meeting

    - Samsung’s WF (listed below)

  - Remaining issues in the next meeting

    - Decide whether to support time/frequency domain interference MR and measurement subset based on previous agreement (FFS: Further details)

2.3 Rate matching

  - Current status

    - No agreement in the MIMO AI

  - Remaining issues in this meeting

    - Nothing

  - Remaining issues in the next meeting

    - Condition on rate matching, e.g., around NZP CSI-RS

    - Rate matching resource (signals/channels to be considered for rate matching including, e.g., ZP CSI-RS for IMR)

    - Additional resource configuration design (if needed), e.g., RE mapping and periodicity

    - Time domain behavior and possible triggering mechanisms, e.g., dynamic triggering

# List of current WFs

- R1-1709779	WF on CSI acquisition	Samsung, Ericsson

  - Propose to support following features for NR CSI acquisition (see details in the WF)

- Time and frequency domain measurement restriction

- Frequency domain subset restriction

- CSI reporting via short/long duration PDCCH

- PUCCH reporting which is contained in a single slot