3GPP TSG RAN WG1 Meeting #90		R1-1712845

Prague, Czech Republic, 21th – 25th August 2017

Source:	vivo

Title:	 DMRS design for NR-PDCCH

Agenda Item:	6.1.3.1.1.2

Document for:	Discussion and Decision

- Introduction & Background

Regarding the RS design aspects for NR-PDCCH, some agreements were made after RAN1 #89 and RAN1 ad-hoc #2 meeting as follows, respectively,

Agreements[1]:

- Confirm working assumption:

  - One-port transmit diversity scheme with REG bundling per CCE is used for NR-PDCCH

- FFS: DMRS RE overhead for the REG transmitting DMRS is 1/3

  - FFS on DMRS pattern

Agreement[2]:

- DMRS is mapped on all REGs on all the OFDM symbols of a given PDCCH candidate

  - The DMRS density is the same on all REGs

In this contribution, we discuss DMRS design for DL control channel (NR-PDCCH) for different aggregation levels, as well as present our view based on link level simulation results on these aspects.

- Discussions

  - DMRS design for 1-symbol NR-PDCCH

Different alternatives for DMRS design for 1-symbol NR-PDCCH are given in Figure 1. 2-port DM-RS sequences can be multiplexed over the four REs in the FDM or CDM manner by OCC (orthogonal cover code). For FDM pattern 1, two RE groups are uniformly distributed over the whole RB. Each group will include two adjacent REs; For FDM pattern 2, four REs are uniformly distributed over the whole RB.

Figure 1. DMRS patterns for 1-symbol NR-PDCCH considered in simulation

  - DMRS design for 2-symbol NR-PDCCH

Different alternatives for DMRS design for 2-symbol NR-PDCCH are given in Figure 2. In addition, we made comparison between 2-symbol NR-PDCCH and 1-symbol NR-PDCCH on the premise that the same number of CCEs are used for NR-PDCCH. Simulations are conducted under various user speeds cases.

Figure 2. DMRS patterns for 2-symbol NR-PDCCH considered in simulation

  - Results & Observations

The following assumptions are made for simulation.

- It is assumed one REG mapping to a whole PRB, which is consisted of 12 subcarriers and 1 symbol.

- 24 PRBs are used for NR-PDCCH (Aggregation level = 4) and the bundling size in frequency-domain is 2 REGs.

- CCE size of 6 REGs is assumed.

- Distributed REG-to-CCE mapping

- Single-port precoding cycling are considered in the simulation.

- Channel model: TDL-C, 300ns

For other simulation assumptions, the simulation follows the agreements made in RAN1#88 meeting. Details are provided in the Annex A.

The simulation results are given as follows,

1-symbol NR-PDCCH

Figure 3. RS CDM vs FDM with/without power boosting

In Figure 3, we can observe that  for NR-PDCCH with 1 OFDM symbol, DMRS design in CDM or FDM manner,

- FDM with pattern 1 and pattern 2 have similar performance

- CDM RS outperforms FDM when no power boosting is adopted

- 3dB RS power boosting can improve the PDCCH performance

- Note: 3dB boosting is adopted when RS power boosting is assumed.

2-symbol NR-PDCCH

Figure 4. Performance for 2-symbol NR-PDCCH with different user speeds

As shown in Figure 4, when UE with different speeds,

- For NR-PDCCH with 2 OFDM symbols and DMRS in each OFDM symbol, FD-OCC (Frequency domain OCC) and TD-OCC (Time domain OCC) have similar performance.

  - For 1-symbol CORESET duration case, only FD-OCC is applicable.

  - For 2 or 3-symbol CORESET duration case, considering the agreement that the REG bundling size can be euqal to the CORESET duration and time-first mapping is agreed as the mapping scheme, TD-OCC seems to be more natural. Because all the DMRS within a OCC can be assumed to the same precoder.

- One symbol NR-PDCCH outperforms two symbol NR-PDCCH at high SNR region due to more frequency diversity

- Two symbol NR-PDCCH outperforms one symbol NR-PDCCH at lower SNR region due to better channel estimation

According to the above simulation results, we have the following observation and proposals:

Proposal 1: CDM with frequency domain OCC RS pattern should be considered for NR-PDCCH in a 1-symbol duration CORESET.

Proposal 2 : CDM with time domain OCC RS pattern should be considered for NR-PDCCH in a 2/3-symbol duration CORESET.

Proposal 3: RS power boosting should be considered if FDM DMRS pattern is used for NR-PDCCH.

- Conclusion

This contribution discusses some considerations on RS design for NR-PDCCH. In summary, proposals are made as follows:

Proposal 1: CDM with frequency domain OCC RS pattern should be considered for NR-PDCCH in a 1-symbol duration CORESET.

Proposal 2: CDM with time domain OCC RS pattern should be considered for NR-PDCCH in a 2/3-symbol duration CORESET.

Proposal 3: RS power boosting should be considered if FDM DMRS pattern is used for NR-PDCCH.

References

- RAN WG1#89 Chairman’s Notes.

- RAN WG1 ad-hoc #2 Chairman’s Notes.

Annex A. Simulation assumptions

Table A.1 Link-level simulation assumptions

| Parameter | Assumption |
| --- | --- |
| Bandwidth | 20 MHz |
| Channel model | For 1-symbol PDCCH: 3 Km/h, TDL-C, 300ns |
|  | For 2-symbol PDCCH:3/70/120 Km/h, TDL-C, 300ns |
| Channel estimation | Ideal MMSE |
| Resource allocation | one/ two-OFDM symbol are reserved for CCE mapping |
|  | One PRB for one REG |
| Control Channel Payload | 60-bit payload, 16-bit CRC |
| Coding scheme | Polar Code Punctured and ordering scheme according to [2] |
| Modulation Scheme | QPSK, Max-log-likehood algorithm |
| Subcarrier spacing | 15kHz |
| Symbol length | Normal CP 160Ts + 2048 Ts, Ts =1/30.72us |
| Antenna Model | 2 Tx, 2 Rx |
| Transmission scheme | Precoder cycling, REG bundling size=2 |
| CCE size | 6 RBs, 72 subcarriers, 1/3 overhead for RS |