3GPP TSG RAN WG1 Meeting #90 	R1-1712995

Prague, Czech Republic, 21st – 25th August 2017

Agenda item:	6.1.3.3.6.1

Source:	NEC

Title:	Number of HARQ processes for co-existence with TD-LTE

Document for:	Discussion and decision

Introduction

In RAN1#89 meeting in Hangzhou, there were agreements on HARQ as follows:

Agreements:

- A set of reference parameters is used for the purpose of soft buffer dimensioning

  - A reference set of parameters includes at least DL HARQ RTT [Y ms] and data rate(s) of X Gbps

  - FFS: values of X and Y

  - FFS: other conditions

  - This does not imply UE has to have a HARQ-ACK timing based on the reference HARQ RTT

  - FFS: how different UE categories are defined

  - LBRM is taken into account

- Maximum number of HARQ processes per carrier supported in NR is 8 or 16

  - This is at least for the single numerology case and a slot-level scheduling and single-TRxP transmission

  - FFS: down-selection of 8 or 16

  - FFS: soft-buffer handling

  - FFS: the value may be different depending on a certain condition (e.g., subcarrier spacing)

In this contribution, we provide our view on an FFS point of the maximum number of HARQ processes.

Discussion

For NR, it would be possible that an NR carrier to be deployed to a carrier adjacent to TD-LTE carrier(s). In our understanding, TD-LTE carriers in adjacent channels adopt the same UL/DL configuration to avoid interference to adjacent channels. Similarly, the NR carrier would need to adopt the same slot configuration, i.e. the same UL/DL direction as adjacent TD-LTE carriers, in order to avoid interference to each other. Otherwise, downlink transmission would give interference to uplink reception between TD-LTE and NR carriers in adjacent channels. In this scenario, we assume a self-contained slot would not be applicable instead of DL only slot because uplink part in the slot might suffer interference from DL transmission of adjacent TD-LTE carrier(s).

In TD-LTE, DL heavy UL/DL configurations are supported. For example, configuration 5 has 8 contiguous downlink subframes followed by DwPTS, and UpPTS followed by only one uplink subframe. With this configuration, 9 PDSCH can be transmitted contiguously in time without any occasion of HARQ feedback.

When a NR carrier with 15 kHz subcarrier spacing is deployed adjacent to a TD-LTE carrier(s) with UL/DL configuration 5, frame format of the NR carrier would be eight consecutive downlink only slots followed by a self-contained subframe and one uplink only slot.

A UE capable of K1=0 may transmit HARQ-ACKs of 9 PDSCHs in self-contained slot so 9 HARQ processes are needed. On the other hand, a UE not capable of K1=0, 9 HARQ processes would not be enough to keep soft channel bits until potential retransmission. Hence, the number of HARQ processes for NR should be more than 9.

Observation: 9 HARQ processes would not be enough to support adjacent channel deployment of NR and TD-LTE with UL/DL configuration 5.

By taking into account the agreement from RAN1 meeting#89, of “the number of HARQ processes is 8 or 16” [1], we would like to make the following proposal:

Proposal: NR should support 16 HARQ processes.

Figure 1. Example of co-existence scenario of TD-LTE and NR in adjacent channel

Conclusion

In this contribution, we discussed about adjacent channel co-existence of NR and TD-LTE and made the following observation:

Observation: 9 HARQ processes would not be enough to support adjacent channel deployment of NR and TD-LTE with UL/DL configuration 5.

And consequently made the following proposal:

Proposal: NR should support 16 HARQ processes.

Reference

[1] RAN1 Chairman’s notes