3GPP TSG RAN WG1 Meeting #90                                            	R1- 1713060

Prague, Czech Republic 21th – 25th August 2017

Agenda Item:	6.1.1.1.2

Source:	Spreadtrum Communications

Title:	Cell searching with multiple SS blocks in wideband CC

Document for:	Discussion and decision

Introduction

In RAN1#89 meeting [1], the following agreements have been achieved:

Agreements:

- Support single and multiple SS block transmissions in wideband CC in the frequency domain

  - For non-CA UE with a smaller BW capability and potentially for  CA UE, the measurement gap for RRM measurement and potentially other purposes (e.g., path loss measurement for UL power control) using SS block is supported (if it is agreed that there is no SS block in the active BW part(s))

  - UE can be informed of the presence/parameters of the SS block(s) and parameters necessary for RRM measurement

    - FFS: via either RMSI, other system information, or RRC signaling

  - FFS: number of SS blocks in wideband

  - FFS: number of SS blocks for RRM measurement

  - FFS: Details of measurement configuration

Agreements:

- For frequency location of CORESET for RMSI scheduling and NR-PDSCH for RMSI, RAN1 will select one or more alternative(s) from followings in the next meeting

  - Alt. 1: CORESET for RMSI scheduling and NR-PDSCH for RMSI are confined within the BW of one NR-PBCH

  - Alt. 2: CORESET for RMSI scheduling is confined within the BW of one NR-PBCH and NR-PDSCH for RMSI is not confined within the BW of one NR-PBCH

  - Alt. 3: CORESET for RMSI scheduling and NR-PDSCH for RMSI are not confined within the BW of one NR-PBCH

In this contribution, we will discuss some remaining issues on the wideband CC with multiple FDMed SS blocks and focus on the association between an SS block and a RMSI.

Discussion

For initial access, an SS block provides the information to find a cell and a corresponding RMSI offers the necessary information to access the cell.

In the RAN1#89 meeting, it comes to the agreements that several SS blocks can be configured in a wideband CC in the frequency domain. Based on the above agreements, considering the relationship between an SS block and a RMSI, there will be two kinds of SS blocks, i.e. the SS block (with a corresponding RMSI) which can be used to access a cell and the SS block (without a corresponding RMSI) which can be used for other purposes, e.g. for RRM measurement. An example is shown in Fig.1, one of these FDMed SS blocks is associated with a RMSI and the others are not. Thus, the UE needs to detect multiple SS blocks until it finds the SS block associated with a RMSI.

Observation 1: To access a cell, a UE needs to find an SS block with a corresponding RMSI.

Fig.1 An example of the association between an SS block and a RMSI. One of the FDMed SS blocks is associated with a RMSI and the others are not associated with the RMSI.

However, it needs to be noted that if some of the SS blocks can be detected during the cell-search process, but they cannot be used to access the corresponding cell, these SS blocks may cause some unnecessary works on PSS/SSS/PBCH detecting and decoding during the cell-search process. This scenario will bring extra complexity and latency to the cell-search procedure.

One direct solution for this issue is that, each of the FDMed SS blocks in a wideband CC is associated with a corresponding RMSI as shown in Fig .2. It allows that every SS block can be used to find and access the corresponding cell, but it may bring multiple FDMed RMSIs in a wideband CC which results in high overheads.

Observation 2: Multiple FDMed RMSIs, which are contained in a wideband CC, result in high overhead.

Fig.2 An example of the association between an SS block and a RMSI. Each of the FDMed SS blocks in a wideband CC is associated with a corresponding RMSI.

Another solution is that, several FDMed SS blocks in a wideband CC can be associated with one RMSI. Thus, the number of FDMed RMSIs will be reduced. In this solution, a UE can use any of the FDMed SS blocks in a wideband CC to find a cell, obtain the corresponding RMSI and access the cell. Fig. 3 gives some examples of the association between multiple SS blocks and a RMSI.

Proposal 1: All SS block in a wideband CC should be associated with a corresponding RMSI and can be used for cell search and accessing.

Fig.3 An example of the association between an SS block and a RMSI. Several FDMed SS blocks in a wideband CC can be associated with one RMSI.

Conclusion

In this contribution, we give our considerations on the association between the SS block and the RMSI. Based on the above discussion, the following proposals is drawn.

Observation 1: To access a cell, a UE needs to find an SS block with a corresponding RMSI

Observation 2: Multiple FDMed RMSIs, which are contained in a wideband CC, result in high overhead.

Proposal 1: All SS block in a wideband CC should be associated with a corresponding RMSI and can be used for cell search and accessing.

References

- 3GPP RAN1, “Chairman’s Notes RAN1_89”, 2017-5.