3GPP TSG RAN WG1 #90	R1-1713331

Prague, Czech Republic, 21st – 25th August 2017

Source:	Sharp

Title:	Group common PDCCH monitoring for NR

Agenda Item:	6.1.3.1.4.3

Document for:	Discussion and Decision

# Background

In RAN1 AH-NR1 meeting, the following agreements were made [1].

Agreements: - NR supports a ‘group common PDCCH’ carrying information of e.g. the slot structure. - If the UE does not receive the ‘group common PDCCH’ the UE should be able to receive at least PDCCH in a slot, at least if the gNB did not transmit the ‘group common PDCCH’. - The network will inform through RRC signalling the UE whether to decode the ‘group common PDCCH’ or not - Common does not necessarily imply common per cell. - Continue the discussion on the detailed content of the ‘group common PDCCH’ including usage for TDD and FDD - The term ‘group common PDCCH’ refers to a channel (either a PDCCH or a separately designed channel) that carries information intended for the group of UEs.

Agreements: - ‘Slot format related information’ - Information from which the UE can derive at least which symbols in a slot that are ‘DL’, ‘UL’ (for Rel-15), and ‘other’, respectively - FFS: if ‘other’ can be subdivided into ‘blank’, ‘sidelink’, etc - FFS: ‘Control resource set duration’ - FFS: Indicates the duration of the control resource set(s) - FFS: Can help the UE skip some of the semi-statically configured blind decodings. If not received, the UE performs all blind decodings.

Furthermore, in RAN1 AH-NR2 meeting the following agreements were made [2].

Agreements: - Prioritize discussion of SFI functionality of a group-common PDCCH. - Further work will be on group-common PDCCH carrying the SFI at least in August meeting.

In this contribution, we present our views on UE behaviours with or without group common PDCCH reception.

# Monitoring of ‘group common PDCCH’

In RAN1#AH-NR1 meeting, it was agreed that the network will inform through RRC signaling UE of whether to decode the group-common PDCCH. The intention to have such configurability of the group-common PDCCH monitoring was to reduce signaling overhead and the UE’s processing for the case when dynamic adjustments are not required. Therefore, even if a UE is not configured to monitor the group-common PDCCH, the UE and the network should be able to communicate with each other.

Here, we should consider two typical cases of “not configured to monitor the group-common PDCCH”. One is the case when the UE is semi-statically configured with SFI instead of monitoring of the group-common PDCCH. In this case, the solution is obvious. The UE just follows the semi-static SFI. The other case is an initial access phase, i.e. before configuring of either monitoring of the group-common PDCCH or the semi-static SFI. RAN1 has not discussed SFI assumptions for the initial phase and needs to clarify a UE behavior.

Proposal 1:

- Even if a UE is not configured to monitor the group-common PDCCH, the UE and the network should be able to communicate with each other.

  - If the network configures an SFI to the UE by RRC signaling, the UE follows the configure SFI.

  - Otherwise, the UE follows the behavior which is defined for initial access phase.

    - RAN1 should define an SFI assumption for initial access phase.

In the same meeting, it was also agreed that a UE should be able to receive at least PDCCH in a slot even if the UE does not receive the group-common PDCCH. The principle is applied to a case that the UE is configured with the group-common PDCCH monitoring. For this case, the simplest way is to follow the UE behavior which is defined for the case that the UE has not been configured to monitor the group-common PDCCH yet, e.g. UE behavior during an initial access phase.

Proposal 2:

- If a UE is configured with group-common PDCCH monitoring, and if the UE does not detect the group-common PDCCH, the UE follows the UE behavior which is defined for the case that the UE has not been configured to monitor the group-common PDCCH yet, e.g. UE behavior during an initial access phase.

# Conclusion

In this contribution, group-common PDCCH are discussed, and we propose:

Proposal 1:

- Even if a UE is not configured to monitor the group-common PDCCH, the UE and the network should be able to communicate with each other.

  - If the network configures an SFI to the UE by RRC signaling, the UE follows the configure SFI.

  - Otherwise, the UE follows the behavior which is defined for initial access phase.

    - RAN1 should define an SFI assumption for initial access phase.

Proposal 2:

- If a UE is configured with group-common PDCCH monitoring, and if the UE does not detect the group-common PDCCH, the UE follows the UE behavior which is defined for the case that the UE has not been configured to monitor the group-common PDCCH yet, e.g. UE behavior during an initial access phase.

# References

- Chairman’s note RAN1 AH-NR1, January 2017.

- Chairman’s note RAN1 AH-NR2, June 2017.

- RP-170847, “New WID on New Radio Access Technology”, NTT DOCOMO, 3GPP TSG RAN Meeting #75, March 2017.