3GPP TSG RAN WG1 Meeting #90	R1-1714547

Prague, P.R. Czechia 21th – 25th August 2017

Source:	Ericsson

Title:	Code Block Segmentation for LDPC Codes

Agenda Item:	6.1.4.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1#88bis, the following agreements on code block size and CRC attachment were reached [1] :

Agreement:

- CBG-level CRC is not adopted

Agreement:

- Same value of Z for code blocks within a TB

In this contribution, we consider the code block size calculation when segmentation is applied. The related issue of transport block size determination is also discussed.

# Code Block Segmentation

For NR, it has been agreed that the CRC attachment is applied at CB-level and TB-level, thus code block segmentation procedure needs to take into account the CRC size at both CB-level and TB-level. The basic principle of the code block segmentation can be described as blow:

If TBS+L1≤Z

Number of code blocks: C=1

CBS = TBS;

else

Number of code blocks: C=(TBS+L2)/(Z-L3)

CBS=(TBS+L2)/C

end if

The variables are described below.

- Z. In LTE (3GPP TS 36.212), variable Z is the maximum information block size that is accepted at the input of turbo encoder, Z=6144 bits. Similar to TS 36.212, variable Z in the above is the maximum information block size at the input of LDPC encoder, and Z=8448 bits for NR.

- L1. Variable L1 is the number of CRC bits that are attached to the TBS when TBS is above a threshold TBSthrsh, TBSthrsh≤Z, FFS final value of TBSthrsh.  Note that when when TBS is lower than the threshold TBSthrsh, a different number of CRC bits, L0, may be applied, L0 < L1. FFS final value of L0.

- L2 and L3. When C > 1, L2 =24 is the number of CRC bits attached to each transport block; L3 is the number of CRC bits are attached to each CB after segmentation.

As can be observed from the equation of CBS above, (TBS+L2) is in general not a multiple of CBS, thus requiring zero padding of one or more CB. On the other hand, it is possible to adjust TBS so that zero padding at CB segmentation is not necessary. This is shown in next section.

# Transport Block Size

The transport block size is general determined by resource allocation, MCS, and number of MIMO layers. For example, the approximate TB size, TBS0, can be calculated according to:

TBS0=NPRB∙NREDL,PRB∙υ∙Qm∙R

where

  - ν is number of MIMO layers the codeword is mapped onto;

  - NREDL,PRB is the number of REs per PRB per slot/mini-slot available for carrying the PDSCH;

  - NPRB is the number of allocated PRBs;

  - modulation order, Qm, and target code rate, R, are read from a MCS table based on IMCS signalled in the DCI;

In the above, NPRB,  NREDL,PRB, υ, Qm, R are signaled through DCI or are configured through higher layers, similar to that of LTE.

TBS0 can be adjusted slightly to facilitate the code block segmentation procedure. The adjustment is done so that each of the code block segments have the same code block size (CBS), thus guaranteeing that the same shift size is used for the LDPC code. The adjustment also ensures that zero padding is not necessary in code block segmentation, similar to that of LTE turbo codes.

The adjustment of TBS, as well as the CBS determination procedure, is described below, with TBS0 as input.

if   8×TBS08+L1≤Z

Number of code blocks: C=1

Transport block size (TBS): TBS=8×TBS08 (bits)

Length of information block to LDPC encoder = (TBS + L1) 	(bits)

else

Number of code blocks: C=(TBS0+L2)/(Z-L3)

Transport block size (TBS): TBS=lcm(C,8)×TBS0+L2lcm(C,8)-L2 	(bits)

Code block size: CBS=TBS+L2C 		(bits)

Length of information block to LDPC encoder = (CBS + L3) 		(bits)

end if

In the above, lcm(C,A) is the least common multiple of 8 and C. In the above, constant ‘8’ is applied where appropriate so that the TBS is rounded up to be byte-aligned, according to the convention of MAC layer procedure.

As can be observed from the above procedure, each code block has the same CBS, and the CBS is not necessarily byte-aligned. This is appropriate since the LDPC parity check matrices is designed to support shortening with bit-level granularity.

- When code block segmentation is applied, each code block has the same CBS.

- CBS is not required to be byte-aligned.

- Transport block size determination procedure ensures that TBS is a multiple of CBS.

# Conclusion

In this contribution, we discussed the code block segmentation procedure of LDPC codes. We have the following proposal:

- When code block segmentation is applied, each code block has the same CBS.

- CBS is not required to be byte-aligned.

- Transport block size determination procedure ensures that TBS is a multiple of CBS.

# References

RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting RAN1 NR AdHoc #2, June 2017.

3GPP TS 36.212.