3GPP TSG RAN WG1 Meeting #90							R1-1715294

Prague, Czechia, 21th – 25th August 2017

Source:	NTT DOCOMO, INC.

Title:	Outcome of offline discussions on UL transmission without UL grant

Agenda Item:	6.1.3.3.3

Document for:	Discussion and Decision

Following issues are discussed:

- For Type 1 UL transmission without UL grant, different RNTI from the RNTI for UL transmission with grant is needed at least for re-transmission of Type 1 UL transmission without UL grant.

  - FFS how to determine the RNTI.

Version 1

- Type 1 UL transmission without UL grant uses different RNTI from the RNTI for UL transmission with grant.

  - FFS how to determine the RNTI.

Supported by HW, DCM, Vivo, Xiaomi,

Version 2

- At least when an UL grant is used for retransmissions of Type 1 UL transmission without UL grant, different RNTI from the RNTI for UL transmission with grant is needed.

  - FFS how to determine the RNTI.

Supported by HW, LGE, DCM, ZTE, NEC, Vivo, Xiaomi, NICT, Ericsson, Panasonic, Sony, Qualcomm, MediaTek, TCL

- To indicate HARQ-ACK status of UL transmission without grant, following options and related UE behavior should be further studied.

  - Option 1: Based on UL grant explicitly or implicitly

  - Option 2: Group-common DCI

    - 2-1: Only ACK

    - 2-2: ACK and NACK

  - Option 3: Define a Timer, when the Timer expires, UE assumes

    - 3-1: ACK

    - 3-2: NACK

Version 1

- If HARQ feedback is supported, to indicate HARQ feedback of UL transmission without grant, following options and related UE behavior should be further studied.

  - Option 1: Based on UL grant to indicate “ACK”

  - Option 2: Group-common DCI

    - 2-1: Only ACK

    - 2-2: ACK and NACK

- NOTE: Option 1 and Option 2 can be used during and after the K repetition

- When no feedback is received after the K repetitions, UE assumes

  - Option 1: ACK if an NACK is not received

  - Option 2: NACK if an ACK is not received and FFS on UE behavior

- NOTE: UL grant used for retransmission of the same TB which indicates “NACK” is already supported.

Supported: HW, DCM, TCL, Convada Wireless, LG, Vivo

Version 2

- If HARQ feedback is supported, to indicate HARQ feedback of UL transmission without grant, following options and related UE behavior should be further studied.

  - Option 1: Based on UL grant to indicate “ACK”

  - Option 2: Group-common DCI

    - 2-1: Only ACK

    - 2-2: ACK and NACK

  - Option 3: Define a Timer, UE assumes following, when the Timer expires

    - 3-1: ACK if an NACK is not received after the K repetitions

    - 3-2: NACK if an ACK is not received

- FFS: Option 1, Option 2 and Option 3-2 can be used during and after the K repetition

- Note: UL grant for the same TB initially transmitted without grant can indicate “NACK”

Supported: Ericsson, ZTE, HW, DCM, NEC, LG, Panasonic, Sony, Qualcomm, MediaTek, TCL

Version 3

- If HARQ feedback is not supported,

  - ACK if an UL grant scheduling re-transmission is not received

  - ACK if an UL grant scheduling new data is received

- If HARQ feedback is supported, to indicate HARQ-ACK status of UL transmission without grant, following options and related UE behavior should be further studied.

  - Option 1: Based on UL grant to indicate “ACK” without scheduling UL transmission.

  - Option 2: Group-common DCI

    - 2-1: Only ACK

    - 2-2: ACK and NACK

  - NOTE: Option 1 and Option 2 can be used during and after the K repetition

  - When no feedback is received after the K repetitions, UE assumes

    - Option 1: ACK if an NACK is not received

    - Option 2: NACK if an ACK is not received

- NOTE: UL grant used for retransmission of the same TB which indicates “NACK” is already supported.

Offline consensus:

- Multiple resources of UL transmission without grant can be configured.

  - FFS whether multiple resources are configured using one set of RRC configuration or using multiple sets of RRC configurations

Not discussed yet. Suggestion: we should discuss resources used for repetitions when K>1 first