3GPP TSG RAN WG1 Meeting #90		R1-1712042

Prague, Czech Republic, 21th – 25th August 2017

Title:	CHARACTERISTICS OF IMT-2020 SYSTEM FOR COEXISTENCE STUDY IN THE FREQUENCY BAND 4 800-4 990 MHz

Release:	Rel-15

Work Item:

Source:	ITU-R Working Party 5D

To:	RAN4, RAN1

Cc:	RAN

Contact Person:

#### Name:	Sergio Buonomo	Counsellor ITU-R SG 5Amaanat Ali

            - E-mail Address:	sergio.buonomo@itu.int

Attachments: None

| Radiocommunication Study Groups |  |
| --- | --- |
| 27th Meeting of Working Party 5D Niagara Falls, Canada, 13-21 June 2017 |  |
| Attachment 7.4 to Document 5D/666 (Source: Document 5D/TEMP/345 Rev.1) |  |
|  | 04 July 2017 |
|  | English only |
| Working Party 5D |  |
| Liaison Statement to 3GPP |  |
| Characteristics of IMT-2020 system for coexistence study in the frequency band 4 800-4 990 MHz |  |

The ITU World Radiocommunication Conference 2015 (WRC-15), which met in November 2015, revised the Resolution 223 which further invites ITU-R to study the technical and regulatory conditions for the use of IMT in the frequency band 4 800-4 990 MHz in order to protect the aeronautical mobile service.

Working Party 5D (WP 5D), as the responsible group for IMT in ITU-R, has initiated the development of the working document towards preliminary draft new Recommendation/Report (PDNR) on “Coexistence conditions between IMT and aeronautical mobile service (AMS) in the band 4 800-4 990 MHz” in its 23rd meeting (Feb. 2016).

WP 5D has previously developed sharing parameters for IMT-Advanced technologies for bands between 3 and 6 GHz, which are contained in Report ITU-R M.2292 and summarized in Annex 1. In addition, the beamforming antenna characteristics of IMT-2020 for frequency bands between 24.25 GHz and 33.4 GHz, which are summarized in Annex 2, have been developed in WP 5D for sharing and compatibility study under WRC-19 agenda item 1.13.

Due to the situation that the parameters and beamforming antenna characteristics of IMT-2020 in the frequency band 4.8-4.99 GHz are absent in ITU-R, WP 5D would ask for any input from 3GPP on the parameters and antenna characteristics for IMT-2020 in 4 800-4 990 MHz. If this information is not available, WP 5D will use the parameters in Annex 1 and the beamforming antenna characteristics in Annex 2 as the baseline to further develop the IMT-2020 parameters in the coexistence study in the frequency band 4 800-4 990 MHz. WP 5D would welcome 3GPP’s views on the appropriateness of this approach.

WP 5D would kindly request the above mentioned information preferably in time for the next meeting of WP 5D which is planned for 3-11th October 2017.

| Status: For action |  |
| --- | --- |
| Contact: Sergio Buonomo Counsellor ITU-R SG 5 | E-mail: sergio.buonomo@itu.int |

Annexes:

1)	IMT-Advanced technology-related and deployment-related parameters for bands between 3 and 6 GHz (Source: Report ITU-R M.2292)

2)	Antenna characteristics for IMT-2020 for bands between 24.25 and 33.4 GHz (Source: Document 5-1/36, Attachment 2, Table 10)

ANNEX 1

IMT-Advanced technology-related and deployment-related parameters 
for bands between 3 and 6 GHz

TABLE A1-1

IMT-Advanced specification related parameters

|  |  | IMT-Advanced |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplex mode | FDD |  | TDD |  |
| No. | Parameter | Base station | Mobile station | Base station | Mobile station |
| 1 | Access technique | OFDM | SC-FDMA | OFDM | SC-FDMA |
| 2 | Modulation parameters | QPSK 16-QAM 64-QAM |  |  |  |
| 3 | Channel spacing | Nominal channel spacing = (BWChannel(1) + BWChannel((2))/2 |  |  |  |
| 4 | Channel bandwidth (MHz) | (1) | (1), (19) | (1) | (1), (19) |
| 5 | Signal bandwidth (MHZ) | (1) | (1), (19) | (1) | (1), (19) |
| 6 | Transmitter characteristics |  |  |  |  |
| 6.1 | Power dynamic range (dB) | (2) | 63(18) | (2) | 63(18) |
| 6.2 | Spectral mask | (3), (17) | (4), (17) | (3), (17) | (4), (17) |
| 6.3 | ACLR | (5), (17) | (6), (17) | (5), (17) | (6), (17) |
| 6.4 | Maximum output power | (7) | (8) | (7) | (8) |
| 6.5 | Spurious emissions | (15), (17) | (16), (17) | (15), (17) | (16), (17) |

TABLE A1-1 (end)

|  |  | IMT-Advanced |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Duplex mode | FDD |  | TDD |  |
| No. | Parameter | Base station | Mobile station | Base station | Mobile station |
| 7 | Receiver characteristics |  |  |  |  |
| 7.1 | Noise figure | 5 dB (macro) 10 dB (micro) 13 dB (pico/femto) | 9 dB | 5 dB (macro) 10 dB (micro) 13 dB (pico/femto) | 9 dB |
| 7.2 | Sensitivity | (9) | (10) | (9) | (10) |
| 7.3 | Blocking response | (11) | (12) | (11) | (12) |
| 7.4 | ACS | (13) | (14) | (13) | (14) |
| Notes to the Table: (1) See 3GPP Document TS 36.101 v.11.2.0, § 5.6. Signal bandwidth in MHz corresponds to “Transmission bandwidth configuration*0.180”. (2) See 3GPP Document TS 36.104 v.11.2.0, § 6.3.2.1. (3) See 3GPP Document TS 36 104 v.11.2.0, § 6.6.3. (4) See 3GPP Document TS 36 101 v.11.2.0, §§ 6.6.2.1, 6.6.2.1A, 6.6.2.2 and 6.6.2.2A describe UE spectrum emissions masks for different channel bandwidths. In case multiple UEs are transmitting simultaneously on the same channel they will share the available radio resource blocks. As the actual transmission bandwidth is thus decreased the unwanted emissions performance might be improved. This may be taken into account during sharing analysis when measurements or detailed models are available. (5) See 3GPP Document TS 36.104 v.11.2.0, § 6.6.2. (6) See 3GPP Document TS 36.101 v.11.2.0, § 6.6.2.3. (7) See 3GPP Document TS 36.104 v.11.2.0, § 6.2. (8) See 3GPP Document TS 36.101 v.11.2.0, § 6.2. (9) See 3GPP Document TS 36.104 v.11.2.0, § 7.2. (10) See 3GPP Document TS 36.101 v.11.2.0, § 7.3. (11) See 3GPP Document TS 36.104 v.11.2.0, § 7.6. (12) See 3GPP Document TS 36.101 v.11.2.0, § 7.6 and § 7.7. (13) See 3GPP Document TS 36.104 v.11.2.0, § 7.5. (14) See 3GPP Document TS 36.101 v.11.2.0, § 7.5. (15) See 3GPP Document TS 36.104 v.11.2.0, § 6.6.4. (16) See 3GPP Document TS 36.101 v.11.2.0, § 6.6.3. (17) These unwanted emission limits are the upper limits from SDO specifications for laboratory testing with maximum transmitting power. It is assumed that when the in-band transmitting power is reduced by x dB through power control, the unwanted emission levels would be reduced by x dB in consequence in the coexistence simulations. (18) See 3GPP Document TS 36.101 v.11.2.0, § 6.3. (19) In case multiple UEs are transmitting simultaneously on the same channel they will share the available radio resource blocks. As the actual transmission bandwidth is thus decreased the unwanted emissions performance might be improved. This may be taken into account during sharing analysis when measurements or detailed models are available. |  |  |  |  |  |

TABLE A1-2

IMT-Advanced Deployment-related parameters for bands between 3 and 6 GHz

|  | Macro suburban | Macro urban | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |
| --- | --- | --- | --- | --- |
| Base station characteristics/Cell structure |  |  |  |  |
| Cell radius/Deployment density | 0.3-2 km (typical figure to be used in sharing studies 0.6 km) | 0.15-0.6 km (typical figure to be used in sharing studies 0.3 km) | 1-3 per urban macro cell <1 per suburban macro site | Depending on indoor coverage/capacity demand |
| Antenna height | 25 m | 20 m | 6 m | 3 m |
| Sectorization | 3 sectors | 3 sectors | Single sector | Single sector |
| Downtilt | 6 degrees | 10 degrees | n.a. | n.a. |
| Frequency reuse | 1 | 1 | 1 | 1 |
| Antenna pattern | Recommendation ITU-R F.1336 (recommends 3.1) ka = 0.7 kp = 0.7 kh = 0.7 kv = 0.3 Horizontal 3 dB beamwidth: 65 degrees Vertical 3 dB beamwidth: determined from the horizontal beamwidth by equations in Recommendation ITU-R F.1336. Vertical beamwidths of actual antennas may also be used when available. |  | Recommendation ITU-R F.1336 omni |  |

TABLE A1-2 (continued)

|  | Macro suburban | Macro urban |  | Small cell outdoor/ Micro urban | Small cell indoor/ Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- |
| Base station characteristics/Cell structure |  |  |  |  |  |  |
| Antenna polarization | Linear/±45 degrees |  | Linear/±45 degrees | Linear | Linear |  |
| Indoor base station deployment | n.a. |  | n.a. | n.a. | 100% |  |
| Indoor base station penetration loss | n.a. |  | n.a. | n.a. | 20 dB (3-5 GHz) 25 dB (5-6 GHz) (horizontal direction) Rec. ITU-R P.1238, Table 3 (vertical direction) |  |
| Below rooftop base station antenna deployment | 0% |  | 50% | 100% | n.a. |  |
| Feeder loss | 3 dB |  | 3 dB | n.a | n.a |  |
| Maximum base station output power (5/10/20 MHz) | 43/46/46 dBm |  | 43/46/46 dBm | 24 dBm | 24 dBm |  |
| Maximum base station antenna gain | 18 dBi |  | 18 dBi | 5 dBi | 0 dBi |  |
| Maximum base station output power/sector (EIRP) | 58/61/61 dBm |  | 58/61/61 dBm | 29 dBm | 24 dBm |  |
| Average base station activity | 50% |  | 50% | 50% | 50% |  |
| Average base station power/sector taking into account activity factor | 55/58/58 dBm |  | 55/58/58 dBm | 26 dBm | 21 dBm |  |
| User terminal characteristics |  |  |  |  |  |  |
| Indoor user terminal usage | 70% |  | 7% | 70% |  | 100% |
| Indoor user terminal penetration loss | 20 dB |  | 20 dB | 20 dB |  | 20 dB (3-5 GHz) 25 dB (5-6 GHz) (horizontal direction) Rec. ITU-R P.1238, Table 3 (vertical direction) |
| User terminal density in active mode to be used in sharing studies | 2.16/5 MHz/km2 |  | 3/5 MHz/km2 | 3/5 MHz/km2 |  | Depending on indoor coverage/capacity demand |

TABLE A1-2 (end)

|  | Macro suburban | Macro urban |  | Small cell outdoor/Micro urban | Small cell indoor/Indoor urban |  |
| --- | --- | --- | --- | --- | --- | --- |
| User terminal characteristics |  |  |  |  |  |  |
| Maximum user terminal output power | 23 dBm |  | 23 dBm | 23 dBm |  | 23 dBm |
| Average user terminal output power | –9 dBm |  | –9 dBm | –9 dBm |  | –9 dBm |
| Typical antenna gain for user terminals | –4 dBi |  | –4 dBi | –4 dBi |  | –4 dBi |
| Body loss | 4 dB |  | 4 dB | 4 dB |  | 4 dB |

ANNEX 2

Antenna characteristics for IMT-2020 for bands 
between 24.25 and 33.4GHz

TABLE A2-1

Antenna Characteristics for IMT-2020 for bands between 24.25 and 33.4 GHz

|  |  | Outdoor Suburban hotspot |  | Outdoor Urban hotspot | Indoor |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 1.2 | Element gain (dBi) | 5 | 5 |  | 5 |
| 1.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 65º for both H/V | 65º for both H/V |  | 90º for both H/V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V |  | 25 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 1.6 | Antenna array configuration (Row × Column) NOTE 2 | 8×8 elements | 8×8 elements |  | 8×8 elements |
| 1.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V | 0.5 of wavelength for both H/V |  | 0.5 of wavelength for both H/V |
| 1.8 | Array Ohmic loss (dB) | 3 | ## 3 |  | 3 |
| 1.9 | Conducted power (before Ohmic loss) per antenna element (dBm/200 MHz) | 10 | 10 |  | 5 |
| 1.10 | Base station maximum coverage angle in the horizontal plane (degrees) | 120 | 120 |  | 120 |
| 2 | User terminal Antenna Characteristics |  |  |  |  |
| 2.1 | Antenna pattern | Refer to Recommendation ITU-R M.2101 |  |  |  |
| 2.2 | Element gain (dBi) | 5 | 5 |  | 5 |
| 2.3 | Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for both H/V | 90º for both H/V |  | 90º for both H/V |
| 2.4 | Horizontal/vertical front-to-back ratio (dB) | 25 for both H/V | 25 for both H/V |  | 25 for both H/V |
| 2.5 | Antenna polarization | Linear ±45º | Linear ±45º |  | Linear ±45º |
| 2.6 | Antenna array configuration (Row × Column) NOTE 2 | 4×4 elements | 4×4 elements |  | 4×4 elements |
| 2.7 | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V | 0.5 of wavelength for both H/V |  | 0.5 of wavelength for both H/V |
| 2.8 | Array Ohmic loss (dB) | 3 | 3 |  | 3 |
| 2.9 | Conducted power (before Ohmic loss) per antenna element (dBm / 200 MHz) | 10 | 10 |  | 10 |

______________

____________________