3GPP TSG RAN WG1 Meeting 90bis           	          R1-1717944

Prague, Czech Republic, 9th – 13th, October 2017

Agenda item: 7.2.3.1

Source: LG Electronics

Title: On multiplexing of different types of RSs

Document for: Decision

Introduction

In 3GPP RAN1 NR Ad-hoc #3 meeting, following agreements were made related to multiplexing of different types of RSs [1].

Agreements: - Down-select among the following two options - Option 1-1: From a UE perspective, CSI-RS is not multiplexed on SS block OFDM symbol(s) - Option 1-2: From a UE perspective, CSI-RS can be multiplexed on SS block symbol(s) - FFS on the conditions when multiplexing is allowed - Down-select among the following two options - Option 3-1: Within a BWP, a UE is not expected to be configured with NZP-CSI-RS in OFDM symbols for which it is configured to receive DMRS - E.g., from a UE perspective in the slot with scheduled PDSCH, CSI-RS can be transmitted on the potential additional DMRS OFDM symbol(s), when the additional DMRS does not exist in the OFDM symbol(s). - Note: In Option 3-1, CSI-RS is not multiplexed on the potential front-loaded DMRS OFDM symbol(s) - Option 3-2: From a UE perspective, CSI-RS can be multiplexed on all any potential DMRS OFDM symbol locations(s) - Note: Additional restriction from cell-/UE-group perspective will be further discussed based on the down-selection above

In this contribution, we further discuss the multiplexing issue between different types of RSs based on the above agreement.

Multiplexing of different types of RS

CSI-RS

CSI-RS multiplexing with other signals such as DMRS, SS block symbol(s) is an important issue. In the previous meeting, CSI-RS multiplexing issue was actively discussed and several down-selection issues were agreed regarding whether or not to allow frequency domain multiplexing of CSI-RS and DMRS/ SS block [1], which are captured in Section 1.

Regarding the down-selection issue for multiplexing of CSI-RS and SS block captured in Section 1, option 1-1 could be beneficial from a UE implementation complexity point of view, and this option fundamentally would not cause signal collision problem between CSI-RS REs and REs occupied by other signals. On the other hand, CSI-RS capacity would be limited in those options. If CSI-RS multiplexing with other signals needs to be unavoidably allowed, it should be limited to a case of RB-level multiplexing due to the high complexity of handling RE-level multiplexing and collision avoidance in the implementation. In other words, CSI-RS can be FDMed with SS block in different PRB set but not in the same PRB set. For RB-level multiplexing, handling of subband CSI would be an issue. If an unexpected and/or expected collision event between aperiodic/semi-persistent CSI-RS and SS block is occurred on SS block symbol(s), the configured CSI-RS REs can be punctured or omitted according to predefined priority rules. Then, it would result in the lack of CSI measurement in a certain subband. Thus, multiplexing of CSI-RS and SS block should be limited to a particular type of CSI-RS not requiring subband CSI reporting such as CSI-RS for beam management or CSI-RS for wideband CSI reporting. Alternatively, we may consider defining a UE behavior for the overlapped subband such as excluding the overlapped subband for subband CSI reporting and so forth.

For similar reasons, multiplexing of CSI-RS and DMRS should be carefully allowed with some restrictions. Depending on the location of additional DMRS symbol, we may need to consider RE-level multiplexing of CSI-RS and DMRS. As a simple example based on the agreements with respect to possible locations of CSI-RS and DMRS, Figure 1 can be considered where 8-port CSI-RS and 4-port type II DMRS are multiplexed on the 6th OFDM symbol in frequency domain. For RE-level multiplexing, CSI-RS type including time-domain behavior, number of ports, and/or RE pattern allowed to be RRC configured in those potential symbol set for DMRS needs to be specified by considering different RE patterns according to different DMRS types.

Figure 1. An example of RE pattern for 8-port CSI-RS with two 1-symbol additional 4-port DMRS

Proposal 1: Regarding multiplexing between CSI-RS and SS block, RB-level FDM can be considered, with some restrictions on applicable CSI-RS types and wideband/subband reporting.

Proposal 2: Regarding multiplexing between CSI-RS and DMRS, RE-level multiplexing can be considered, with some restrictions on applicable CSI-RS types including time-domain behavior and DMRS types.

SRS

Whether or not to allow FDM of SRS and UL DMRS depends on the possible symbol locations of SRS. Current agreement states that a X-port SRS resource can span 1, 2, or 4 adjacent symbols within a slot where all X ports are mapped to each symbol of the resource. For UL beam management, it can be considered to use more than 4 symbols in a slot. In our view, 4 symbols in a slot would be sufficient for UL beam management purpose as well as UL CSI acquisition purpose. In this regard, UL DMRS may not have a chance to be collided with SRS unless additional DMRS is located at symbols later than 10th symbol in a slot.

Proposal 3: TDM of SRS and UL DMRS would be sufficient.

Conclusion

In this contribution, the multiplexing issue between CSI-RS and SS block/DL DMRS is addressed, and multiplexing of SRS and UL DMRS is discussed. Based on the discussion, following proposals are given:

Proposal 1: Regarding multiplexing between CSI-RS and SS block, RB-level FDM can be considered, with some restrictions on applicable CSI-RS types and wideband/subband reporting.

Proposal 2: Regarding multiplexing between CSI-RS and DMRS, RE-level multiplexing can be considered, with some restrictions on applicable CSI-RS types including time-domain behavior and DMRS types.

Proposal 3: TDM of SRS and UL DMRS would be sufficient.

Reference

[1] RAN1 Chairman’s Notes of 3GPP TSG RAN WG1 Meeting NR#3.